3GPP TSG RAN WG4 Meeting #107	R4-2309649

Incheon, Korea, May 22-26, 2023

Agenda Item:	8.11.2.3

Source: 	Ericsson

Title:	Further analysis of BWP operation without restriction option B-1-2

Document for:	Discussion

# Introduction

In the last RN4 meeting the WF containing the agreements and open issues related to the RLM/BFD/BM requirements based on option B-1-2 was approved [1] i.e. BM/RLM/BFD based on SSB outside the active BWP with interruption.

This paper further analyzes the open issues related to option B-1-2 identified in the last RAN4 meeting [1].

# Analysis of option B-1-2

## Interruption requirements

The main discussion for the UE supporting option B-1-2 is related to the interruption requirements. There was no consensus on any aspect of the interruption requirements for option B-1-2. However, the following options for defining the interruption requirements for option B-1-2 were identified [1]:

- Option 1:

  - Introduce symbol level interruption requirement for Option B-1-2, which is equal to the symbol length of MG for CD-SSB.  The portion of interrupted symbols would be decided by the SSB periodicity for BM/RLM/BFD.

- Option 2:

  - Define explicit interruption length without mentioning interruption ratio.

- Option 3:

  - Support NW to control the interruption location. Interruption ratio are not defined. Define requirements on length of each interruption. Use the following values as baseline

    - FR1: 0.5ms

    - FR2: 0.25ms

- Option 4:

  - Interruption requirements for UE supporting option B-1-2 are specified with probability of missed ACK/NACK.

    - Alt 1: For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP) measurements based on SSB outside active BWP.

    - Alt 3: Specify 0.5% probability of missed ACK/NACK as the interruption requirements during BM/RLM/BFD measurements based on SSB outside the active BWP.

- Option 5:

  - Define interruption ratio and interruption length for each interruption

    - Alt 2: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. And the length for each interruption shall not exceed the RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

- Option 6:

  - RAN4 shall leverage the interruption requirements (NCSG and NFG) from L3 measurements to define the interruption requirements for RLM/BFD/BM measurements.

Broadly all the above options support the idea of defining the interruption requirements. The RF retuning performed by the UE to measure on the SSB for the RLM/BFD/BM will cause interruption on other activated serving cells if the UE is configured in NR CA or DC (i.e. EN-DC, NE-DC or NR-DC).

The UE will periodically sample/measure the SSB for the RLM/BFD/BM periodically depending on the periodicity of the configured SSB resources. The measurement sampling for obtaining the measurements for RLM/BFD/BM is up to the UE implementation.

The UE is configured with different set of SSB resources for different set of RLM and LRP procedures via RRC. According to TS 38.331, the following independent set of SSB resources are configured for various RLM and link recovery (LRP) procedures:

- RadioLinkMonitoringRS; same for both RLM and BFD

- CandidateBeamRS; for CBD

- CSI-SSB-ResourceSet; same for both L1-RSRP and L1-SINR measurements

Therefore, the interruption requirements should be defined in terms of both:

- Maximum interruption probability and

- Maximum length of each interruption

The above interruption metrics are described below:

Maximum interruption probability:

For example, assuming SSB periodicity of 40 ms, each interruption length of one slot for 15 kHz SCS and assuming one interruption before and one after the SSB resource, the total number of interruptions on other active serving cells during the out-of-sync evaluation period, which requires 10 SSB samples, will be 20 slots. This corresponds to 5% interruption probability. There will be similar level of interruptions on other active serving cells due to CBD and L1-RSRP/L-SINR measurements. This level of interruptions is certainly unacceptable from the network performance point of view.

We therefore suggest that the maximum allowed interruption probability is limited to around 0.5% due to the measurements for RLM/BFD, CBD and L1-RSRP/L1-SINR. Therefore, one option is to define individual interruption probability for each of the three categories of the measurements e.g.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

Another alternative is to define one aggregated interruption probability covering all the three categories of the measurements. In this case in our view the aggregared interruption probability should be limited to 1%. This is because it is unlikely that the UE simultaneously (during overlapping time) performs the three categories of the measurements (RLM/BFD, CBD and L1-RSRP/L1-SINR) all the time. Therefore, we suggest that the aggregared interruption probability is less than 1% e.g.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD, CBD and L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 1 %.

Furthermore, the interruption should only be allowed in the following cases with respect to the DRX cycle on the serving cell on which the RLM/LRP measurements are performed:

- When no DRX cycle is used on the serving cell on which the RLM/LRP measurements are performed.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is shorter than or equal to 320 ms and the DRX ON duration is aligned with the SSB

Otherwise, no interruption should be allowed in the following cases i.e.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is less than or equal to 320 ms and the DRX ON duration is misaligned with the SSB.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is larger than 320 ms.

The reason is that when DRX cycle is long (i.e. ≥ 640 ms) the UE can retune its receiver between the ON duration and perform the RLM/LRP measurements without causing any interruption on other serving cells.

Maximum interruption length:

The maximum interruption length of the interruption on the other active serving cells (victim cell) should also be defined. The existing interruption lengths defined in TS 38.133 and TS 36.133 various scenarios can be reused. The RF tuning time is 0.5 ms and 0.25 ms in FR1 and FR2 respectively. The interruption length can be expressed in or rounded off to slots.

Interruption in NR CA:

When the UE is configured with NR CA then Table 8.2.2.2.2-1 defined in TS 38.133 will be reasonable proposal, which can be applicable for the inter-band CA. On the other hand, Table 8.2.2.2.2-2 defined in TS 38.133 can be reused with necessary modification for intra-band CA. In both cases in order to minimize the interruption, the UE should retune immediately before and immediately after the SSB resource i.e. slots containing the SSB(s) for the RLM or RLP. The proposals are shown in table 2 and table 3 for inter-band CA and intra-band CA respectively.

Table 2: Interruption due to RLM and RLP measurements on other active serving cell in inter-band NR CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 3: Interruption due to RLM and RLP measurements on other active serving cell in intra-band NR CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + NSSB |
| 1 | 0.5 | 1 + NSSB |
| 2 | 0.25 | 2 + NSSB |
| 3 | 0.125 | 4 + NSSB |
| 5 | 0.03125 | 16 + NSSB |
| 6 | 0.015625 | 32 + NSSB |
| NOTE: NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR |  |  |

Interruption in NR DC:

The NR DC supports both synchronous and asynchronous inter-band DC. For synchronous inter-band NR-DC, the interruption length shown in table 2 can be reused since the MRTD for NR CA and synchronous inter-band NR-DC are the same or in the same range.

For asynchronous inter-band NR-DC, 1 slot needs to be added to the interruption length in table 4 for SCS ≥ 60 kHz to account for the MRTD for NR DC.

Table 4: Interruption due to RLM and RLM on other active serving cell in asynchronous inter-band NR-DC

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 4 |
| 3 | 0.125 | Aggressor cell is on FR2 | 5 |
|  |  | Aggressor cell is on FR1 | 6 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 18 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 34 |

Interruption on LTE serving cell(s) in EN-DC and NE-DC:

In EN-DC, the interruption on the active LTE serving cell(s) depends on whether it is inter-band (which can be synchronous or asynchronous EN-DC) or intra-band EN-DC (which can be only synchronous EN-DC). The interruption on the active LTE serving cell(s) for EN-DC and NE-DC can be derived from section 7.32.2.6.1 in 36.133 as follows:

For inter-band EN-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band EN-DC or

- 2 subframes for asynchronous inter-band EN-DC.

For synchronous intra-band EN-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

For inter-band NE-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band NE-DC or

- 2 subframes for asynchronous inter-band NE-DC.

For synchronous intra-band NE-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

Where, NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR.

Table 5 and table 6 lists high level sections in TS 38.133 and TS 36.133 where the interruption requirements on other active serving cells due to the RLM/LRP measurements performed based on option B-1-2 should be defined.

Table 5: List of top sections on interruption requirements in TS 38.133 impacted for UE supporting option B-1-2:

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 8.2.2 | SA: Interruptions with Standalone NR Carrier Aggregation |
| 2 | 8.2.4 | NR-DC: Interruptions |

Table 6: List of top sections on interruption requirements in TS 36.133 impacted for UE supporting option B-1-2:

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 7.32 | 7.32 Interruptions with EN-DC |
| 2 | 7.36 | 7.36 Interruptions with NE-DC |

# Summary

This paper further analyzes the interruption requirements for RLM and link recovery procedure (LRP) for the UE capable of option B-1-2 i.e. for the UE supporting BM/RLM/BFD based on SSB outside the active BWP with interruptions. The following are the observations and proposals:

Interruption due to RLM/LRP measurements:

- Observation #1: According to TS 38.331, the following independent set of SSB resources are configured for various RLM and LRP procedures:

  - RadioLinkMonitoringRS; same for both RLM and BFD

  - CandidateBeamRS; for CBD

  - CSI-SSB-ResourceSet; same for both L1-RSRP and L1-SINR measurements

- Observation #2: UE will periodically sample/measure the SSB for the RLM/BFD/BM periodically depending on the periodicity of the configured SSB resources.

- Observation #3: Due to the periodic sampling of the SSB for the RLM/BFD/BM it is important to define the maximum allowed interruption probability.

- Proposal #1: Define the interruption requirements in terms of both:

  - The maximum interruption probability and

  - Maximum length of each interruption

- Proposal #2: The maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined using either option 1 or option 2; we prefer option 1:

  - Option 1:

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %.

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %.

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

  - Option 2:

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD, CBD and L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 1 %.

- Proposal #3: The maximum length of each interruption is defined as follows:

Interruption length in NR CA:

Table 2: Interruption due to RLM and RLP measurements on other active serving cell in inter-band NR CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 3: Interruption due to RLM and RLP measurements on other active serving cell in intra-band NR CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + NSSB |
| 1 | 0.5 | 1 + NSSB |
| 2 | 0.25 | 2 + NSSB |
| 3 | 0.125 | 4 + NSSB |
| 5 | 0.03125 | 16 + NSSB |
| 6 | 0.015625 | 32 + NSSB |
| NOTE: NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR |  |  |

Interruption length in NR DC:

- For synchronous inter-band NR-DC, the interruption length is the same as in table 2.

- For asynchronous inter-band NR-DC, the interruption length is shown in table 4.

Table 4: Interruption due to RLM and RLM on other active serving cell in asynchronous inter-band NR-DC

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 4 |
| 3 | 0.125 | Aggressor cell is on FR2 | 5 |
|  |  | Aggressor cell is on FR1 | 6 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 18 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 34 |

Interruption length in EN-DC and NE-DC:

For inter-band EN-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band EN-DC or

- 2 subframes for asynchronous inter-band EN-DC.

For synchronous intra-band EN-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

For inter-band NE-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band NE-DC or

- 2 subframes for asynchronous inter-band NE-DC.

For synchronous intra-band NE-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

Where, NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR.

Proposal #4: The interruption requirements in proposals # 2 and # 4 are allowed only under certain conditions wrt the DRX cycle:

- The interruption is allowed only when any of the following conditions are met:

  - When no DRX cycle is used on the serving cell on which the RLM/LRP measurements are performed.

  - When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is shorter than or equal to 320 ms and the DRX ON duration is aligned with the SSB

- Otherwise, no interruption is allowed in any of the following cases:

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is less than or equal to 320 ms and the DRX ON duration is misaligned with the SSB.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is larger than 320 ms.

# References

- R4-2306397, WF on BWP operation without restriction, vivo

- R4-2306393, Topic summary for [106-bis-e][208] NR_BWP_wor, vivo.