3GPP TSG RAN WG4 Meeting #107	R4-2309650

Incheon, Korea, May 22-26, 2023

Agenda Item:	5.2.5.3

Source: 	Ericsson

Title:	Updates to offset for cell specific RSRP thresholds for 1Rx Redcap UE

Document for:	Discussion

- Introduction

RAN4 has received a reply to RAN4 LS from RAN2 regarding the offset for cell specific RSRP thresholds for 1Rx Redcap introduced by RAN4 in TS 38.133, Rel-17. However, there was no consensus on the replies to some of the questions in the RAN2 LS in RAN4#106.

In this contribution we further analyze RAN2 recommendation regarding the offset for the cell specific RSRP thresholds indicated in the LS.

# Analysis of offset for change thresholds for low mobility and stationary criteria

The RAN2 LS has identified that the offset should not be applied to the following change/delta thresholds, which are used by the UE for low mobility and stationary relaxed measurement criteria respectively:

- s-SearchDeltaP-r16

- s-SearchDeltaP-Stationary-r17

RAN4 agreed to remove the above thresholds according to following agreement in the WF [2]:

- Agreements

Remove the offset from s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17.

The reason is that low mobility and stationary relaxed measurement criteria are evaluated based on the change in the Srxlev, which in turn is derived based on the serving cell RSRP. Therefore, the offset will be redundant for the above delta thresholds. We therefore suggest upholding the agreement in the last meeting.

# Analysis of offset for thresholds for cell selection and not-at-cell-edge criterion

The following options related to the offset for thresholds related to the cell selection and not-at-cell-edge criterion [2]:

- Option 1: Remove the offset from all not-at-cell-edge thresholds.

- Option 2: RAN4 to keep the current offset value determined in previous RAN4 meetings.

  - Option 2a: Keep the offset and change it to +2dB for all not-at-cell-edge thresholds.

- Option 3:

  - Keep current RAN4 agreed offsets for SearchThresholdP-r16 and s-SearchThresholdP2-r17

  - Remove the offset from the thresholds SearchThresholdQ-r16 and s-SearchThresholdQ2-r17.

- Option 4: Apply “0 dB” 1 Rx. RedCap offset to Qrxlevmin and Qqualmin

The RAN2 LS has also identified that the offset should not be applied to both cell selection thresholds and not-at-cell-edge thresholds. The latter thresholds are used by the UE for the relaxed measurement criteria related to not-at-cell-edge. RAN4 has introduced an offset to both cell selection thresholds i.e. Qrxlevmin and Qqualmin. The UE evaluates the not-at-cell-edge related criteria by comparing the Srxlev and Squal with their respective absolute thresholds according to TS 38.304. The Srxlev and Squal are function of Qrxlevmin and Qqualmin respectively. Therefore, we agree with RAN2 assessment that the offset should not be applied to the thresholds related to not-at-cell-edge relaxed measurement criteria. Therefore, the offset should be removed from the following not-at-cell-edge related thresholds:

- s-SearchThresholdP-r16,

- s-SearchThresholdQ-r16,

- s-SearchThresholdP2-r17

- s-SearchThresholdQ2-r17

An alternate solution is to keep the offset for the above not-at-cell-edge thresholds but remove the offset from the following thresholds related to the cell selection:

- Qrxlevmin

- Qqualmin

However, the offset applied to the cell selection thresholds extends the cell coverage. Therefore, we prefer to keep the -1 dB offset to the above cell selection thresholds

# Analysis of offset for missing thresholds

The RAN2 LS has identified that the offset is missing for the following thresholds:

- rsrp-ThresholdSSB-SUL

- rsrp-ThresholdMsg3

The above thresholds were also discussed. But there was no consenus as noted in the WF [2]:

- No consensus on whether to apply the 1 rx offset to the above two thresholds (rsrp-ThresholdSSB-SUL and rsrp-ThresholdMsg3).

According to TS 38.331, the above thresholds are absolute thresholds and applied by the UE during random access procedure. As they are absolute threshold and used for random accces, therefore, we suggest to apply +1 dB offset like for other random access related thresholds.

# Summary

The following are the observations and the proposals related to the offset for some of the existing and missing thresholds:

Offset for thresholds related low mobility and stationary criteria:

- Observation #1: RAN2 has recommended that the offset should not apply to delta thresholds, i.e. s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17.

- Observation #2: RAN4 agreed that the offset should not apply to delta thresholds, i.e. s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17.

- Proposal #1: Remove the offset from s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17.

Offset for thresholds related to cell selection and not-at-cell-edge criteria:

- Observation #3: RAN2 has recommended that the offset should not apply to both cell selection thresholds (i.e. Qrxlevmin and Qqualmin) and not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17).

- Observation #4: According to TS 38.133 currently, the offset is applied to both cell selection thresholds and to all the not-at-cell-edge thresholds.

- Proposal #2: Following options are considered; our preference is option 1:

  - Option 1:

    - Remove the offset from all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17).

    - Keep the existing offset (-1 dB) to Qrxlevmin and Qqualmin

  - Option 2:

    - Keep the existing offset (+1 dB) to all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17).

    - Remove the offset (-1 dB) from Qrxlevmin and Qqualmin

Offset for missing thresholds:

- Observation #5: RAN2 has recommended that the offset is missing for the following thresholds: rsrp-ThresholdSSB-SUL and rsrp-ThresholdMsg3.

- Observation #6: The thresholds, rsrp-ThresholdSSB-SUL and rsrp-ThresholdMsg3, are absolute thresholds and applied by the UE during the random access procedure.

- Proposal #3: Add +1 dB of offset to ThresholdSSB-SUL and rsrp-ThresholdMsg3.

A Rel-17 CR to remove and add the offsets based on the above proposals is provided in [3].

- References

- R4-2300016, Reply LS on configuring margin for 1 Rx RedCap UEs, RAN2 LS.

- R4-2303264, WF on R17 RedCap RRM maintenance, Ericsson.

- R4-2309651, Correction to offset for cell specific RSRP thresholds for 1Rx Redcap UE in 38.133, Ericsson.