3GPP TSG-RAN WG4 Meeting #107			     R4-2309667

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on RRM core requirements for RedCap

Agenda item:	5.2.5.2

Document for:	Discussion

- Introduction

- The discussion on RedCap core requirements was continued at RAN4 #106. Agreements and open issues are listed in the WF [1].

In addition, RAN4 received an LS from RAN2[2].

In this contribution, we discuss the remaining issues on RRM core requirement maintenance for RedCap UE.

- Discussion

- The open issues from last meeting are discussed in this section.

2.1 Monitoring of paging occasions for CG-SDT

RAN4 received LS from RAN2 [2]. The LS is replicated below.

# 1 Overall description RAN2 has discussed possible clarifications on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs based on specification text in RAN2 and relevant sections in RAN1 and RAN4. Current RAN2 specifications do not explicitly specify what happens for UEs in half duplex mode if a paging occasion conflicts with a CG-SDT occasion. It is RAN2’s understanding that although information pertaining to this can be found in e.g., 38.213, clause 17.2 or in 38.133, clause 5.1B.2.6, the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. Similar to connected mode behaviour, since the UE is only required to monitor the paging in any paging occasion at least once per modification period, there should be other paging occasions available (within the modification period) to monitor the paging for SI change even if some of them overlap with the CG-SDT occasion(s). Hence, RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications. # 2 Actions To RAN WG1 and RAN WG4 ACTION: RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications for CG-SDT with HD-FDD Redcap.

As RAN4 understanding on the collision between CG-SDT and paging occasion has changed, changes in 38.133 are needed. In particular, the following change is needed in subclause 5.1B.2.6 Maximum interruption in paging reception.

For RedCap UE in HD-FDD mode, the UE shall monitor paging for SI change indication in any paging occasion at least once per modification period [2] during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. In case the determined paging occasion overlaps with the CG-SDT transmission, the UE shall determine another paging occasion in the modification period, else if no paging occasion is identified, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.

The change is implemented in the accompanying CR [3].

RAN4 to adopt the following change in subclause 5.1B.2.6 Maximum interruption in paging reception:

- For RedCap UE in HD-FDD mode, the UE shall monitor paging for SI change indication in any paging occasion at least once per modification period [2] during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. In case the determined paging occasion overlaps with the CG-SDT transmission, the UE shall determine another paging occasion in the modification period, else if no paging occasion is identified, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.

2.2 Other open issues

Other open issues are captured in [1] and replicated below.

### Mobility procedures with NCD-SSBs Handover to a BWP with unmeasured SSB of a known cell - No conclusion on whether to add following condition on Tx power of the SSB of the target and measured SSBs: - Option 1: Add condition on assumption on the TX power of the SSB of the target SSB and measured SSB as follows (underlined highlighted text is the addition to previous agreement). - For the case when the UE performs measurement on a SSB and handover to a BWP with un-measured SSB, the following apply: - The requirements apply for the scenario where the measured SSB and the SSB in the target BWP for HO belong to the same target cell, i.e., they share the same physCellID (PCI) - UE may assume that the TX power of the SSB of the target cell’s active BWP is same as that of the measured SSBs that belong to the same PCI - Note: the above agreement is for RedCap, where the measured SSB and the target SSB for HO of the same target cell are: - NCD-SSB and NCD-SSB, respectively - CD-SSB and NCD-SSB, respectively - NCD-SSB and CD-SSB, respectively - One additional sample is needed for known inter-frequency handover. Revision of the intra/inter-frequency definition in handover - FFS on whether to add following clarification to handover requirement: - Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where: - Option 1a: The reference SSB of the serving cell is the SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO - Option 1b: The reference SSB of the serving cell is the SSB configured in the active BWP of the serving cell - The reference SSB of the target cell is the SSB configured in the firstActiveBWP of the target cell Intra-frequency cell measurement - Discuss whether to add follow condition to intra-frequency measurement: - UE is not required to perform intra-frequency cell detection if the intra-frequency cell is already detected, where an intra-frequency cell is regarded as already detected if it has been meeting the following conditions: -The MO with the same SSB frequency has already be configured, and FFS on the following: - the UE has sent a valid measurement report for the MO with the same frequency, and - the SSB measured remains detectable according to the cee identification conditions specified in clause 9.2 and 9.3 Revision of the intra/inter-frequency definition in RRC re-establishment - No consensus to add following definition in RRC re-establishment requirement: - Option 1: A neighbor cell for RRC re-establishment procedures for a RedCap UE is defined as an intra-frequency cell if the center frequency (and subcarrier spacing (SCS)) of the reference SSB of the serving cell is same as the center frequency (and SCS) of the reference SSB of the neighbor cell; else it is considered as inter-frequency cell, where: - The reference SSB of the target cell is the CD-SSB of the target cell - The reference SSB of the serving cell is - Option 1: SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO - Option 2: CD-SSB of the serving cell - Option 3: SSB within the active BWP

Handover to a BWP with unmeasured SSB of a known cell

We can agree with option 1.

Proposal 2: Option 1 is supported related to open issue on handover to a BWP with unmeasured SSB of a known cell.

Revision of the intra/inter-frequency definition in handover

We have a slight preference for option 1a.

Proposal 3: Option 1a is supported related to open issue related to revision of the intra/inter-frequency definition in handover.

Intra-frequency cell measurement

We are not sure, if the proposal adds a clarification. How about a new intra-frequency cell becoming available with different PCI? Is this being covered. We think more discussion is needed here.

Proposal 4: Further discussion may be needed on the open issue related to intra-frequency cell measurement.

- Conclusions

In this document, Nokia’s view regarding open issues related to RRM core requirements for RedCap UEs is shared. The following proposals are made:

- RAN4 to adopt the following change in subclause 5.1B.2.6 Maximum interruption in paging reception:

- For RedCap UE in HD-FDD mode, the UE shall monitor paging for SI change indication in any paging occasion at least once per modification period [2] during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. In case the determined paging occasion overlaps with the CG-SDT transmission, the UE shall determine another paging occasion in the modification period, else if no paging occasion is identified, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.

- Option 1 is supported related to open issue on handover to a BWP with unmeasured SSB of a known cell.

- Option 1a is supported related to open issue related to revision of the intra/inter-frequency definition in handover.

- Proposal 4: Further discussion may be needed on the open issue related to intra-frequency cell measurement.

The companion CR for proposal 1 is submitted in [3].

References

- R4-2303264, “WF on R17 RedCap RRM maintenance”, Ericsson

- R2-2304562, “LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs”, RAN2 #121bis-e

- R4-2309668, “CR 38.133 Correction of RedCap UE behaviour in case of overlap of paging occasion and CG-SDT transmission”, Nokia, Nokia Shanghai Bell