3GPP TSG-RAN WG4 Meeting #107	R4-2309676

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SL positioning

Agenda item:	8.23.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on the SL positioning RRM impacts for Rel-18. The updated WID [1] of Expanded and Improved NR Positioning from 3GPP TSG RAN Meeting #99 is presented below.

Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

Regarding the WF for SL positioning [3], in RAN4 Meeting #106bis-e, the following agreements were made:

## Sub-topic 1-1 Genaral and scenarios #### Issue 1-1-1: Applicable coverage scenarios: Agreements: - RAN4 to define the requirements for SL positioning aiming to support all the coverage scenarios (in-coverage, out-of-coverage and partial coverage). #### Issue 1-1-3: Applicable number of carriers: Agreements: - Define UE requirements for SL PRS-based positioning measurements performed within a single SL BWP and carrier. #### Issue 1-1-6: Applicable gap condition: Agreements: - RAN4 to define the requirements for SL-PRS based measurement without measurement gap. ## Sub-topic 1-2 Core requirements #### Issue 1-3-2: Whether to define report mapping for the following measurement types Agreements: - Define report mapping for the following measurement types: - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - SL-PRS RTOA, - SL-PRS AOA/ZOA

Regarding the WF for SL positioning [3], in RAN4 Meeting #106bis-e, the following way forward and FFS is agreed:

## Sub-topic 1-1 Genaral and scenarios #### Issue 1-1-2: Applicable CBW and SCS: Agreements: - Specify RRM requirement for sidelink positioning up to 40MHz CBW and all supported SCS for sidelink for FR1. - FFS for support of larger CBW for up to 100 MHz #### Issue 1-1-4: Applicable SL PRS patterns: Agreements: - The measurement period requirements (formula) are agnostic to the comb configuration. - FFS the applicable SL PRS patterns for defining accuracy requirements. #### Issue 1-1-5: Applicable number of samples: Agreements: - RAN4 to discuss the number of samples for SL PRS measurement period and for accuracy requirements. #### Issue 1-1-7: Spec structure: < Way forward > - Option 1: - RAN4 to discuss, from spec maintenance point of view, whether common measurement period requirements can be defined for all SL PRS measurements. - Option 2: - RAN4 to define the RRM core and performance requirements in sub clause 9.9X, 10.1X and 13.X for SL positioning RRM core requirements, RRM UE performance requirements and RRM Issue 1-1-8: TerminologygNB performance requirements, respectively. #### Issue 1-1-8: Terminology: < Way forward > - Option 1: - RAN4 to define the RRM core and performance requirements for Sidelink positioning using terminology of “V2X” instead of “SL” in order to maintain uniformity in the spec while referring to V2X/SL requirements, as specified in current specification. - Option 2: - Use the SL terminology in RAN4 requirements. ## Sub-topic 1-2 Core requirements #### Issue 1-2-1: Whether to define measurement period requirements for the following measurement types: Agreements: - Define measurement period requirements for the following measurement types - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - FFS for SL-PRS RTOA, SL-PRS AOA/ZOA #### Issue 1-2-2: How to define the measurement period requirements: < Way forward > - Option 1: - RAN4 waits for more RAN1/2 progress before discussing exact SL PRS measurement period requirements. - Option 2: - RAN4 prioritizes measurement period requirements for SL-PRS-RSRP and further discuss whether the measurement period requirements for other SL PRS-based measurements are different from that for SL-PRS-RSRP. - Option 3: - For the measurement requirements for SL positioning, the R17 PRS measurement requirements can be the baseline taking the following possible differences into account: - UE processing capability - Number of samples - Number of Rx beams - Gap sharing with Uu positioning and/or RRM measurement ## Sub-topic 1-3 Performance requirements #### Issue 1-3-1: Whether to define accuracy requirements for the following measurement types < Way forward > - FFS whether to define measurement accuracy requirements for the following measurement types: - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - SL-PRS RTOA, - SL-PRS AOA/ZOA #### Issue 1-3-3: Channel conditions for accuracy requirements: Agreements: - For the accuracy requirements for SL positioning, RAN4 to discuss the scenarios and channel models used for simulation and wait for more progress from RAN1 on the SL PRS configuration. #### Issue 1-3-4: Reporting granularity Agreements: - Existing report mapping are re-used as baseline for SL PRS measurements. - Further adaptations can be discussed case by case. #### Issue 1-3-5: Side condition < Way forward > - Option 1: - Define requirements for SL PRS measurement based on Es/Iot side condition of [-6, 0 or 3] dB. #### Issue 1-3-6: Reference time for SL PRS based RTOA measurement < Way forward > - Option 1: - RAN4 to discuss whether and how to define absolute accuracy requirements for SL RTOA measurements. - In particular, RAN4 would need to discuss how to account for timing uncertainty of acquiring the RTOA Reference Time (T0) in the requirements. ## Sub-topic 1-4 Timing-related issues #### Issue 1-4-1: Timing error limit requirements Agreements: - RAN4 to discuss the impact of UE timing error requirements on the sidelink positioning measurement period and accuracy requirements. < Way forward > - Option 1: - RAN4 needs to specify a more stringent timing error limit requirement for SL positioning. - Can be necessary the anchor UE’s transmission time error requirement. (e.g. less than 10 nsec which equates to about 3m distance offset) - Option 2: - RAN4 can reuse current Te requirements in order to define the performance requirements. In parallel RAN4 could check feasibility for improved Te requirements. - Option 3: - Reuse the existing Te requirements for SL positioning in Rel-18. - Option 4: - Other options are not precluded. #### Issue 1-4-2: Transmission time adjustment < Way forward > - Option 1: - RAN4 needs to specify transmission time adjustment with high granularity requirement for SL positioning. - Option 2: - No new requirements related to transmit timing adjustment for SL positioning. #### Issue 1-4-3: Synchronization source change Agreements: - RAN4 to discuss the impact of a synchronization source change on an SL positioning measurement (e.g., on measurement performance, measurement procedure, UE behaviour, etc.). ## Sub-topic 1-5 Others #### Issue 1-5-1: Impact of SL-DRX < Way forward > - Option 1: - RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements. - Option 2: - RAN4 to discuss whether the measurement period for SL PRS-based measurements is impacted by the SL-DRX cycle length. #### Issue 1-5-2: Impact of S-SSB < Way forward > - Option 1: - Considering that the resources for SL PRS are TDMed with that for S-SSB, the sharing mechanism such as CSSF or K defined in NR Uu-link cannot be reused for sidelink and a new sharing mechanism between SL PRS and S-SSB should be considered. #### Issue 1-5-3: Impact of UE mobility < Way forward > - Option 1: - Anchor UEs should be capable of simultaneous transmission SL-PRS. - Target UE can be capable of simultaneous SL-PRS measurement with UE capability. - Option 2: - RAN4 to study the impact of change of network coverage scenario of SL UE(s) on measurement requirement, measurement accuracy, etc. #### Issue 1-5-4: Impact on existing RRM requirements Agreements: - RAN4 to study the possible impacts of SL PRS measurement on existing RRM requirements for both Uu and SL. #### Issue 1-5-5: Initiation/cease of SL transmissions for positioning < Way forward > - Option 1: - RAN4 to define the requirements for initiation/cease of SL transmissions for positioning once the UE behavior and procedure is defined in RAN1. #### Issue 1-5-6: The minimum time separation between transmission and reception of SL UE Agreements: - RAN4 to wait for the RAN1 SL positioning RS design and discuss the guard time period issue upon the need.

Agreements achieved in RAN1 #112bis-e related to SL positioning are listed in the Annex.

Discussion

In RAN Meeting #99, the agreed objective is to specify the solutions to support sidelink positioning (including ranging)  in NR systems. From RAN1 perspective, the design of sidelink PRS and its resource allocation mechanism in scheme1 i.e. network centric and scheme 2 i.e. UE autonomous needs to be specified. RAN1/2 also needs to specify the measurements to support SL-RTT, SL-AoA, SL-TDOA, etc. including signaling and associated UE behavior to support unicast, groupcast and broadcast SL PRS transmission in all coverage scenario. RAN3 is to work on signalling for NG-RAN to support sidelink positioning and ranging service authorizations. In this section we discuss scope of RAN4 work, i.e. impacts to RRM requirements due to introduction of SL positioning.

General and Scenarios

Applicable CBW and SCS

#### Issue 1-1-2: Applicable CBW and SCS: Agreements: - Specify RRM requirement for sidelink positioning up to 40MHz CBW and all supported SCS for sidelink for FR1. - FFS for support of larger CBW for up to 100 MHz

    - Based on the WID objective [1] , it is already agreed to specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. Therefore, RAN4 should consider defining the requirements for SL PRS bandwidth of up to 100 MHz.

    - Proposal 1: RAN4 should consider defining the requirements for SL PRS bandwidth of up to 100 MHz

Applicable SL PRS patterns

#### Issue 1-1-4: Applicable SL PRS patterns: Agreements: - The measurement period requirements (formula) are agnostic to the comb configuration. - FFS the applicable SL PRS patterns for defining accuracy requirements.

    - The support of different comb configurations for SL PRS is under discussion in RAN1, both fully and partially staggered patterns will be supported. In Rel-16/17 the accuracy requirement is defined only for fully staggered pattern. However, based on RAN1 agreements for comb configurations, the accuracy requirement can be considered.

    - Proposal 2: RAN4 can wait for RAN1 agreement on SL PRS comb configurations.

Applicable number of samples

#### Issue 1-1-5: Applicable number of samples: Agreements: - RAN4 to discuss the number of samples for SL PRS measurement period and for accuracy requirements.

- As specified in Rel-16/17, the reduced latency measurement with reduced number of samples is supported. RAN4 can consider measurement requirement for both Nsample =1 and Nsample > 1 as required for SL positioning.

    - Proposal 3: RAN4 should discuss if reduced latency measurement needs to be defined for SL positioning.

Core requirements

Whether to define measurement period requirements for the following

- measurement types

#### Issue 1-2-1: Whether to define measurement period requirements for the following measurement types: Agreements: - Define measurement period requirements for the following measurement types - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - FFS for SL-PRS RTOA, SL-PRS AOA/ZOA

- In sidelink positioning the measurements are taken by SL UE for all measurement types, and hence the core requirements need to be well specified for SL-PRS RTOA, SL-PRS AOA/ZOA as well.

- Observation 1: Sidelink UE performs measurements for all positioning measurement types.

    - Proposal 4: RAN4 to define measurement period requirement for SL-PRS RTOA, SL-PRS AOA/ZOA.

How to define the measurement period requirements

#### Issue 1-2-2: How to define the measurement period requirements: < Way forward > - Option 1: - RAN4 waits for more RAN1/2 progress before discussing exact SL PRS measurement period requirements. - Option 2: - RAN4 prioritizes measurement period requirements for SL-PRS-RSRP and further discuss whether the measurement period requirements for other SL PRS-based measurements are different from that for SL-PRS-RSRP. - Option 3: - For the measurement requirements for SL positioning, the R17 PRS measurement requirements can be the baseline taking the following possible differences into account: - UE processing capability - Number of samples - Number of Rx beams - Gap sharing with Uu positioning and/or RRM measurement

- Measurement periods should be aligned for RTOA and SL-PRS-RSRP measurement types and hence should be defined in RAN4.

- Proposal 5: RAN4 to study measurement period requirement based on the combination of RTOA and SL-PRS-RSRP measurements.

Performance requirements

Whether to define accuracy requirements for the following measurement types

#### Issue 1-3-1: Whether to define accuracy requirements for the following measurement types < Way forward > - FFS whether to define measurement accuracy requirements for the following measurement types: - SL-PRS RSRP - SL-PRS RSRPP - SL-PRS RSTD - SL-PRS Rx-Tx - SL-PRS RTOA, - SL-PRS AOA/ZOA

System performance is not reliable if we do not define UE performance requirements. Hence, RAN4 can define the performance requirements for all measurement types.

Proposal 6: RAN4 should define the accuracy requirement for all of the measurement types.

Channel conditions for accuracy requirements

#### Issue 1-3-3: Channel conditions for accuracy requirements: - Proposals: - Option 1: (CATT) - For the accuracy requirements for SL positioning, RAN4 to discuss the scenarios and channel model used for simulation and wait for more progress from RAN1 on the SL PRS configuration. - Option 2: (Intel) - RAN4 can FFS on the different accuracy requirements under the different channel conditions (LOS/NLOS). - Recommended WF - Need discussion.

Partial coverage with single hop is most relevant scenario for which performance requirements should  be defined.

Proposal 7: Measurement accuracy needs to be defined for AWGN and fading channel.

Proposal 8: RAN4 should study the measurement accuracy requirement for SL-PRS assuming partial coverage with single hop scenario.

Side condition

#### Issue 1-3-5: Side condition < Way forward > - Option 1: - Define requirements for SL PRS measurement based on Es/Iot side condition of [-6, 0 or 3] dB.

Es/Iot levels level in SL measurements is different from Uu measurements, as physical distance between the SL UEs is less in comparison to UE distance from gNB. However, it also depends on the channel condition and profile.

Proposal 9: RAN4 can investigate further on the side condition, as Es/Iot condition also depends on the channel condition and profile. These are typically performance requirement and can be discussed further in the performance part.

Reference time for SL PRS based RTOA measurement

#### Issue 1-3-6: Reference time for SL PRS based RTOA measurement < Way forward > - Option 1: - RAN4 to discuss whether and how to define absolute accuracy requirements for SL RTOA measurements. - In particular, RAN4 would need to discuss how to account for timing uncertainty of acquiring the RTOA Reference Time (T0) in the requirements.

- Measurement accuracy performance of anchor should be specified for SL PRS based RTOA, as it should not be left to the UE implementation.

- Proposal 10: RAN4 to study on how to define the conditions for measurement accuracy requirements for SL PRS based RTOA measurement.

Timing-related issues

Timing error limit requirements

#### Issue 1-4-1: Timing error limit requirements Agreements: - RAN4 to discuss the impact of UE timing error requirements on the sidelink positioning measurement period and accuracy requirements. < Way forward > - Option 1: - RAN4 needs to specify a more stringent timing error limit requirement for SL positioning. - Can be necessary the anchor UE’s transmission time error requirement. (e.g. less than 10 nsec which equates to about 3m distance offset) - Option 2: - RAN4 can reuse current Te requirements in order to define the performance requirements. In parallel RAN4 could check feasibility for improved Te requirements. - Option 3: - Reuse the existing Te requirements for SL positioning in Rel-18. - Option 4: - Other options are not precluded.

For the timing-based positioning methods the transmit timing error Te could result in measurement uncertainties, which is even higher SL-PRS Rx-Tx method because of anchor UE.

Proposal 11: RAN4 should consider R17 requirement as baseline and can study if further improvement is required s.t. for RTT based method.

Transmission time adjustment

#### Issue 1-4-2: Transmission time adjustment < Way forward > - Option 1: - RAN4 needs to specify transmission time adjustment with high granularity requirement for SL positioning. - Option 2: - No new requirements related to transmit timing adjustment for SL positioning.

- When the anchor UEs are synchronized to same sync source, due to difference in physical distance of anchor UEs with sync source could lead that could lead to different synchronization offset for different anchor UEs and this timing compensation due to distance between anchor UE and sync source could be discussed in RAN1.

- Proposal 12: The timing compensation due to different propagation delay needs to be agreed at RAN1 discussion.

- Proposal 13: RAN4 needs to investigate the transmit time accuracy from different anchor UEs.

Synchronization source change

#### Issue 1-4-3: Synchronization source change Agreements: - RAN4 to discuss the impact of a synchronization source change on an SL positioning measurement (e.g., on measurement performance, measurement procedure, UE behavior, etc.).

    - When the UE changes its synchronization source to the new synchronization source successfully while the SL positioning measurement is still on-going, the UE behavior is not clear and needs to be clarified.

- Proposal 14: RAN4 to investigate the impact of synchronization source change on SL positioning measurement.

Others

Impact of SL-DRX

#### Issue 1-5-1: Impact of SL-DRX < Way forward > - Option 1: - RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements. - Option 2: - RAN4 to discuss whether the measurement period for SL PRS-based measurements is impacted by the SL-DRX cycle length.

    - The measurement period requirement for both SL-DRX and non-SL-DRX is of importance in SL positioning, as availability of SL PRS at target could not ensure during SL-DRX off period. RAN4 could study if the requirement for SL could be different, or it could be aligned with current requirements.

    - Proposal 15: RAN4 to study the requirement for both SL-DRX and non-SL-DRX case.

Impact of S-SSB

#### Issue 1-5-2: Impact of S-SSB < Way forward > - Option 1: - Considering that the resources for SL PRS are TDMed with that for S-SSB, the sharing mechanism such as CSSF or K defined in NR Uu-link cannot be reused for sidelink and a new sharing mechanism between SL PRS and S-SSB should be considered.

    - Since the transmission of S-SSB is excluded from the resource pool, potentially there could be no overlap amongst SL PRS which is transmitted on resource pool and S-SSB. However, RAN4 can investigate if requirement for CSSF needs to define for SL positioning.

    - Proposal 16: RAN4 can investigate further if legacy CSSF requirement could be used or not.

Impact of UE mobility

#### Issue 1-5-3: Impact of UE mobility < Way forward > - Option 1: - Anchor UEs should be capable of simultaneous transmission SL-PRS. - Target UE can be capable of simultaneous SL-PRS measurement with UE capability. - Option 2: - RAN4 to study the impact of change of network coverage scenario of SL UE(s) on measurement requirement, measurement accuracy, etc.

The coverage scenario of SL UE can change due to mobility, radio condition, etc. In order to locate the target UE, the SL positioning measurement should be performed with minimum interruption, which is subject to change in SL UE network coverage scenario. For example, during SL-RTT based measurement if the anchor UE moves from in-coverage scenario to out-of-coverage scenario during SL positioning measurement period it may lead to interruption and the target UE would need to reinitiate the positioning measurement.

    - Proposal 17: RAN4 to support option 2, the transition from one to another scenario.

- Proposal 18: RAN4 to study the impact of change of network coverage scenario of SL UE(s) on measurement requirement, measurement accuracy, etc.

Initiation/cease of SL transmissions for positioning

#### Issue 1-5-5: Initiation/cease of SL transmissions for positioning < Way forward > - Option 1: - RAN4 to define the requirements for initiation/cease of SL transmissions for positioning once the UE behavior and procedure is defined in RAN1.

    - This is an important aspect to improve the performance of sidelink positioning. Based on the procedure and behavior of UE defined in RAN1, RAN4 can define the requirements for Initiation/cease of SL transmissions

- Proposal 19: RAN4 to define the requirements for initiation/cease of SL transmissions for positioning once the UE behavior and procedure is defined in RAN1.

Conclusion

In this paper, we provided our views on the SL positioning RRM impacts for Rel-18.

The following observations and proposals are made:

- Proposal 1: RAN4 should consider defining the requirements for SL PRS bandwidth of up to 100 MHz.

    - Proposal 2: RAN4 can wait for RAN1 agreement on SL PRS comb configurations.

    - Proposal 3: RAN4 should discuss if reduced latency measurement needs to be defined for SL positioning.

    - Observation 1: Sidelink UE performs measurements for all positioning measurement types.

    - Proposal 4: RAN4 to defined measurement period requirement for SL-PRS RTOA, SL-PRS AOA/ZOA.

Proposal 5: RAN4 to study measurement period requirement based on the combination of RTOA and SL-PRS-RSRP measurements.

Proposal 6: RAN4 should define the accuracy requirement for all of the measurement types.

Proposal 7: Measurement accuracy needs to be defined for AWGN and fading channel.

Proposal 8: RAN4 should study the measurement accuracy requirement for SL-PRS assuming partial coverage with single hop scenario.

Proposal 9: RAN4 can investigate further on the side condition, as Es/Iot condition also depends on the channel condition and profile. These are typically performance requirement and can be discussed further in the performance part.

Proposal 10: RAN4 to study on how to define the conditions for measurement accuracy requirements for SL PRS based RTOA measurement.

Proposal 11: RAN4 should consider R-17 requirement as baseline and can study if further improvement is required s.t. for RTT based method.

Proposal 12: The timing compensation due to different propagation delay needs to be agreed at RAN1 discussion.

Proposal 13: RAN4 needs to investigate the transmit time accuracy from different anchor UEs.

Proposal 14: RAN4 to investigate the impact of synchronization source change on SL positioning measurement.

Proposal 15: RAN4 to study the requirement for both SL-DRX and non SL-DRX case.

Proposal 16: RAN4 can investigate further if legacy CSSF requirement could be used or not.

Proposal 17: RAN4 to support option 2, the transition from one to another scenario.

Proposal 18: RAN4 to study the impact of change of network coverage scenario of SL UE(s) on measurement requirement, measurement accuracy, etc.

Proposal 19: RAN4 to define the requirements for initiation/cease of SL transmissions for positioning once the UE behavior and procedure is defined in RAN1.

- References

- RP-230328, Revised WID on Expanded and Improved NR positioning, 3GPP TSG RAN#99.

- 3GPP TS-38.133, “Requirements for support of radio resource management (Release 17)”

- R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT

- R4-2306246, Topic summary for [106bis-e][215] NR_pos_enh2_part2, Moderator (CATT)

- Annex: RAN1 #112bis-e Agreements on SL positioning

- 9.5.1.1 SL positioning reference signal Agreement For SL PRS sequence generation, no additional parameters other than the following input parameters are used: slot number, symbol number, and the parameter . Agreement TDM-based multiplexing in a slot of SL PRS from different UEs is NOT supported for a shared resource pool. Agreement SL PRS resource sets are not defined in Rel-18. Agreement (M, N) patterns with M > N with full staggering are supported. - In the last (M-N) symbols, the SL PRS symbols are repeated with same order of comb offsets as in the first N symbols. Agreement An AGC symbol preceding a SL PRS resource is not considered as part of the SL PRS resource itself. Agreement For the SL PRS open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss. - The same principle as for PSSCH power control is applied for deciding which (i.e., SL, DL, or SL and DL) pathloss to use. - FFS: SL pathloss reference for open-loop power control for SL PRS. R1-2303964 FL summary #2 on SL positioning reference signal Moderator (Intel Corporation) Agreement At least for dedicated SL PRS resource pools, in addition to already-agreed (M, N) = (2, 2), (4, 4), fully staggered pattern with (M, N) = (6, 6) is supported. - FFS: Other values of (M, N). - FFS: Applicability to shared resource pools. Agreement - A SL PRS resource refers to a time-frequency resource within a slot of a dedicated SL PRS resource pool that is used for SL PRS transmission. - FFS: for a shared resource pool - Characteristics associated with a SL PRS resource include at least: - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation, - Note: Additional parameters can be included as/when identified. - FFS: other time domain aspects, if any - A SL PRS resource is identified by a SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. NOTE 1: The above does not imply need for signalling/(pre-)configuration of all these parameters Agreement For SL PRS sequence generation, one of the following options is down-selected to define the parameter : - Option 1: is a higher layer parameter. - FFS: How the higher layer parameter is obtained, e.g., (pre-)configuration or via LPP/SLPP, etc. - Option 2: is based on 12 LSB bits CRC of PSCCH associated with the SL PRS. - Option 3: based on a combination of higher layer parameter from a configured ID list and 12 LSB bits of CRC of PSCCH associated with the SL PRS. - FFS: How the higher layer parameter/ID list is determined/obtained, e.g., (pre-)configuration or via LPP/SLPP, etc. Agreement For a dedicated SL PRS resource pool, options for SL pathloss reference for OLPC for SL PRS are (to be down-selected from): - Option 1: SL PRS as pathloss reference - Option 2: PSCCH DMRS as pathloss reference - Option 3: Both Options 1 and 2 - FFS: Selection between Option 1 and Option 2, including (pre-)configuration. Agreement For shared resource pools, a UE does not map SL-PRS and PSSCH DMRS in the same OFDM symbol(s). Agreement For SL pathloss-based OLPC for SL PRS in unicast, filtered RSRP is reported by a receiving UE. - Measurements and reporting for SL positioning Conclusion - For SL-PRS based RTOA definition, the selection between SFN0 vs DFN0 and SFN vs DFN is determined based on the selection of the synchronization source for SL PRS transmission, as in legacy specifications. - When the UE selects a cell as the synchronization reference source, SFN0/SFN is used for SL-PRS based RTOA. - Otherwise, DFN/DFN0 is used for the definition of the SL-PRS based RTOA. - FFS on indication of whether SFN or DFN is used along with the RTOA measurement reporting Agreement - SFN/DFN Initialisation Time can be provided by UE or by LMF. - FFS: which UEs can provide initialisation time (note: which may be decided by other WGs) - FFS: further details of the definition of DFN Initialisation Time - For the definition of SL-PRS based RTOA, update the definition of reference timing as: - the reference timing is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0 provided by SFN/DFN Initialisation Time - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS: The timing of SFN0/SFN for RTOA reference timing is determined by advancing DL SFN0/SFN with TA/2 when the UE is in-coverage and in connected state Agreement Support both the case with and without translation of the LCS to GCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. Agreement SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a transmitting UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901. The SL AoA is determined at the receiving UE’s antenna(s) for a SL channel corresponding to the transmitting UE. Agreement Support SL-based RSTD, Rx-Tx time difference, RToA, AoA, RSRPP measurement and report for the first path and optionally additional path. - No specification impact for how to set the additional path measurements - From RAN1 perspective, no performance requirements are expected to be defined for the additional-path measurements in Rel-18. Agreement For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE. FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs) FFS on quality information of anchor UE location information. Agreement Support per ARP based measurement in sidelink positioning. The ARP related information can be reported along with the SL measurement. FFS on details of ARP related information, including whether TEG ID can be reused for such purpose. R1-2303957 Summary #2 on Measurements and reporting for SL positioning Moderator (vivo) R1-2303958 Summary #3 on Measurements and reporting for SL positioning Moderator (vivo) Agreement For definition of SL-PRS based Rx-Tx measurement, further consider Alt1 and Alt3 until RAN1#113: - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu Agreement SL reference signal time difference (SL RSTD) is the SL relative timing difference between the UE j and the reference UE i, defined as TSubframe_SL-Rxj – TSubframe_SL-Rxi, where: - TSubframe_SL-Rxj is the time when the UE receives the start of one subframe from UE j. - TSubframeSL-Rxi is the time when the UE receives the corresponding start of one subframe from UE i that is closest in time to the subframe received from UE j. FFS: whether or not impact due to mobility or synchronization timing change should be considered for SL RSTD Agreement Support higher layer signaling for sidelink positioning measurement report and report triggering. - Up to RAN2 to discuss detailed signaling design. FFS on SCI based report triggering. - Resource allocation for SL positioning reference signal Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details Agreement For a dedicated resource pool for positioning: - No additional slots are needed to be supported Agreement For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier. - A UE can be (pre-)configured with one or more dedicated SL resource pools. - A UE can be (pre-)configured with one or more shared SL resource pools. Agreement Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool. - Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation. Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool Agreement With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication - FFS: Details including a new or existing 2nd stage SCI Agreement In a shared resource pool: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. A.1: Only TDMing is supported - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. B.1: Only TDMing is supported - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting): - Alt. C.1: 2nd SCI only - Alt. C.2: 2nd SCI and SL-SCH - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH” - FFS: Handling of PT-RS and SL-PRS Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH Agreement For the scheme 2 sensing-based resource allocation: - Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Note: This means that Rel-17 partial sensing is not considered a starting point for the design Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Modification 6: For the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - Option 3: Provided by higher layers with values TBD - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%): - Options TBD - Modification 8: For the pre-emption of the reserved resources: - Options TBD - Note 1: Other potential modifications and/or other options within each modification are not precluded - Note 2: Multiple options for each potential modification may be supported Agreement In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS