3GPP TSG-RAN WG4 Meeting #107	R4-2309700

# Incheon, Korea, May 22nd – May 26th, 2023

# Title:	Initial NTN calibration results for above 10 GHz Coexistence Studies

# Source:	THALES, Magister Solutions Ltd

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.27.2

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At RAN4#105 the following WFs have been approved:

- R4-2220239 (source: THALES, “WF for above 10GHz band definition and system parameters”) with the latest references for Ka-Band frequency bands;

- R4-2220241 (source: Samsung, “Simulation assumptions for NTN co-existence study in bands above 10GHz”) with the latest discussions for the coexistence studies.

Agreements R4-2220239 1/ Companies are invited to study (general) ITU/national regulations and bring contributions/provide more information at this meeting (RAN#105) and next RAN4 meeting. 2/ Since it has been agreed to define several NTN Ka-bands to address the diversity of spectrum allocation (see agreement 15/11/2022 during online session), moderator proposes recommendation based on Option 2b. Option 2b (as starting point for next meeting discussion): RAN4 to consider defining/continue to discuss at next meeting (all options have same priority, companies need more time to check): - n511 with consideration of US/FCC regulations. - [n512 with consideration of CEPT regulations.] Note 1: All companies to further check impacts of latest revision of ECC Decision(05)01 on ECC Decision(13)01 cited in the current WF. - [n510 with consideration of US/FCC regulations.] Note 1: Provide the 3GPP definition that specifies the prevention of the use of a fixed terminal in the Fixed Service Spectrum (FSS). Note 2: Also provide information of a movable NTN user terminal in FSS spectrum in the US. with the following considerations: - DL: 17.7-20.2 GHz (n512, n511, n510); - UL: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510).

Agreements R4-2220241 Table 1.1-1 Aggressor and victim combination No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL. 2; TN with NTN; TN UL; NTN UL. 3; TN with NTN; NTN UL; TN DL. 4; TN with NTN; TN DL; NTN UL. 5; TN with NTN; TN DL; NTN DL. 6; TN with NTN; NTN DL; TN DL. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

Moreover, at RAN4#106 the following agreements have been made:

Agreement 1: Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Furthermore, for the assumption on TN ACLR/ACS for co-existence simulation, the values for 17 GHz in below table are considered as starting point for co-existence simulation purpose yet other options not precluded.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Agreement 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

The following WF documents shall be also taken into account:

- R4-2217468, WF on [313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, 10 - 19 October, 2022;

- R4-2220241, Way forward on [105][313] NR_NTN_enh_Part2 (Samsung), 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, November 14 – November 18, 2022;

- R4-2302878, WF for above 10GHz NTN-TN co-existence study (Samsung), 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023.

In previous contribution R4-2302535 (“NTN Simulation Parameters for above 10 GHz Coexistence Studies”, THALES), several NTN simulation parameters were proposed for above 10 GHz coexistence studies. The contribution R4-2305847 (THALES, Magister Solutions Ltd) provided material for discussion with respect to Ka-Band NTN-TN coexistence simulations and NTN UE terminal and Satellite antenna parameters. Parameters from R4-2302535 then were updated in R4-2305847, together with initial simulation results.

At RAN4#106-bis-e, the following open issues have been also captured in R4-2306002 (“WF on [311] NR_NTN_enh_Part3”, Samsung):

Issue 3-2-1: NTN SAN UL parameters – NF

- Proposals

  - Option 1: 3.5

  - Option 2: 5.9

  - Option 3: Other values

- Recommended WF

  - Pause the discussion by this meeting

  - Companies are encouraged to bring more technical analysis for this issue

Issue 3-4-1: NTN UE parameters - NF

- Proposals:

  - Option 1: 2.1 dB (baseline assumption)

  - Option 2: 5.9 dB

  - Option 3: Other values

- Recommended WF

  - Further discuss the value for co-existence simulation.

For other information, please also check R4-2306003 (“Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung).

# Discussion with respect to Coexistence Use Cases

For the calibration phase, FRF=1 was used. However, for NTN coexistence analysis (simulation phase) in above 10 GHz, it is recommended to use an FRF>1 (e.g. 2 or 3) for NTN.

Proposal 5: Use (for NTN) an FRF>1 (e.g. 2 or 3) for NTN coexistence analysis in above 10 GHz.

Proposal 6: Preferably use (for NTN) an FRF=2 with two polarizations (RHCP, LHCP) for NTN coexistence analysis in above 10 GHz.

NF SAN=5.9 dB was used for calibration purpose. For coexistence simulations, preferably use NF SAN=3.5 dB as compromise.

Proposal 7: Preferably use NF SAN=3.5 dB for all constellations (GEO, LEO@1200, LEO@600) for NTN coexistence analysis in above 10 GHz.

Proposal 8: Companies proposing NF=5.9 dB for NTN SAN coexistence simulations in above 10 GHz need to provide a justification paper.

NF UE=4dB was used for calibration purpose. For coexistence simulations, preferably use NF=2.1 dB for NTN UE.

Proposal 9: For coexistence simulations in above 10 GHz, preferably use satcom NF NTN UE=1.2 dB from TS 38.821 and TS 38.811 (equivalent NF=2.1 dB) for NTN UE.

Proposal 10: Companies proposing NF=4 dB for NTN UE coexistence simulations in above 10 GHz need to provide a justification paper.

Proposal 11: As baseline, adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern (equivalent equations as also described in TR 38.863):

ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2)

- with ABS [degree]=180/pi x ABS[rad] and

- with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern.

# Parameters for calibration phase (results in attached Excel file)

The following parameters have been used for calibration phase:

NTN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UEs per cell | 10 |
| UE height | 1.5 m |
| UE Tx power | 3.0 dBW |
| UE antenna gain dBi | Tx = 43.2, Rx = 39.7 |
| UE antenna diameter | 0.60 m |
| UE noise figure | 4 dB |
| Satellite antenna type | R4-2306003, SAN circular aperture (TR 38.811 Bessel) |
| UE antenna type | R4-2306003, UE circular aperture (TR 38.811 Bessel) |
| Shadowing | Enabled |
| Uplink Tx power control | R4-2306003, CLxile = 88.0 + 10*log10(200/BW [Mhz]) dB |
| Cell selection | RSRP |
| Frequency re-use | 1 |

NTN Scenario Specific Parameters

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite Tx power | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite antenna gain dBi | 58.5 | 38.5 | 38.5 |
| Noise figure | 5.9 dB | 5.9 dB | 5.9 dB |
| G/T dB/K | 28 | 8 | 8 |
| Beamwidth deg [to be confirmed/updated] | [0.1763] | [1.7337] | [1.7337] |
| ABS (Adjacent Beam Spacing) deg | 0.1527 | 1.5014 | 1.5014 |

TN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UE height | 1.5 m |
| gNB antenna element gain dBi | 5.5 |
| gNB antenna elements | 16x8 |
| gNB antenna beamwidth deg | Horizontal = 90.0, Vertical = 90.0 |
| Shadowing | Enabled |
| UE Tx power | 22.4 dBm EIRP (max EIRP including antenna gain) |
| Tx power control | R4-2306003, CLxile = 88.0 + 10*log10(200/BW [Mhz]) dB |
| UE antenna type | 3GPP TR 38.901, 2x2 antenna array with 5.0 dBi element gain |
| gNB antenna type | 3GPP TR38.901, 16x8 antenna array with RAN4 agreements |
| BS NF | 10 dB |
| UE NF | 10 dB |

# Proposals resubmitted/updated from R4-2302535 and R4-2305847

First, coexistence parameters should be chosen to allow NTN broadband connectivity in above 10 GHz.

Proposal 1: Coexistence parameters (i.e. for SAN, NTN UE) for simulation phase should be chosen to allow NTN broadband connectivity in above 10 GHz.

This means for instance that RAN4 should consider sufficient transmission power, antenna gain, adapted RB allocation, to allow a reasonable link budget between NTN UE and SAN in above 10 GHz.

Proposal 2: RAN4 should consider sufficient transmission power, antenna gain, relevant NTN RB allocation, to allow a reasonable link budget (e.g. above 10 dB) between NTN UE and SAN in above 10 GHz.

Moreover, initial simulation results showed that FRF>1 is necessary to achieve this broadband connectivity because satellite beams are overlapping which may cause increase interference not corresponding to broadband NTN communication.

RAN4 could consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB). Either Option 1 or Option 2 (optimised with respect to secondary lobes) can be used, with a preference for Option 2:

| Options | Equations | Half-power beam-width 2θ-3dB Value |
| --- | --- | --- |
| Option 1 (see TR 38.811) | Fθ=2 J1uu | 2θ-3dB=2×Arcsin1.616×λπD |
| Option 2 (new equation) | Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=2×Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

However, as a compromise, Option 1 should be sufficient. Moreover the following parameters were approved at RAN4#106-bis-e in R4-2306002:

Ka-Band DownLink (i.e. ~17 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,9 | 0,6 | 0,6 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

with the corresponding UL assumptions from R4-2305847:

Ka-Band UpLink (i.e. ~27 GHz for UL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,66 | 0,36 | 0,36 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 3,5 | 3,5 | 3,5 |

Based on the (agreed & approved) DL parameters, therefore, the following proposal can be made:

Proposal 3: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB).

| Equation | Half-power beam-width 2θ-3dB Value | GEO constellation type @ 17GHz | LEO constellation type @ 17GHz |
| --- | --- | --- | --- |
| Fθ=2 J1uu | 2θ-3dB=2×Arcsin1.616×λπD | 0.1763 degrees | 1.7337 degrees |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 4: RAN4 to select between [Option1] and [Option2] to define the NTN UE parameters:

[Option1] For the coexistence analysis in Ka-Band, RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

[Option2] RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

# Conclusions

Proposal 1: Coexistence parameters (i.e. for SAN, NTN UE) for simulation phase should be chosen to allow NTN broadband connectivity in above 10 GHz.

Proposal 2: RAN4 should consider sufficient transmission power, antenna gain, relevant NTN RB allocation, to allow a reasonable link budget (e.g. above 10 dB) between NTN UE and SAN in above 10 GHz.

Proposal 3: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB).

| Equation | Half-power beam-width 2θ-3dB Value | GEO constellation type @ 17GHz | LEO constellation type @ 17GHz |
| --- | --- | --- | --- |
| Fθ=2 J1uu | 2θ-3dB=2×Arcsin1.616×λπD | 0.1763 degrees | 1.7337 degrees |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 4: RAN4 to select between [Option1] and [Option2] to define the NTN UE parameters:

[Option1] For the coexistence analysis in Ka-Band, RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

[Option2] RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

Proposal 5: Use (for NTN) an FRF>1 (e.g. 2 or 3) for NTN coexistence analysis in above 10 GHz.

Proposal 6: Preferably use (for NTN) an FRF=2 with two polarizations (RHCP, LHCP) for NTN coexistence analysis in above 10 GHz.

Proposal 7: Preferably use NF SAN=3.5 dB for all constellations (GEO, LEO@1200, LEO@600) for NTN coexistence analysis in above 10 GHz.

Proposal 8: Companies proposing NF=5.9 dB for NTN SAN coexistence simulations in above 10 GHz need to provide a justification paper.

Proposal 9: For coexistence simulations in above 10 GHz, preferably use satcom NF NTN UE=1.2 dB from TS 38.821 and TS 38.811 (equivalent NF=2.1 dB) for NTN UE.

Proposal 10: Companies proposing NF=4 dB for NTN UE coexistence simulations in above 10 GHz need to provide a justification paper.

Proposal 11: As baseline, adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern (equivalent equations as also described in TR 38.863):

ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2)

- with ABS [degree]=180/pi x ABS[rad] and

- with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern.

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220241, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload”, S. K. Jain and all.

- ITU-R Rec. P.618-12.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

- R4-2305847, “Updated NTN Simulation Parameters for above 10 GHz Coexistence Studies and Initial Simulation Results”, THALES, Magister Solutions Ltd.

- R4-2306002, “WF on [311] NR_NTN_enh_Part3”, Samsung.

- R4-2306003, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

# ANNEX - Initial NTN simulation results in above 10 GHz (from R4-2305847, RAN4#106-bis-e)

First, coexistence parameters should be chosen to allow NTN broadband connectivity in above 10 GHz.

NTN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UEs per cell | 10 |
| UE height | 1.5 m |
| UE Tx power | 3.0 dBW |
| UE antenna gain dBi | Tx = 42.9, Rx = 39.0 |
| UE antenna diameter | 0.60 m |
| UE noise figure | 2.1dB |
| Satellite antenna type | Circular aperture RAN4 THALES |
| UE antenna type | ITU-R S.465-6 |
| Shadowing | Enabled |
| Uplink Tx power control | Disabled |
| Cell selection | RSRP |
| Frequency re-use | 1 and 3 |

NTN Scenario Specific Parameters

|  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite Tx power | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite antenna gain dBi | 58.5 | 38.5 | 38.5 |
| Noise figure | 3.5 dB | 3.5 dB | 3.5 dB |
| G/T dB/K | 30.4 | 10.4 | 10.4 |
| Beamwidth deg | 0.1884 | 1.9012 | 1.9012 |

As seen from the next simulation results, FRF>1 is preferable for broadband NTN communications above 10 GHz.

3.1 Results for FRF1

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the expected SINR is quite low for broadband communication purpose.

3.2 Results for FRF3

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=3 is an option for broadband NTN communications above 10 GHz.

3.3 Results for FRF2 with dual polarization

1/ Coupling loss

2/ Carrier signal power

3/ SINR

As seen from simulation results, FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

Observation 1: FRF=1 is not preferable for broadband NTN communications above 10 GHz, since the SINR values are limited due to inter-neighbour beam interference.

Observation 2: FRF=3 is an option for broadband NTN communications above 10 GHz.

Observation 3: FRF=2 with dual polarization is an option for broadband NTN communications above 10 GHz.

# ANNEX - Initial TN simulation results in above 10 GHz (from R4-2305847, RAN4#106-bis-e)

TN Common Parameters

| Bandwidth | 200 MHz, numerology 3 (132 PRBs) |
| --- | --- |
| Downlink center frequency | 17 GHz |
| Uplink center frequency | 27 GHz |
| UE height | 1.5 m |
| gNB antenna element gain dBi | 5.5 |
| gNB antenna elements | 16x8 |
| gNB antenna beamwidth deg | Horizontal = 90.0, Vertical = 90.0 |
| Shadowing | Enabled |
| UE Tx power | 22.4 dBm EIRP |
| Tx power control | R4-2120671, CLxile = 71.0 + 10*log10(200/BW [Mhz]) dB |
| UE antenna type | 3GPP TR38.901, 2x2 antenna array with 5.0 dBi element gain |
| gNB antenna type | 3GPP TR38.901, 16x8 antenna array with RAN4 agreements |

1/ Coupling loss

2/ Carrier signal power

3/ SINR

-END-