3GPP TSG-RAN WG4 Meeting #107	R4-2309739

Incheon, South Korea, 22 – 26 May 2023

Source:	Skyworks Solutions, Inc.

Title:	Remaining items on new template for EN-DC cross-band isolation MSD

Agenda item:			8.1.3 Simplification of specification and reduction of test burden	[FS_SimBC]

Document for:	Approval

# Introduction

At the last RAN4 meeting #106b-e, the agreed way forward (WF) [1] captured guidelines on how to specify MSD test points due to cross-band isolation interference for EN-DC. These guidelines are inherited from the new table template that was adopted in Release-17 for NR-CA TS 38.101-1. At this meeting, we tentatively propose a CR to migrate all power class 3 (PC3) and PC2 test points under this new template. This contribution captures the key changes, as well as items that require further study to ensure technical correctness and compliance with WF[1] guidelines. We also briefly discuss test points DC_12_n71 and DC_71_n12 that were treated in a separate WF [2].

# Discussion

### 2.1 PC3 DC_11_n3 and DC_3_n51 MSD test point completion [1]

Background

It was agreed in WF-5 [1] to further study (FFS) the evaluation of the band 11 and the band n51 MSD  as shown in Table 1 below.

Table 1: DC_11_n3 and DC_3_n51

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Guidelines compliant - Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | 11 | 1735 | 50 | 15 | 50 (RBstart=0) | 1493.4 | 5 | [FFS] | 1st test point |
| 3 | n51 | 1720 | 20 | 15 | 50 (RBstart=0) | 1429.5 | 5 | [FFS] | 1st test point |

The current Release 18.1.0 MSD specifications assume:

- For band B11 5MHz channel bandwidth (CBW) MSD: band n3 uplink (UL) RB allocation (Lcrb) of 25RB, 5MHz CBW

- For band n51 5MHz CBW MSD: band B3 UL Lcrb of 25RB and 5MHz UL CBW.

MSD re-evaluation

For the band B11 MSD at downlink (DL) carrier 1493.5MHz, our measurements captured in Figure 1 indicate that the power amplifier (PA) interference level is not impacted by changing the UL band configuration from 5MHz CBW/Lcrb 25(RBstart=0) (blue trace) to 50MHz CBW/Lcrb= 50(RBstart=0) (orange trace). The same observation is made for band n51 due to band B3 UL configuration change from 5MHz Lcrb 25 (RBstart=0) to 20MHz Lcrb=50 (RBstart=0).

Figure 1:  Measured band n3 PA noise emissions at 27dBm with -28dB image rejection, -28dB LO leakage, -60dBc CM-IM3 and -70dBc C-IM5. Blue: UL CBW 5MHz Lcrb=25(RBstart=0), orange UL CBW=50MHz Lcrb=50(RBstart=0).

We therefore propose to adopt the legacy MSD levels into the new table template.

Proposal 1: For DC_11_n3 and DC_3_n51, adopt the Table 2 MSD test points. The 2nd “optional” MSD test points have been agreed in WF [1].

Table 2: PC3 MSD test points for DC_11_n3 and DC_3_n51

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | 11 | 1735 | 50 | 15 | 50 (RBstart=0) | 1493.4 | 5 | 6.4 | >ACLR2 |
| n3 | 11 | 1712.5 | 5 | 15 | 25 (RBstart =0) | 1493.4 | 5 | [2.6] | >ACLR2 |
| 3 | n51 | 1720 | 20 | 15 | 50 (RBstart=0) | 1429.5 | 5 | 6.4 | >ACLR2 |
| 3 | n51 | 1712.5 | 5 | 15 | 25 (RBstart =0) | 1429.5 | 5 | [2.6] | >ACLR2 |

### 2.2 PC3 DC_12_n71 and DC_71_n12 MSD test points

Proposal 2: Based on WF [2] agreements, it is proposed to adopt the Table 3 MSD test points for DC_12_n71 and DC_71_n12.

Table 3: PC3 MSD test points for DC_12_n71 and DC_71_n12

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| 71 | n12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| 12 | n71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |
| n12 | 71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

### 2.4 DC_48_n46 PC3 MSD test points

Background on Rel-18 current test-point:

The band B48 MSD is 13.3dB at 5MHz for band n46 UL 80MHz CBW and 216RB UL RB allocation is summarized in Table 4. These specifications are incorrect since band B48 maximum supported CBW is 20MHz. We also notice from Table 5 that the UL configurations for band n46 are restricted to 80MHz CBW. Compliance to WF [1] guidelines require the band 48 MSD is re-evaluated for band n46 maximum UL CBW of 100MHz.

Table 4: PC3 B48 MSD test-points for DC_48_n46

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 | 48 | 13.3 | 10.4 | 8.8 | 7.8 | 7.8 | 7 | 6.5 | 5.7 | 5.4 | 5.1 |

For the band n46 MSD, we notice from Table 5 that the UL configurations for band B48 are erroneous since B48 can not support Lcrb=216RB SCS 15kHz due to a maximum supported UL CBW of 20MHz.

Table 5: UL configurations for DC_48_n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 48 | n46 | 15 |  |  |  | 216 |  |  | 216 |  | 216 |  | 216 |  |  |
| n46 | 48 | 30 | 216 | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |

Observation: There are several technical errors in the specifications of the DC_46_n48 MSD test points due to cross-band isolation:

- For the band B48 MSD:

  - the MSD can not be specified for CBW greater than 20MHz,

  - the band n46 UL CBW needs to be increased to 100MHz,

- For the band n46 MSD:

  - the band B48 UL CBW can not be specified for CBW that exceed 20MHz CBW.

Proposal 3: Due to limited time to bring technical corrections, it is proposed to port the legacy agreed test points to the new table template as captured in Table 6 below. The band B48 MSD test point is inherited from CA_n46-n48. The band n46 MSD is proposed in brackets until further study is brought.

Table 6: DC_46_n48 PC3 MSD test points for new table template .

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n46 | 48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| 48 | n46 | 3690 | 20 | 15 | 100 (RBstart=6) | 5155 | 10 | [7] | >ACLR2 |

Proposal 4: Interested companies are invited to evaluate the band B48 MSD for the test points of Table 7. Further checks are also welcome for the band n46 [7]dB MSD (Table 6).

Table 7: UL configurations for DC_48_n46

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n46 | 48 | 5200 | 100 | 30 | 270 (RBstart=0) | 3697.5 | 5 | [FFS] | >ACLR2 |

### 2.5 Handling of clause 7.3B.2.3.6

Background on Rel-18 current test-point:

Clause 7.3B.2.3.6 was agreed to account for concerns on the impact on cross-band isolation MSD due to the request for increasing UL CBW, for example for DC_3_n1. Additional test points were agreed to ensure gNB schedulers are aware that certain UL configurations may severely impact the DL affected band reference sensitivity. These test points are reproduced in Table 8.

Table 8: Reference sensitivity exceptions (MSD) due to Tx non-linearity interference in 1st or 2nd adjacent channel of UL band for EN-DC in NR FR1

| UL band | DL band | SCS of UL band (kHz) | LCRB of UL band | Applicable UL BW(MHz) | MSD value of DL band (dB) | The adjacent channel of UL band |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 3 | 15 | 25 | ≥ 25 | 4.5 | 2nd adjacent channel |
| n1 | 3 | 15 | 25 | 50 | 17 | 1st adjacent channel |
| n5 | 28 | 15 | 6 | ≥ 15 | 7.9 | 2nd adjacent channel |
| n40 | 1 | 30 | 25 | ≥ 70 | 21.5 | 2nd adjacent channel |

These test points become redundant with the introduction of the new table template. For example, the new template compliant MSD test points for DC_3_n1 proposed in Table 9 below may be considered as addressing the initial concerns and motivations for creation of clause 7.3B.2.3.6.

Table 9: PC3 MSD test points for DC_3_n1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | 3 | 1922.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n1 | 3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | ACLR1 |

It is therefore proposed to void clause 7.3B.2.3.6.

Proposal 5: Void clause 7.3B.2.3.6 due to duplication of test points with the new table template.

# 3. Conclusion

This contribution is a companion discussion paper to the change request that introduces the new table template to specify the EN-DC MSD due to cross-band isolation. Further evaluation is required for DC_46_n48 at next meeting, interested companies are invited to review these proposals.

Proposal 1: For DC_11_n3 and DC_3_n51, adopt the Table 2 MSD test points. The 2nd “optional” MSD test points have been agreed in WF [1].

Table 2: PC3 MSD test points for DC_11_n3 and DC_3_n51

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | 11 | 1735 | 50 | 15 | 50 (RBstart=0) | 1493.4 | 5 | 6.4 | >ACLR2 |
| n3 | 11 | 1712.5 | 5 | 15 | 25 (RBstart =0) | 1493.4 | 5 | [2.6] | >ACLR2 |
| 3 | n51 | 1720 | 20 | 15 | 50 (RBstart=0) | 1429.5 | 5 | 6.4 | >ACLR2 |
| 3 | n51 | 1712.5 | 5 | 15 | 25 (RBstart =0) | 1429.5 | 5 | [2.6] | >ACLR2 |

Proposal 2: Based on WF [2] agreements, it is proposed to adopt the Table 3 MSD test points for DC_12_n71 and DC_71_n12.

Table 3: PC3 MSD test points for DC_12_n71 and DC_71_n12

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| 71 | n12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| 12 | n71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |
| n12 | 71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

Observation: There are several technical errors in the specifications of the DC_46_n48 MSD test points due to cross-band isolation:

- For the band B48 MSD:

  - the MSD can not be specified for CBW greater than 20MHz,

  - the band n46 UL CBW needs to be increased to 100MHz,

- For the band n46 MSD:

  - the band B48 UL CBW can not be specified for CBW that exceed 20MHz CBW.

Proposal 3: Due to limited time to bring technical corrections, it is proposed to port the legacy agreed test points to the new table template as captured in Table 6 below. The band B48 MSD test point is inherited from CA_n46-n48. The band n46 MSD is proposed in brackets until further study is brought.

Table 6: DC_46_n48 PC3 MSD test points for new table template .

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n46 | 48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| 48 | n46 | 3690 | 20 | 15 | 100 (RBstart=6) | 5155 | 10 | [7] | >ACLR2 |

Proposal 4: Interested companies are invited to evaluate the band B48 MSD for the test points of Table 7. Further checks are also welcome for the band n46 [7]dB MSD (Table 6).

Table 7: UL configurations for DC_48_n46

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |

Proposal 5: Void clause 7.3B.2.3.6 due to duplication of test points with the new table template.

# References

[1]	R4-2306582 WF on cross-band isolation MSD simplification for EN-DC, 3GPP TSG-RAN WG4 Meeting # 106bis-e , Electronic meeting, Skyworks Solutions, Inc.

[2]	R4-2306479 WF on CA_n71-n85 and related band 12 and 71 combinations, 3GPP TSG-RAN WG4 Meeting # 106bis-e , Electronic meeting, T Mobile USA, Apple, Murata, Nokia, Skyworks.