3GPP TSG-RAN WG4 Meeting #107                                                                                             R4-2309770

Incheon, KR, May 22 – Mat 26, 2023

Source: 	CATT

Title: 	WF on above 10 GHz SAN RF requirements

Agenda item:	8.27.6

Document for:	Approval

- Introduction

This contribution provides the WF for the remaining issues for above 10 GHz SAN RF requirements.

- WF

OTA reference sensitivity level

The OTA reference sensitivity level will be defined

Where

PkT = -174 dBm/Hz

NF will be aligned with the co-existence study assumption conclusion,

SNR FFS,

IM = 2 dB,

The following is used for the starting point for G:

G = [50.5-58.5] dBi, [30.5-38.5] dBi, and [30.5-38.5] dBi for GEO, LEO-1200, and LEO-600.

Note: The above Rx max Gain ranges are taken from Table 6.1.1.1-1 and Table 6.1.1.1-2 of  TR 38.821.

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

FRC can be discussed in future meetings.

Out of band blocking

FFS if this requirement is needed. Companies are encouraged to contribution on the justification for the necessity of this requirement.

- References

[1] R4-2307382, “Further discussion on SAN RF requirements for above 10GHz bands”, CATT, RAN4#107