3GPP TSG-RAN WG4 Meeting # 107      	R4-2309796

Incheon, KR, May 22 – May 26, 2023

Title: 	WF for ATG demodulation requirements

Agenda Item:	8.14.6

Source: 	CMCC

Document for:	Approval

- General

Issue 1-1: Channel model:

Agreement:

- Only consider single path AWGN channel with Doppler for ATG incremental requirements

Issue 1-2: Doppler shift assumption:

Agreement:

- For FDD: Set Doppler as 200Hz for UL, 220Hz for DL

- For TDD: Set Doppler as 500Hz

Issue 1-3: TDD pattern

Agreement:

- New TDD pattern together with the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ can be considered as one of possible solution to mitigate the guard period impact for Rel-18 ATG scenario.

- Other solutions not precluded including the UE specific TA reporting as adopted in RRM

- For UE demodulation:

  - New UE demodulation requirement can be specified for the static TDD pattern (if introduced)

    - FFS on exact test case(s) which may be configured with new TDD pattern.

    - FFS on test applicability rules.

  - FFS on the impact of UE demodulation for all candidate options

- For BS demodulation: Further discuss test applicability rules and impact with new TDD pattern.

Issue 1-4: Specification impact

- FFS in next meeting

- UE demodulation

  - PDSCH requirements

Issue 2-1-1: Test scope for PDSCH

- New PDSCH test cases for ATG with FDD slot pattern can be as follows.

| FDD 10 MHz 15kHz SCS | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |
|  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

- New PDSCH test cases with static TDD slot pattern can be as follows (if introduced):

| TDD 40MHz 30kHz SCS | FFS on new TDD pattern: 30D4S6U | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  | 7D1S2U | 2T2R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

- For existing PDSCH test cases:

  - Choose 1 test case per Modulation Order for PDSCH Mapping Type A only, for FDD and TDD, FFS on which test cases to choose.

  - Consider Single Carrier requirements only

Issue 2-1-2: Bandwidth & SCS

Agreement:

- For FDD:10MHz

- For TDD: 40MHz

Issue 2-1-3: Antenna Configuration

Agreement:

- Cover 2T4R and 2T2R.

Issue 2-1-4: MCS&Rank

Agreement:

- Cover 256QAM for ATG demod

- Further discuss the MCS value based on simulation results from companies.

Issue 2-1-5: Applicability rule

- Option 1: New UE feature for ATG scenario and corresponding applicability rule shall be introduced, for instance

| UE capability | Test type |  | Test list |
| --- | --- | --- | --- |
| ATG with 2RX | FR1 FDD | PDSCH | All tests in Clause 5.2.1.1.x |
|  |  | PDCCH | All tests in Clause 5.3.2.1.2 |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.x |
|  |  | PDCCH | All tests in Clause 5.3.2.2.2 |
| ATG with 4RX | FR1 FDD | PDSCH | All tests in Clause 5.2.2.1.x |
|  |  | PDCCH | All tests in Clause 5.3.3.1.2 |
|  | FR1 TDD | PDSCH | All tests in Clause 5.2.1.2.x |
|  |  | PDCCH | All tests in Clause 5.3.3.2.2 |

- Option 2: For the applicability of the ATG UE Demod requirements, follow ongoing discussion in RF on related ATG UE capabilities;

  - PDCCH requirements

Issue 2-2-1: Test scope for PDCCH

Agreement:

- No new test cases for PDCCH under the assumption that new PDSCH test cases will be specified.

  - FFS on the selected legacy test cases for ATG UEs

  - CSI reporting requirements

Issue 2-3-1: Test scope for CSI reporting

Agreement:

- FFS: The feasibility of the CSI reporting given the large propagation delay in the ATG scenario.

- BS demodulation

  - PUSCH requirements

Issue 3-1-1: Test scope for PUSCH

Agreement:

- Reuse the existing TN BS requirement and companies discuss which requirements can be taken as mandatory for ATG BS demodulation. Other features should be optional for the ATG BS and which cases can be tested should be based on BS manufacturer declaration

  - Candidate mandatory requirements

    - Normal PUSCH demodulation

    - UCI multiplexing on PUSCH

- Introduce new dedicated requirement with limited test cases for PUSCH, which only applied for BS declared to support ATG scenario.

Issue 3-1-2: Bandwidth & SCS

Agreement:

- For the test case which reusing existing requirements, reuse all channel bandwidth requirements in TS38.104 8.2.1 and 8.2.3 for ATG PUSCH demodulation.

- For the new test case with new dedicated ATG requirement, use 5MHz for FDD and 10MHz for TDD.

Issue 3-1-3: Antenna Configuration

Agreement:

- For the new test case with new dedicated ATG requirement, 1T2R for 1-C/1-H and 1T2R for 1-O.

Issue 3-1-4: Rank and MCS

Agreement:

- For the test case which reusing existing requirements, cover 16QAM, 64QAM at least

  - FFS for 256QAM supporting pending on UE RF session conclusion on the supporting UL 256QAM Tx

- For the new test case with new dedicated ATG requirement, cover 16QAM and 64QAM at least

  - FFS for 256QAM supporting pending on UE RF session conclusion on the supporting UL 256QAM Tx

Issue 3-1-5: Transform precoding

Agreement:

- Only consider CP-OFDM.

Issue 3-1-6: PUSCH mapping type

Agreement:

- Type A and B for legacy requirements

- Type A for new defined requirements

- Test applicable rules for Type A and Type B should be reused.

Issue 3-1-7: Test metric

- Option 1: Consider 70% and 30% throughput requirements for ATG PUSCH demodulation. (Ericsson, HW)

- Option 2: 70% throughput requirements (CMCC, ZTE, Ericsson (for new incremental requirements), Samsung)

Issue 3-1-8: Other parameters

Agreement:

| parameters |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos 1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

  - PUCCH requirements

Issue 3-2-1: Test scope

Agreement:

- Reuse the existing TN BS requirement and applicable rules for ATG BS

- No need to define incremental requirement.

  - PRACH requirements

Issue 3-3-1: Test scope

Agreement:

- Reuse the existing normal PRACH requirements and same appliable rules for ATG BS

- No need to define incremental requirement

- Simulation assumption

Simulation assumption for PDSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
|  |  |  |  | 30D4S6U S:40G |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Antenna configuration |  |  | 2x2 | 2x2 |
|  |  |  | 2x4 | 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| MCS/Rank |  |  | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 20/24/27 | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 20/24/27 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 for 7DS2U 32 for 30D4S6U |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

Simulation assumption for PUSCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 5 | 10 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
| Channel model |  |  | AWGN+200Hz | AWGN+500Hz |
| MCS/Rank |  |  | Rank 1 16QAM MCS 16 in Table 1 64QAM MCS 20 or 28 in Table 1 256QAM MCS 20 or 22 in Table 2 | Rank 1 16QAM MCS 16 in Table 1 64QAM MCS 20 or 28 in Table 1 256QAM MCS 20 or 22 in Table 2 |
| Antenna configuration |  |  | 1T2R | 1T2R |
| Transform precoding |  |  | CP-OFDM |  |
| HARQ | Maximum number of HARQ transmissions |  | 4 |  |
|  | RV sequence |  | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | pos 1 |  |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port |  | {0} |  |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type |  | A |  |
|  | Start symbol |  | 0 |  |
|  | Allocation length |  | 14 |  |
| Frequency domain resource assignment | RB assignment |  | Full applicable test bandwidth |  |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| Test metric |  |  | 70% of max throughput at target SNR. |  |