3GPP TSG-RAN WG4 Meeting #107												R4-2309815

Incheon, KR, May 22nd – May 26th, 2023

Title: 	WF for Rel-18 TRP/TRS WI

Agenda Item:	8.16.4

Source: 	vivo

Document for:	Approval

Topic #1: Anechoic Chamber (AC) test methodology

- Sub-topic 1-1 NR bandwidth for TRP/TRS OTA testing (GSMA LS)

Issue 1-1-1: How to address GSMA request on NR channel bandwidth for test and requirements

Agreements:

  - FSS whether RAN4 will add an additional set of test parameters for NR bands in the Annex part of TR 38.870 for information.

  - In the reply LS, just mention how to scale the TRS requirements. And asking for more information about the background from GSMA is needed.

  - How to scale the requirements should follow the following rules:

    - Based on RB equation in Table 7.3.2-1b in TS 38.101-1 for REFSENS.

    - RAN4 may also consider other aspects impact the scaling factor, e.g. UE spectrum flatness.

- Sub-topic 1-2 Single-layer UL-MIMO TRP test method

Issue 1-2-1: For non-coherent UE support fullpowerMode1 TPMI index 2

Agreements:

  - FFS, RAN4 targets to finalize the TPMI index selection for non-coherent UE in August RAN4 meeting.

Issue 1-2-2: For fully Coherent UE support multiple TPMI index 2~5

- Proposals

  - Option 1: multi-TPMI based test method

    - Option 1a: measure TRP under each TPMI, and then average TRPs as final performance metric. FFS TPMI index: TPMI 2~5 or 2&3 or 4&5;

        - FFS the naming of the performance metric, e.g. keep current TRPaverage or new term as combined-TRP

    - Option 1b: measure TRP under each TPMI with index 2~5, no further processing. How to define requirement is FFS. FFS TPMI index: TPMI 2~5 or 2&3 or 4&5;

    - Option 1c: measure and record best EIRP at each test point (swept over all applicable TPMIs at each measurement grid), and then integrate all the measured best EIRPs into a TRP-like performance metric. TPMI index 2~5;

        - FFS the naming of the performance metric, e.g. keep current TRPaverage or new term as FR1 averaged spherical coverage;

  - Option 2: single-TPMI based test method

    - Option 2a: measure TRP under TPMI index 2, as the final performance metric;

    - Option 2b: measure TRP under one of TPMI index within 3~5, as the final performance metric;

Agreement:

- RAN4 target to finalize this issue on TPMI index selection for single layer UL-MIMO in August RAN4 meeting

- Encourage companies to bring more analysis on test time and performance impact with above list candidate options.

- Sub-topic 1-3 TxD test method

Issue 1-3-1: TxD test procedure for 2Tx simultaneously (2Tx-based TxD)

Agreement:

- RAN4 target to introduce test methods which can take UE implementation agonistic

  - Encourage companies bring more analysis on the impact with different UE implementation under different test methods.

  - Test time impact also need to be considered

Issue 1-3-2: TxD phase shift issue for 2Tx-based TxD

- Proposals

  - Option 1: A new 2Tx test mode can resolve/stabilize potential 2Tx-based TxD phase variation.

  - Option 2: A new 2Tx test mode can not resolve/stabilize the phase variation, given this could be physical basis of some TxD implementation approaches. Similar to RxD, no test mode is needed currently.

  - Option 3: others

- Agreements:

  - RAN4 to further discussion the above options

Issue 1-3-3: If 2Tx-based TxD power variation is not resolved fundamentally, how to consider this impact

- Proposals

  - Option 1: A MU element and additional TT for 2Tx-based TxD test case.

  - Option 2: No new MU element is needed, similar to RxD test case.

  - Option 3: new test procedure to ensure a more statistical TxD measurement, e.g. define a min. EIRP averaging time.

- Agreements:

  - RAN4 further discuss above options. Other approach is not precluded

Issue 1-3-4: TxD test procedure for 1Tx-based TxD measurement

- Proposals

  - Option 1: RAN4 confirm that 2Tx-based testing for TxD can not be achieved. 1Tx-based measurement with a new 1Tx-based test mode is a must.

  - Option 2: Keep 1Tx-based TxD measurement as low priority.

      - 1Tx test mode (e.g. enable only Tx1, test Tx1 TRP, enable only Tx2, test Tx2 TRP, then combine into one metric) is needed

      - FFS the naming of the performance metric, e.g. keep current TRPaverage or new term as sum-TRP;

Agreements:

  - 2Tx-based testing for TxD UE is achievable. Keep 1Tx-based TxD measurement as low priority.

Topic #2: Reverberation Chamber (RC) test methodology

- Sub-topic 2-1 RC test method

Issue 2-1-2: Coherence bandwidth of RC

Agreements:

  - Based on the measurements, RAN4 confirm that at least 2MHz coherence bandwidth of RC system is sufficient for NR FR1 measurement.

- Sub-topic 2-2 Harmonization and lab alignment activity for RC

Issue 2-2-1: RC harmonization and alignment scope

Agreements:

  - RC harmonization test scope should consider both Rel-17 and Rel-18, i.e. at least browsing mode and talk mode at band n28 and n78.

Issue 2-2-2: Rel-18 RC harmonization test labs

Agreements:

  - Collect and confirm the following volunteered test labs for Rel-18 RC harmonization activity:

| Rel-18 Reverberation Chamber (RC) harmonization activity |  |  |
| --- | --- | --- |
| Volunteer lab | City | Contact |
| vivo | Dongguan, Guangdong | Ruixin. Wang, ruixin.wang@vivo.com |
| SRTC | Beijing | Gong Jian, gongjian1@srtc.org.cn |
| Huawei | Shanghai | Jinxing Li, lijinxing3@huawei.com, Hai Zhou, hai.zhou1@huawei.com |
| EMITE | Fuente Álamo de Murcia, Murcia | Miguel Á. García-Fernández miguel.garcia@emite-ingenieria.es |
| Samsung | Suwon, KR | Bozhi Li, bozhi.li@samsung.com Jeongheum Lee, jh0413.lee @samsung.com |
| OPPO | Dongguan, Guangdong | Qifei Liu, liuqifei@oppo.com |
| CAICT | Beijing | Siting Zhu, zhusiting@caict.ac.cn |
| Note: the volunteered test lab for harmonization activity just needs to provide RC measurement results. |  |  |

Issue 2-2-3: RC harmonization timeline and framework

Agreements:

  - Take the working procedure and timeline in Annex 1 for RC harmonization activity as a baseline. Further update is not precluded.

Topic #3: Testing time reduction solutions

- Sub-topic 3-1 Measurement grid reduction for AC method

Issue 3-1-1: measurement Grid update for TR 38.870

Agreement:

  - The update of measurement grids in TP R4-2308824 to TR 38.870 has been approved.

- Sub-topic 3-2 Test time reduction solutions

Issue 3-2-1: Fast TRS method

Agreement:

  - RAN4 further discuss the RSRPB-based fast TRS test method.

Topic #4: Requirements work

- Sub-topic 4-1 Rel-18 AC lab alignment activity

Issue 4-1-1: Test lab information collection and confirmation

Agreements:

  - Collect and confirm the following volunteered test labs for Rel-18 Anechoic Chamber (AC) lab alignment activity for head and hand scenario (talk mode).

| Rel-18 Anechoic Chamber (AC) lab alignment activity (talk mode) |  |  |
| --- | --- | --- |
| Volunteer lab | City | Contact |
| vivo | Dongguan, Guangdong | Ruixin, Wang, ruixin.wang@vivo.com |
| CAICT | Beijing | Siting Zhu, zhusiting@caict.ac.cn Xuan Yi, yixuan@caict.ac.cn |
| Huawei | Shanghai | Li Jinxing, lijinxing3@huawei.com, Hai Zhou, hai.zhou1@huawei.com |
| Sporton USA | Milpitas, CA | Will Ni (WillNi@sporton-usa.com) Alex Ho (Alexander@sporton.com.tw) |
| OPPO | Dongguan, Guangdong | Qifei Liu, liuqifei@oppo.com |
| SGS Wireless | Shenzhen | Panta Sun (Panta.Sun@sgs.com) Kenny Lai (Kenny.Lai@sgs.com) |
| Element Materials Technology | San Jose, CA | Edward Ortiz edward.ortiz@element.com Soohee Kwon soohee.kwon@element.com |
| SRTC | Beijing | Gong Jian, gongjian1@srtc.org.cn |

Issue 4-1-2: Timeline and workplan for Rel-18 AC lab alignment activity

Agreement:

  - RAN4 should provide a schedule for AC lab alignment, RC lab alignment, RC and AC harmonization and performance test campaigns based on the current progress of Rel-18 TRP TRS WI.

  - After RAN4#107 meeting, WI rapporteur will with work with all the volunteer test labs to figure out a practical timeline to organize the activities, which would be shared in RAN4 reflector.

Issue 4-1-3: How to manage outcome of lab alignment activity

Agreements:

  - Rel-17 and Rel-18 anechoic chamber lab alignment outcome should be recorded in the TR with a dedicated Annex part, for information.

- Sub-topic 4-2 Rel-18 TRP TRS requirement scope

Issue 4-2-1: Band prioritization to gather group efforts

Agreement:

  - The Rel-18 TRP/TRS work item shall aim to complete full coverage of band n1, n28, n41, and n78 requirements, including performance objectives which were not concluded in Rel-17.

  - RAN4 should target to finalize Rel-18 TRP/TRS activities for lab alignment and measurement campaigns to proceed without delay.

- Sub-topic 4-3 Framework for Rel-18 TRP TRS requirement

Issue 4-3-1: Framework for Rel-18 TRP TRS requirement work

Agreements:

- RAN4 further discuss how to disclosed UE information (and thresholds) for Rel-18 TRP TRS requirement work

- Encourage operator test labs to join Rel-18 AC lab alignment activity and performance measurement campaign

- Reference

[1] R4-2309750 Ad-hoc minutes for R18 TRP_TRS WI, vivo, RAN4#107

- Annex 2, Rel-18 RC harmonization activity guideline (baseline for further refinement)

- The purpose of RC harmonization activity is to ensure there is no unexpected lab deviation among different RC systems, and the RC harmonization results will be compared with reference AC results to demonstrate the gap between different methodologies.

- RC Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should have Reverberation chamber(s) ready to support testing based on latest version of 3GPP TR 38.870.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TR 38.870;

- Test cases for RC harmonization Campaign:

  - Test bands: n28, n78; one low and one high band is sufficient

  - Number of test cases per band for each scenario: left and right at low/mid/high channel, total 6 test cases;

  - Use scenario: Both Head and Hand phantom (Talk mode, BHHL and BHHR) and Hand phantom only (Browsing mode, HL and HR);

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 1Tx.

- Harmonization Devices:

  - Reuse the same lab alignment devices (LAD1 and LAD2) from Rel-17 AC lab alignment activity

- Test results submitting:

  - Using the same worksheet template in [TBD] to submit the measurement results

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

- Harmonization criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of harmonization data pool from ≥3 labs submitted before [TBD] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as X*MU (X is TBD) as baseline. MU value is the expanded MU for BHH (to be defined in RAN5).

  - How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LADs delivery scheme

    - LAD delivery scheme will be decided after the confirmation of all the RC volunteer labs.

    - LAD delivery will also consider the parallel Rel-18 AC lab alignment activity to ensure efficiency

  - LAD measurement time in each test lab: finalize LAD measurement within [TBD] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems