3GPP TSG-RAN WG4 Meeting #107	R4-2309824

Incheon, KR May 22nd ‒ May 26th, 2023

Title: 	WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

Agenda Item:	8.8.4

Source: 	Qualcomm

Document for:	Approval

Moderator’s note: For all the open Issues listed in the WF below, other options are not precluded unless explicitly specified.

# Topic #1: General Aspects for FR2 Multi-Rx Demod

Issue 1-1-1: Test method for demodulation requirements for FR2 multi-Rx.

Agreement:

- RAN4 agreed to adopt virtual cable setup to test the FR2 multi-Rx demodulation requirements.

Issue 1-1-2: On new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements.

Agreement:

Consider the baseline correlation model assuming cross-talk between the TRPs and UE Rx antenna as below

R16×16=Eh16×1h16×1H=11+ρ∙diag1  ρ  ρ  1⊗RMIMO

=11+ρ∙RMIMOρRMIMOρRMIMORMIMO,

where h16×1=vec(H4x4) is the re-arranged channel values and

- ⊗ denotes the Kronecker product, diag denotes the diagonal matrix.

- RMIMO is the 4×4 correlation matrix for each link between TRPs to UE panels generated with parameters α (Tx correlation coefficient), β (Rx correlation coefficient), and γ (cross-polarization correlation coefficient) as stated in 38.101-4 B2.3.2

- ρ is the cross-talk power

- Candidate cross-talk power values for initial simulation purpose:

  - -15, -12, -9, -6 dB

  - Other options are not precluded.

- Candidate cross-polarization correlation coefficient values for initial simulation purpose:

  - γ = 0.125

  - Other options are not precluded.

Issue 1-1-3: Receiver assumption.

Agreement:

    - Keep both options (independent and joint processing) open at this point of evaluation.

    - Companies are encouraged to bring results evaluating both independent and joint processing in August meeting.

Issue 1-1-4: Number of receive antennas per Rx chain.

Agreement:

  - 2 Rx per chain

Issue 1-1-5: Duplex mode for demodulation requirements.

Agreement:

  - TDD mode only

# PDSCH Demodulation Requirements

### Sub-topic 2-1: Simulation assumptions for PDSCH demodulation requirements

Issue 2-1-1: Channel model

Agreement

  - At least consider TDLA30-75 for 100 MHz/120 kHz

  - FFS TDLA30-300 for 100 MHz/120 kHz.

Issue 2-1-2: PTRS Port for sDCI schemes

<Way forward >

- Proposals

  - Option 1: One PTRS port per TRP for sDCI schemes

  - Option 2: One PTRS port for sDCI SDM based scheme.

- Recommended WF:

Encourage comments if any

Issue 2-1-3: PTRS Port for mDCI schemes

Agreement:

  - One PTRS port per TRP for mDCI schemes

Issue 2-1-4: Time offset between TRPs

Agreement:

  - Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}.

  - Further discuss which time offset to be used with certain configurations.

Issue 2-1-5: Frequency offset between TRPs

Agreement:

- Assume 600Hz frequency offset between TRPs

Issue 2-1-6: PDSCH rate matching in mTRP transmission

<Way forward >

- Proposals

- Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP.

- Option 2: PT-RS allocation does not overlap with any PDSCH allocation.

- Recommended WF:

Encourage comments if any

Issue 2-1-7: Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous)

<Way forward >

- Proposals

- Option 1: Yes

- Recommended WF:

Encourage comments if any

Issue 2-1-8: Test cases and simulation parameters for sDCI SDM

<Way forward >

- Proposals

  - Option 1: Consider PDSCH test cases and simulation parameter for sDCI based SDM scheme as presented in Tables 1 and 2. Tables can be found in R4-2308945.

- Recommended WF:

Encourage comments if any

Issue 2-1-9: Test cases and simulation parameters for mDCI fully overlapping.

<Way forward >

- Proposals

  - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based SDM scheme as presented in Tables 3 and 4. Tables can be found in R4-2308945.

- Recommended WF:

Encourage comments if any

Issue 2-1-10: Test cases and simulation parameters for mDCI non-overlapping

<Way forward >

- Proposals

  - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based non-overlapping scheme as presented in Tables 5 and 6. Tables can be found in R4-2308945.

- Recommended WF:

Encourage comments if any

# CSI Reporting Requirements

### Sub-topic 3-1: Simulation assumption for PMI reporting requirement

Issue 3-1-1: Number of CSI-RS ports

Agreement:

    - 2-port per TRP.

Issue 3-1-2: Whether to assume LI in report quantity for multi-Rx FR2.

Agreement:

    - Assume no LI in report quantity for multi-Rx FR2

Issue 3-1-3: Simulation parameters for PMI reporting requirement for sDCI SDM scheme

<Way forward >

- Proposals

    - Option 1:

        - Consider the parameters in Table 7 to define PMI reporting requirements for sDCI SDM based scheme. Table 7 can be found in R4-2308946.

        - Consider PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-1 as given by TS 38.101-4 Table A.3.2.2.5-8, with MCS Table 64QAM and MCS index MCS13 (16QAM with CR = 0.48).

- Recommended WF:

Encourage comments if any