3GPP TSG-RAN WG4 Meeting #107												R4-2309863

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			8.34.5

Source:	Moderator (Qualcomm Incorporated)

Title:	WF on mIAB RF requirements and coexistence

Document for:	Approval

# Introduction

This document provides way-forward on mobile IAB (mIAB) based on the outcomes of “Topic summary for [107][314] NR_mobile_IAB_RF [1]

# RF requirements

## IAB-MT RF requirements

Agreement:

- Based on the outcome of the coexistence study:

  - Further study whether existing wide area IAB-MT RF or local area IAB-MT RF requirements can be reused for mobile IAB, or a new set of requirements need to be specified.

  - Limit output power in UL or introduce output power control dynamics following UE.

# Coexistence study

## Coexistence scenarios

Agreement:

- Exclude scenarios 1, 4, 6 and 8 from scenarios given in R4-2305935

## IAB-MT antenna modelling

IAB-MT antenna modelling:

Agreement:

- Consider the 3 panel with 120-degree shift IAB-MT antenna model in coexisting simulation.

IAB-DU antenna modelling:

Agreement:

- Use same assumption for mobile IAB-DU and IAB-MT as starting point for initial co-existence simulations.

- Other options not precluded for mobile IAB-DU antenna modelling and deployment scenarios.

## Additional simulation considerations

Agreement:

- Mobile IAB nodes randomly dropped in terms of location and rotation within each cell.

- RAN4 agree on static drop methodology without any temporal evolution modelling for the coexistence study of mobile IAB.

- Further discuss Whether the O2I car penetration loss in TR 38.901 is applicable for the interference link for mobile IAB DU and further discuss differences to propagation models considered in TR 38.809.

- RAN4 to re-use the assumptions agreed in TR 38.874 for the IAB-node and candidate serving IAB-nodes/donors pathloss model pathloss model either for FR1 and FR2.

- RAN4 to adopt the additional simulation parameters presented in the table below.

| Large-scale channel parameters | FR1: - Macro-to-UE: UMa (TR 38.803) - Macro-to-Micro: UMa (hUE =10m, 4m for mIAB) - Micro-to-Micro: UMi (hUE =10m, 4m for mIAB) - UE-to-UE: UMi (hBS =1.5 m ~ 22.5 m) + penetration loss (TR 38.803) Note: UMi model is not applicable when 2D distance is less than 10m, instead FSPL is applicable FR2: - Micro-to-Micro: UMi-Street canyon (hUE =10m, , 4m for mIAB) The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N =3 independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected. |
| --- | --- |
| mIAB node Tx power | FR1 & FR2: 33dBm as a baseline (based on Rel-16 study) Note: Final value would be derived based on the coexistence study. |
| mIAB node antenna configurations | FR1: (Mg,Ng,M,N,P)=(1,1,8,8,2), dH,dV)=(0.5,0.0.5)λ mIAB node antenna element gain: 5 dBi FR2: 16x8 Antenna Array (Mg,Ng,M,N,P)=(1,1,8,16,2), dH,dV)=(0.5,0.0.5)λ mIAB node antenna element gain: 3 dBi Note 1, 2 |
| UE antenna configuration | See Table A.2.1-4 of TR8.802. UE height follows TR36.873 |
| IAB node antenna height | 4 m |
| IAB node receiver noise figure | 9 dB for FR1 and 10dB for FR2 |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |  |

# References

- R4-2310452, “Topic summary for [107][314] NR_mobile_IAB_RF” Qualcomm Incorporated.