3GPP TSG-RAN WG4 Meeting #107 	R4-2309896

# Incheon, Korea, 22th - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-4: ATP Requirements for FR2-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2023-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add FR2-1 requirements for Application Layer Throughput. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section added under section 7 (7.X) to introduce ATP requirements New section added under A.3.2.2 (A.3.2.2.X) to introduce new channel model as the existing models do not support link adaptation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements defined for Application Layer Througput for FR2-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2307941 |  |  |  |  |  |  |  |  |

## << Start of change 1>>

## 7.X	PDSCH absolute physical layer throughput requirements

### 7.X.1	1Rx requirements

(Void)

### 7.X.2	2Rx requirements

#### 7.X.2.1	FDD

(Void)

#### 7.X.2.2	TDD

##### 7.X.2.2.1	Minimum requirements with Link Adaptation

The purpose of the requirements is to verify the PDSCH absolute physical layer throughput with link adaptation performance for FR2-1 under 2 receive antenna conditions.

For the parameters specified in Table 7.X.2.2-1 and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 7.X.2.2-2.

Table 7.X.2.2-1: Test parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 |
| Subcarrier spacing |  | kHz | 120 |
| Duplex Mode |  |  | TDD |
| Propagation channel |  |  | TDLA30-35 |
| Antenna configuration |  |  | 2x2 |
| Correlation configuration |  |  | ULA Low |
| Beamforming Model |  |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | CSI-RS periodicity and offset | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 1.375 |
| Maximum number of HARQ transmissionNote2 |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Measurement channel |  |  | As specified in Table A.4-1 Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| Note1: The common test parameters are defined in Section 7.2.1 unless otherwise stated in this table. Note2: For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission |  |  |  |

Table 7.X.2.2-2: Minimum requirements

|  | Fraction of maximum throughput (Note 1) | SNR(dB) |
| --- | --- | --- |
| Test 1 | 35% | [16.5] |
| Note 1: Fraction of maximum throughput is defined as the throughput using the TBS corresponding to CQI index 15 with rank 2. |  |  |

## << End of change 1 >>