3GPP TSG-RAN WG4 Meeting #107	R4-2310039

Incheon, Korea, May 22 ‒ May 26, 2023

Title: 	WF on RRM requirements for NR support for dedicated spectrum less than 5MHz for FR1

Agenda Item:	8.15.6

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# Topic #1

## Sub-topic 1-1: Measurements

Issue 1-1: Priority for L3 measurements

Agreement: RAN4 will define both intra-frequency and inter-frequency measurement requirements

Issue 1-2: L3 Measurement requirements:

Agreement: RAN4 will define SSB-based L3 measurement requirements. RAN4 will not define CSI-RS based L3 measurement requirements

Issue 1-3: L1 Measurement requirements:

Agreement: RAN4 will define SSB-based L1 measurement requirements.

Way forward: Define CSI-RS-based measurement requirements

-	Option 1: RAN4 will define CSI-RS-based L1 measurement requirements

-	Option 2: RAN4 will not define CSI-RS-based L1 measurement requirements in this release

Issue 1-4: L1-SINR measurement requirements:

Agreement: RAN4 will not define L1-SINR measurement requirements

## Sub-topic 1-2 Radio Link Monitoring

Issue 1-5: The BW in Hypothetical PDCCH transmission parameters:

Agreement: For 3MHz case, and band n100 the BW is 12PRBs for SSB based RLM. FFS for other bands with 3MHz CBW

Agreement: For 5MHz case the BW is 24PRBs for SSB based RLM

Issue 1-6: PDCCH transmission parameters changes:

Way forward: Reduce aggregation level (CCE) or to increase the number of control OFDM symbols from 2 to 3

-	Option 1: Agree (please include further details)

-	Option 2: Other (please describe detailed proposal)

Issue 1-7: RLM OOS evaluation period:

Way forward: FFS For SSB-based OOS in 3MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB for CBW less then 5MHz

Agreement: For SSB-based OOS in 5MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB for CBW less then 5MHz

Way forward: For CSI-RS based OOS in 3MHz and 5MHz

-	Option 1: Legacy OOS evaluation period

-	Option 2: Extend the CSI-RS based RLM evaluation period TEvaluate_out_CSI-RS for CBW less then 5MHz, where Mout = [30] (=1.5*20) for OOS for CSI-RS within the channel bandwidth below 24RB

-	Option 3: Other

Issue 1-8: PDCCH transmission parameters for out-of-sync evaluation:

Way forward: Agree on PDCCH transmission parameters for out-of-sync evaluation as follows:

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | [2] – Issue 1-6 |
| Aggregation level (CCE) | [8] – Issue 1-6 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | [4]dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | [4]dB |
| Bandwidth (PRBs) | As agreed – Issue 1-5 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

FFS: if same transmission parameters for 3MHz and 5MHz will be in same table or not.

Issue 1-9: RLM IS evaluation period:

Way forward: FFS for SSB-based IS in 3MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_in_SSB for CBW less then 5MHz

Agreement: For SSB-based IS in 5MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_in_SSB for CBW less then 5MHz

Way forward: For CSI-RS based IS in 3MHz and 5MHz

-	Option 1: Legacy IS evaluation period

-	Option 2: Extend the CSI-RS based RLM evaluation period TEvaluate_in_CSI-RS for CBW less then 5MHz, where Mout = [30] (=1.5*20) for OOS for CSI-RS within the channel bandwidth below 24RB

-	Option 3: Other

Issue 1-10: PDCCH transmission parameters for In-sync evaluation:

Way forward: Agree on PDCCH transmission parameters for in-sync evaluation as follows:

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | [2] |
| Aggregation level (CCE) | [4] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | As agreed – Issue 1-5 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

FFS: if same transmission parameters for 3MHz and 5MHz will be in same table or not.

## Sub-topic 1-3 Link Recovery Procedures

During the online session is was agreed that the agreements made for RLM can be applied also for link recovery procedures.

Issue 1-11: The BW in Hypothetical PDCCH transmission parameters:

Agreement: For 3MHz case, and band n100 the BW is 12PRBs for SSB based Link recovery procedure. FFS for other bands with 3MHz CBW

Agreement: For 5MHz case the BW is 24PRBs for SSB based Link Recovery Procedure

Issue 1-12: Beam Failure Detection evaluation period:

Way forward: FFS for SSB-based BFD in 3MHz, RAN4 will keep the existing SSB based BFD evaluation periods TEvaluate_BFD_SSB for CBW less then 5MHz

Agreement: For SSB-based BFD in 5MHz, RAN4 will keep the existing SSB based BFD evaluation periods TEvaluate_BFD_SSB for CBW less then 5MHz

Way forward: For CSI-RS based BFD in 3MHz and 5MHz

-	Option 1: Legacy BFD evaluation period

-	Option 2: Extend the CSI-RS based BFD evaluation period TEvaluate_BFD_CSI-RS for CBW less then 5MHz, where Mout = [30] (=1.5*20) for BFD for CSI-RS within the channel bandwidth below 24RB

-	Option 3: Other

Issue 1-13: PDCCH transmission parameters for beam failure instance:

Way forward: Agree on PDCCH transmission parameters for beam failure instance:

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | [2] – Issue 1-6 |
| Aggregation level (CCE) | [8] – Issue 1-6 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | As agreed – Issue 1-11 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

FFS: if same transmission parameters for 3MHz and 5MHz will be in same table or not.

Issue 1-14: Candidate beam detection evaluation period:

Way forward: FFS for SSB-based CBD in 3MHz, RAN4 will apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less then 5MHz.

Agreement: For SSB-based CBD in 5MHz, RAN4 will apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less then 5MHz.

Way forward: For CSI-RS based CBD in 3MHz and 5MHz

-	Option 1: Legacy CBD evaluation period

-	Option 2: Extend the CSI-RS based CBD evaluation period TEvaluate_CBD_CSI-RS for CBW less then 5MHz, where MBFD = [5] (= 1.5*3) for CSI-RS within the channel bandwidth below 24RB

-	Option 3: Other

## Sub-topic 1-4 CGI Reading

Issue 1-15: CGI Reading

Agreement:

- RAN4 will not define new CGI reading requirements

- RAN4 agreed that the legacy CGI reading requirements do not apply

## Sub-topic 1-5 MIB/PBCH Reading

Issue 1-16: Soft Combining

Way forward: Soft Combining

-	Companies are encouraged to collect simulation results with and without soft combining to assess the performance impacts and make the decision then.

## Sub-topic 1-6 BW for PBCH in HO command

Issue 1-17: BW for PBCH (e.g., 12 PRBs) of target cell shall be provided to UE in HO command

Way forward: BW for PBCH (e.g., 12 PRBs) of target cell shall be provided to UE in HO command

-	Option 1: BW for PBCH (e.g., 12 PRBs) of target cell shall be provided to UE in HO command

-	Option 2: BW for PBCH (e.g., 12 PRBs) of target cell need not be provided to UE in HO command

-	Option 3: FFS

## Sub-topic 1-7 Side Conditions

Issue 1-18: Use side condition Es/Iot≥-4 dB for NR target cell detection

Way forward: Use side condition Es/Iot≥-4 dB for NR target cell detection

-	Option 1: Use side condition Es/Iot≥-4 dB for target NR cell detection for RRC connection re-establishment and RRC connection release with re-direction

-	Option 2: Other

# Topic #2 CR Split

| Section # | Section name | Company |
| --- | --- | --- |
| 6 | RRC_CONNECTED state mobility | Ericsson |
| 8.1 | Radio Link Monitoring | MediaTek |
| 8.5 | Link Recovery Procedures | Huawei |
| 9.2 | NR intra-frequency measurements | Nokia |
| 9.3 | NR inter-frequency measurements |  |
| 9.5 | L1-RSRP measurements for Reporting |  |