3GPP TSG-RAN WG4 Meeting #107	R4-2310042

Incheon, KR, May 22 – May 26, 2023

Title: 	WF on FR2 HST RRM requirements (part 2)

Agenda Item:	8.13.6

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# Topic#1: UL TX timing adjustment

## Sub-topic#1-1: Impacts of MAC-CE based cross-RRH network signalling assistance

Issue 1-1-1: Impact on One-shot large timing adjustment requirements

Agreement

- For Rel-18 TCI state switching with MAC-CE based cross-RRH network signaling assistance:

  - For indicated cross-RRH TCI state switching,

    - the existing R17 UL timing adjustment requirements in clause 7.1.2.3 apply to the first UL transmission after TCI state switch

      - FFS, whether the condition that UE measurement on DL timing difference is larger than certain threshold (as specified in Rel-17) shall not be included as the applicable condition.

  - If it is indicated that TCI state switch is not across non-collocated RRHs,

    - Gradual timing adjustment requirements in Clause 7.1.2.1 apply to the first UL transmission after TCI state switch

Issue 1-1-2: Conditions to apply one-shot large timing adjustment

Agreement:

- The condition for UE to apply one-shot large uplink timing adjustment at TCI state switch:

  - UE capability to support the cross-RRH TCI state switching with MAC-CE based network signaling assistance

  - NW flag signaling highSpeedMeasFlagFR2-r17 to indicate UE be in the HST scenario

  - Reuse Rel-17 large one step UL timing adjustment signalling (highSpeedLargeOneStepUL-TimingFR2-r17 NW signalling and [largeOneStepUL-timingFR2-r17] UE capability)

Issue 1-1-4: HST FR2 TCI state switch delay

Agreement:

- When it is indicated that TCI state switch is across non-collocated UE shall be able to receive PDCCH with the target TCI state after slot n+ THARQ + 3Nslotsubframe,µ + TOk*(Tfirst-SSB + TSSB-proc + Trs + Trs-proc) / NR slot length (i.e., Rel-17 requirement)

- FFS, how to define requirement when it is indicated that TCI state switch is not cross-RRH:

  - Option 1: slot n+ THARQ + 3Nslotsubframe,µ + TOk*(Tfirst-SSB + TSSB-proc) / NR slot length when it is indicated that TCI state switch is not across non-collocated RRHs,

  - Option2: Keep Rel-17 requirement without changes

Issue 1-1-5: Enhanced TCI States Activation/Deactivation

Way forward

Open issue needs further discussion to provide feedback to RAN2:

- FFS, whether enhanced TCI Activation/Deactivation is considered in Rel-18 HST FR2 (indicating two TCI states for simultaneous multi-panel reception)

  - Option 1: Only consider Rel-15 TCI state switch

  - Option2: Also consider indication of two TCI states in addition to Option 1

- FFS, whether each TCI state ID shall be associated with one 1-bit indication.

Issue 1-1-6: Triggering RACH-based procedure

Agreement:

- Do not take new signalling into account for triggering RACH procedure

Issue 1-1-7: UE MAC timeAlignmentTimer behaviour

Way forward:

Open issue needs further discussion

- FFS, timeAlignmentTimer behaviour when one-shot large UL timing adjustment is not in use (RACH-based timing adjustment):

  - Option 1: In HST FR2 scenarios, UE MAC timeAlignmentTimer should be stopped or suspended after indicated inter-RRH TCI state switch

  - Option 2: No need to introduce new timeAlignmentTimer related enhancements

## Sub-topic #1-2: General

Issue 1-2-1: A need for timing adjustment at UL spatial relation switch

Way forward:

Open issue needs further discussion:

- Option 1: Apply existing one-shot larger UL timing adjustment mechanism (Clause 7.1.2.3) at UL spatial relation switch

- Option 2: UL spatial relation switch shall always be executed strictly when the corresponding DL TCI state switches

- Option 3: The existing gradual timing adjustment requirements can be applied, and there is no need to define additional UL transmit timing adjustment

Issue 1-2-2: Applicability of gradual timing adjustment in between one-shot large timing adjustments

Way forward:

Open issue needs further discussion:

- Option 1: UE to report the value of one-shot large UL timing adjustment back to the network.

- Option 2: Follow the current UE autonomous timing adjustment procedure and requirements. Discussion not in Rel-18 scope.

- Option 3: Describe UE behaviour after one shot UL timing adjustment in the TR.

# Topic #2: Tunnel deployment

Issue 2-1: Solution to the mobility issue in the tunnel

Agreement:

- Consider bi-directional deployment with simultaneous multi-panel reception in the tunnel scenario to alleviate the mobility issues.

Issue 2-2: Use of DRX in the tunnel deployment

Way forward:

- FFS, whether to restrict the use of DRX in tunnel deployment.

Issue 2-3: Indication of tunnel deployment

Way forward:

- FFS, whether indication of tunnel deployment is necessary.

# Topic #3: CR Split

The companies can use the table below to indicate their interest in providing CRs to the corresponding section, if changes found to be needed:

| Clause, TS 38.133 | Company |
| --- | --- |
| 3.6 Applicability of requirements in this specification version |  |
| 6 RRC_CONNECTED state mobility | No impact is expected from UL timing enhancements or Tunnel deployment |
| 7 Timing | Huawei |
| 8 Signalling characteristics | [Nokia] |
| 9 Measurement Procedure | No impact is expected from UL timing enhancements or Tunnel deployment |