3GPP TSG-RAN WG4 Meeting # 107 	    R4-2310075

Incheon, South Korea, 22nd May – 26th May 2023

Agenda Item:	8.23.3.4

Source: 	Ericsson

Title:	Updated simulation assumptions for 1Rx RedCap positioning

Document for:	Approval

# Introduction

RAN4#107 agreed to update the simulation assumptions for 1Rx RedCap UE captured in R4-2306350 for further evaluation of positioning measurement accuracy. In this contribution updated link level simulation assumption for PRS based RSTD, PRS-RSRP, UE Rx-Tx, and PRS-RSRPP measurement for 1Rx UE is captured. Companies are encouraged to provide simulation results based on the agreed performance metric in section 3.

- Simulation assumptions

Table 1: General parameters.

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |
| Duplex modes | FDD and TDD |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |
| Cyclic prefix | Normal |
| DRX | OFF |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz, 60 kHz - FDD, TDD |
| Propagation conditions [TS 38.101-4] | For RSTD, PRS-RSRP and UE Rx-Tx: AWGN, TDL-A (30 ns delay spread, 5Hz), |
|  | For PRS-RSRPP: Two-tap channel defined in 38.101-4 Annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | AWGN (reuse Rel. 17 side conditions): - RSTD: (-3, -6, -6), for Nsample = 1 - RSTD: (-6, -13, -13), for Nsample = 4 - UE Rx-Tx/PRS-RSRP/PRS-RSRPP: (0, -6, -6), for Nsample = 1 - UE Rx-Tx/PRS-RSRP/PRS-RSRPP: (-3, -13, -13), for Nsample = 4 Fading channel (side conditions for Nsample = 4) For Rx-Tx/RSRP/PRS-RSRPP measurements: (-3, -6)dB (-3, -10)dB For RSTD measurement: (-6,-10,-10)dB (-6, -6, -6)dB Fading channel (side conditions for Nsample = 1 is TBD) |
| Number of UE receive antennas | 1 rx |
| UE measurement bandwidth | Full carrier bandwidth |
| Number of transmit PRS antennas | 1 |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |
| PRS muting | No muting |
| Power boosting | No power boosting |
| ExpectedRSTD-UncertaintyNOTE 2 | 5 us (15 kHz), 2.5 us (30 kHz) |
| NOTE 1: nr-DL-PRS-ReferenceInfo is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 3: Companies are encouraged to check suitability of the parameters and the values. |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  | 60 | 24 | 4 | 64 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1, 4 |  |  |  |
| PRS periodicity | 40ms |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |
| NOTE 1: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. NOTE 2: Companies are encouraged to check suitability of the parameters and the values. NOTE 3: For number of samples equal to 1, only configurations with RB num ≥ 48 are simulated. |  |  |  |  |

- Performance metric

At least the following performance characteristics are to be provided for PRS RSTD:

- RSTD error CDFs for the 2 neighbour cells

- 5%-ile and 95%-ile of the RSTD errors

At least the following performance characteristics are to be provided for PRS RSRP:

- RSRP error CDFs for 3 cells

- 5%-ile and 95%-ile of the RSRP errors for each cell

At least the following performance characteristics are to be provided for PRS-RSRPP:

- PRS-RSRPP error CDFs for 3 cells

- 5%-ile and 95%-ile of the PRS-RSRPP errors for each cell

At least the following performance characteristics are to be provided for UE Rx-Tx:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge).

- PRS-RSRP error = estimated PRS-RSRP – ideal PRS-RSRP (based on perfect channel knowledge).

- PRS-RSRPP error = estimated PRS-RSRPP – ideal PRS-RSRPP (as defined in 10.1.38.2.1 of TS 38.133).

- UE Rx-Tx error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

- References

- R4-2009127, “Link-level simulation assumptions for UE Rx-Tx time difference measurements”, Qualcomm.

- R4-2211061, “Simulation assumption for PRS-RSRPP”, Huawei.

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”, Huawei.

- R4-2303318, “Simulation assumptions for 1Rx RedCap UE PRS measurements”, Ericsson.

- R4-2306350, “Updated simulation assumptions for 1Rx RedCap UE PRS  measurement without FH”, Ericsson.