3GPP TSG-RAN WG4 Meeting # 107												R4-2310088

Incheon, Korea, May 22-26, 2023

Agenda item:			8.23.4

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for NR Positioning RRM requirements

Document for:	Information

# Introduction

The document contains discussion related to the positioning core requirements across the following 3 threads:

- Topic # 1: [107][219] NR_pos_enh2_part1

- Topic # 2: [107][220] NR_pos_enh2_part2

- Topic # 3: [107][221] NR_pos_enh2_part3

# Topic #1: [107][219] NR_pos_enh2_part1

## RedCap Positioning (Agenda 8.23.3.4)

### Sub-topic 2-1: General aspects/scenarios for RedCap positioning

#### Issue 2-1-1: Applicability of RRC_IDLE state for Redcap positioning without FH:

- Proposals

  - Option 1: CATT

    - The requirements for redcap UE positioning in RRC_IDLE can be discussed in LPHAP part.

  - Option 2: OPPO, QC

    - For RedCap UE without FH, define RRM requirements in RRC idle state.

- Recommended WF

  - Discuss the options.

Agreements:

- The requirements for redcap UE positioning without FH in RRC_IDLE are discussed in LPHAP part

#### Issue 2-1-2: Applicable RRC states for Redcap positioning with FH:

- Proposals

  - Option 1: CATT, Nokia

    - Define requirements for RedCap UE positioning with frequency hopping in RRC_CONNECTED and RRC_INACTIVE.

    - Option 1A: CATT

      - The requirements for redcap UE positioning in RRC_IDLE can be discussed in LPHAP part.

  - Option 2: OPPO

    - For RedCap UE with FH, prioritize RRM requirements in RRC connected state.

  - Option 3: HW

    - For RedCap UE positioning with FH, RAN4 to decide whether to define requirements for INACTIVE/IDLE after requirements for CONNECTED are stable.

  - Option 4: QC

    - FFS define requirements for RedCap positioning with frequency hopping in RRC_IDLE state

- Recommended WF

  - Discuss the options.

Agreements:

None. No discussion.

#### Issue 2-1-4: Relation with Rel-16/Rel-17 positioning for PRS measurements with FH

- Proposals

  - Option 1: QC

    - PRS requirements for both 1Rx and 2Rx RedCap UE with FH will be defined for all the Rel-16 positioning features. FFS which Rel-17 positioning features will be covered by the requirements with FH.

  - Option 2: OPPO

    - MG-less based measurement:

      - For RedCap UE with FH, whether to support MG-less based measurement is up to RAN1

    - For RedCap UE without FH and with FH, support reduced samples.

    - For lower Rx beam sweeping factor in FR2:

      - For RedCap UE with FH, not support lower Rx beam sweeping factor.

    - For RedCap UE without FH and with FH, support per-FR gap and MAC-CE based Pos gap.

- Recommended WF

  - Discuss the options.

Agreements:

None. No discussion.

#### Issue 2-1-5: TEG framework for RedCap positioning

  - Option 1: OPPO

  - For RedCap UE without FH and with FH, not support TEG feature.

- Option 2: E///

  - Rel. 17 TEG framework is supported for RedCap positioning.

- Recommended WF

  - Discuss the options.

Agreements:

Already agreed that all Rel-16/Rel-17 features apply for redcap UE without FH

### Sub-topic 2-2: PRS measurements for RedCap without FH

#### Issue 2-2-2: PRS measurement requirements in RRC inactive state without FH

- Proposals

  - Option 1: E///, CMCC

    - For positioning measurement core requirement for RedCap UEs without FH in RRC_INACTIVE state, Kcarrier_PRS in Rel. 17 RRC_INACTIVE core requirement is updated to Kcarrier_PRS_RedCap. Kcarrier_PRS_RedCap = 1 if RedCap UE is able to perform RRM and PRS measurements in parallel to each other, otherwise Kcarrier_PRS_RedCap is defined as:

      - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4.

      - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

- Recommended WF

  - Check if Option 1 is agreeable?

Agreements:

- For positioning measurement core requirement for RedCap UEs without FH in RRC_INACTIVE state, Kcarrier_PRS in Rel. 17 RRC_INACTIVE core requirement is updated to Kcarrier_PRS_RedCap. Kcarrier_PRS_RedCap = 1 if RedCap UE is able to perform RRM and PRS measurements in parallel to each other, otherwise Kcarrier_PRS_RedCap is defined as:

  - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4.

  - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

#### Issue 2-2-3: PRS measurement requirements in RRC connected state without FH

- Proposals

  - Option 1: E///

  - For positioning measurement core requirement for RedCap UEs without FH in RRC_CONNECTED state, CSSFPRS,i in Rel. 17 RRC_CONNECTED core requirement is updated to CSSFPRS,RedCap,i. The value of CSSFPRS,RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2.

- Recommended WF

  - Discuss the option.

Agreements:

- For positioning measurement core requirement for RedCap UEs without FH in RRC_CONNECTED state, CSSFPRS,i in Rel. 17 RRC_CONNECTED core requirement is updated to CSSFPRS,RedCap,i. The value of CSSFPRS,RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2.

- Clause 9.1A.5.2 needs to be updated to include the impact of the positioning measurements. Details are FFS.

### Sub-topic 2-3: PRS measurements for RedCap with FH

#### Issue 2-3-1: When to start PRS measurements for RedCap with FH?

- Proposals

  - Option 1: HW, E///, QC, Nokia

    - RAN4 to wait for more progress in RAN1 before starting to work on detailed PRS measurement period requirements with frequency hopping for RedCap UEs.

- Recommended WF

  - Discuss the option.

Agreements:

None. No discussion.

#### Issue 2-3-2: Scenarios for PRS measurements for RedCap with FH

- Proposals

  - Option 1: CATT

    - Define measurement requirements for the following cases separately:

      - Case 1: A single measurement is reported based on receiving multiple hops of the DL PRS or UL SRS for positioning

      - Case 2: One [or more] measurements are reported where each measurement is associated with one received hop

- Recommended WF

  - Discuss the option.

Agreements:

None. No discussion.

#### Issue 2-3-3: Relation between FH and measurement gaps for PRS measurements for RedCap with FH

- Proposals

  - Option 1: HW

    - RAN4 to define requirements for PRS measurement with FH for the case where UE hops between different repetitions of same resource within a single MG occasion.

  - Option 2: QC

    - For defining DL PRS measurement requirements with FH within measurement gaps, prioritize the case where the multiple hops are all received within a single gap instance.

- Recommended WF

  - Discuss the options.

Agreements:

None. No discussion.

#### Issue 2-3-4: Effective BW in gap for PRS measurements for RedCap with FH

- Proposals

  - Option 1: HW

    - RAN4 to discuss how to determine the effective BW with FH in a single MG occasion and the applicable accuracy requirements based on following parameters

      - Number of repetitions, which depends on PRS resource configuration

      - Switching time, which depends on UE capability

      - Number of overlapping RBs between hops, which is FFS

- Recommended WF

  - Discuss the option.

Agreements:

None. No discussion.

#### Issue 2-3-5: PRS measurement period requirements for RedCap with FH

- Proposals

  - Option 1: CATT

    - The existing Rel-17 measurement period and accuracy requirements can be used as baseline for each hop in case 2.

    - The measurement period requirements for case 1 can take Rel-17 requirements as baseline with the update on the definitions of Tavailable and Lavailable.

- Recommended WF

  - Discuss the option.

Agreements:

None. No discussion.

## PRS/SRS bandwidth aggregation (Agenda 8.23.3.5)

### Sub-topic 3-1: General aspects/scenarios for PRS/SRS BW aggregation

#### Issue 3-1-1: Applicable RRC states

- Proposals

  - Option 1: E///

    - RAN4 to not define requirements for PRS bandwidth aggregation outside of the MG in RRC_CONNECTED state.

- Recommended WF

  - Discuss the option.

Agreements:

- RAN4 to not define requirements for PRS bandwidth aggregation outside of the MG in RRC_CONNECTED state.

#### Issue 3-1-2: Applicable bandwidth

- Proposals

  - Option 1: Nokia

    - RAN4 to wait on further RAN1 agreements related to maximum total aggregated bandwidth for PRS/SRS and on equal/unequal bandwidths of aggregated PFLs.

- Recommended WF

  - Discuss the option.

Agreements:

- RAN4 to wait on further RAN1 agreements related to maximum total aggregated bandwidth for PRS/SRS and on equal/unequal bandwidths of aggregated PFLs.

### Sub-topic 3-2: PRS measurement requirements for PRS/SRS bandwidth aggregation

#### Issue 3-2-1: Conditions for PRS/SRS bandwidth aggregation requirements:

- Proposals

  - Option 1: ZTE

    - RAN4 shall study the core requirements based on the current RAN1 progress as below:

      - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna)

      - The same number of symbols, symbol location within one slot, repetition factor,

      - The same numerology, i.e. the same CP and SCS

      - The same or different bandwidths

      - The same comb size

      - The same power per subcarrier

      - Aggregated PFLs are configured on the same aligned numerology grid

      - Phase continuity between aggregated PFLs

  - Option 2: HW

    - Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL.

    - Requirements for PRS CA are applicable to PRS resource sets or PRS resources across PFLs that are linked. Other conditions can be discussed based on RAN1 progress.

- Recommended WF

  - Discuss the options.

Agreements:

None. No discussion.

#### Issue 3-2-2: Impact of number of PFLs on PRS/SRS bandwidth aggregation requirements:

- Proposals

  - Option 1: LG

  - RAN4 to consider the PFL group concept which means aggregated PFLs. For example, in the RSTD case

TRSTD,Total=j=1GPRSTD,j+G-1×maxPeffect, j

where,

j: Index of PFL group

G: Total number of PFL groups

Peffect,j: Periodicity of the PRS RSTD measurement in PFL group j

PRSTD,j: Measurement period for PRS RSTD measurement in PFL group j

  - Option 2: ZTE

    - The PRS periodicities from different PFLs are the same and it is not the strong influence factor when defining the measurement period.

    - The referent PFL(r) can be defined, the measurement period requirement of a referent PFL(r) can be used as the reference of other PFLs’ measurement period requirement in the same group.

- Recommended WF

  - Discuss the options.

Agreements:

None. No discussion.

#### Issue 3-2-3: PRS measurement period for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: ZTE, HW, OPPO, E///

    - Existing measurement period requirements can be used as baseline for defining PRS CA requirements, and aggregated PFLs are considered as one PFL.

    - Option 1A: E///

      - Rel. 17 gap-based measurement period requirement for RRC_CONNECTED mode is used as a baseline for defining measurement period requirement for PRS bandwidth aggregation within MG in RRC_CONNECTED mode.

      - Rel. 17 measurement period requirement for RRC_INACTIVE mode is used as a baseline for defining measurement period requirement for PRS bandwidth aggregation in RRC_INACTIVE mode.

  - Option 2: QC

    - To define the PRS measurement period requirement for RSTD and UE Rx-Tx when the LMF requests the UE to aggregate PRS resources across groups of PFLs (super-PFLs), as a baseline, the measurement period can be defined as a summation of two terms:

      - The first term equals the Rel-17 the measurement period requirement, with the modification that only PRS resources that are not aggregated, according to the LMF request and applicability conditions, are counted in L_(available_PRS,i) for each PFL.

      - The second term is the summation of the measurement delays for the super-PFLs. As a baseline, only PRS resources to be aggregated according to the LMF request and applicability conditions, are counted in  L_(available_PRS,i) for each super-PFL.

      - FFS whether any additional modifications to the first term are needed due to new capabilities defined by RAN1 for PRS BW aggregation.

      - FFS the exact formula for the delay for each super-PFL based on RAN1 agreements on UE capabilities, etc.

      - FFS how to handle the scenario where the PRS RSTD reference is not aggregated

      - FFS any changes to the measurement requirement for PRS-RSRP and/or PRS-RSRPP when reported as supporting measurements for DL-TDOA and multi-RTT

    - Wait for further progress in RAN1 to finalize the definition of the measurement period requirement.

  - Option 3: Nokia

    - RAN4 to focus on MG assisted PRS measurements and wait on RAN1 agreements related to the support of PPW for gapless measurements in RRC_CONNECTED state.

    - RAN4 to define measurement reporting requirements including measurement delay with measurement gaps for RRC_CONNECTED and for RRC_INACTIVE states.

- Recommended WF

  - Discuss the options.

Agreements:

None. No discussion.

#### Issue 3-2-4: Impact of SRS CA for communication on PRS/SRS bandwidth aggregation

- In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

  - Any impact of the CA for communication on the SRS aggregation (UL part) is up to RAN1 discussion.

- Proposals

  - Option 1: HW

    - RAN4 to discuss possible impacts of SRS CA based on RAN1 progress.

  - Option 2: Nokia

    - RAN4 to base RRM requirements for RRC_INACTIVE on the scenario that aggregated carriers with SRS for positioning are located outside the initial UL BWP.

- Recommended WF

  - Discuss the options.

Agreements:

- RAN4 to discuss possible impacts of SRS CA based on RAN1 progress.

#### Issue 3-2-5: Guard period between data and positioning period in UL

- Proposals

  - Option 1: QC

    - RAN4 to discuss requirements for a switching/guard period between UL transmissions within active BWP and transmissions outside carriers/BWP in support of positioning BW aggregation.

  - Option 2: LG

    - RAN4 to wait for more RAN1 progress to discuss the guard period.

- Recommended WF

  - Discuss the options.

Agreements:

- RAN4 to wait for more RAN1 progress to discuss the guard period.

#### Issue 3-2-6: TEG for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: OPPO

    - Consider TEG to defined measurement requirements for aggregated PFLs.

- Recommended WF

  - Discuss the option.

Agreements:

None. No discussion.

#### Issue 3-2-7: Impact of PRS resource dropping on PRS/SRS bandwidth aggregation requirement

- Proposals

  - Option 1: HW

    - RAN4 to discuss the impact of PRS resource dropping in one or more PFLs on the requirements based on further RAN1 agreements.

  - Option 2: Nokia

    - RAN4 to investigate the case of PRS collision with SSB or other DL channels for one aggregated PFL and wait for further RAN1 agreements related to supported total aggregated BW and equal/unequal bandwidths of aggregated PFLs.

- Recommended WF

  - Discuss the options.

Agreements:

- RAN4 to discuss the impact of PRS resource dropping in one or more PFLs on the requirements based on further RAN1 agreements.

### Sub-topic 3-3: PRS measurement report mapping for PRS/SRS bandwidth aggregation

#### Issue 3-3-1: Whether report mappings with PRS/SRS bandwidth aggregation need to be updated?

- Proposals

  - Option 1: LG

    - Existing report mapping can be reused for RSTD and UE Rx-Tx. If necessary RAN4 can make a decision after performance evaluation.

  - Option 2: HW

    - RAN4 to update the report mapping for RSTD and UE Rx-Tx

      - FR1: support range of reporting granularity from 32Tc to [1]Tc

      - FR2: support range of reporting granularity from 32Tc to [0.25]Tc

      - Define new mapping table for reporting granularity of 0.5Tc and 0.25Tc

  - Option 3: E///

    - Update report mapping for RSTD and UE Rx-Tx with PRS/SRS bandwidth aggregation based on maximum supported aggregated SRS/PRS bandwidth and performance evaluation.

- Recommended WF

  - Discuss the options.

Agreements:

- For FR1 the additional reporting granularity values are 0.5 Tc, 1 Tc and 2 Tc.

- For FR2 the additional reporting granularity values are 0.25 Tc and 0.5 Tc.

- The above reporting granularity values apply to both UE and gNB positioning measurements.

- Send LS to RAN2 and RAN3 (and CC to RAN1) to define signaling for UE and gNB positioning measurement reporting respectively.

#### Issue 3-3-2: RSTD and UE Rx-Tx time difference measurement report

- Proposals

  - Option 1: Nokia

- RAN4 to specify RSTD and UE Rx-Tx time difference measurement reports to be common for all CCs/PFLs.

- RAN4 to investigate whether to specify PRS-RSRP and PRS-RSRPP measurement reports to be either common for all CCs/PFLs or individual for each CC/PFL as configured by LMF.

- Recommended WF

  - Discuss the option.

Agreements:

- RAN4 will study the impact of PRS-RSRP and PRS-RSRPP measurement on the reporting requirements based on RAN1 and RAN2 agreements/progress.

### Sub-topic 3-4: Accuracy requirement for PRS/SRS bandwidth aggregation

#### Issue 3-4-1: Number of carriers/PFLs for accuracy requirement

- Proposals

  - Option 1: Nokia

- RAN4 to define measurement accuracy requirements for PRS/SRS aggregation for two and three contiguous carriers/PFLs, respectively.

- Recommended WF

  - Based on agreement at RAN4#106bis-e (R4-2306349), it is recommended to postpone this issue until the performance part of the WI starts.

Agreements:

None. No discussion.

# Topic #2: [107][220] NR_pos_enh2_part2

## Sidelink Positioning (Agenda 8.23.3.2)

### Sub-topic 1-1 Measurement period requirements

#### Issue 1-1-1: Measurement types to be defined for SL positioning measurement requirements:

- Proposals:

  - Option 1: (CATT, CMCC, vivo, Huawei, Nokia)

    - Besides agreed RAN4 SL-based RSTD, Rx-Tx time difference, RSRP, RSRPP measurements, RAN4 also to define the SL based measurement period requirements for SL-PRS based RTOA and AOA/ZOA.

  - Option 2: (OPPO)

    - Define the measurement period requirements for SL-PRS RTOA

    - Not necessary to define delay requirements for AoA/ZoA if no performance requirement defined.

- Recommended WF

Agreements:

- RAN4 to start with defining requirements for SL RSTD.

#### Issue 1-1-2: The principle for defining the measurement period requirements:

- Proposals:

  - Proposal 1: (CATT)

    - The following factors should be considered for SL-PRS based measurement requirements:

      - Available SL-PRS periodicity considering muting

      - Available time duration of SL-PRS resource

      - UE processing capability

      - The number of carriers

      - The number of samples

      - The number of beams

      - Scaling factor due to TEG

      - The definition of measurement window

      - The measurement duration of the last sample

  - Proposal 2: (vivo, Ericsson, Huawei, Qualcomm)

    - RAN4 could follow the principal that try to define a common measurement period requirement for different measurement types. If technical issue exists, future adaptions will be discussed case by case.

    - FFS which measurement can be used as baseline (SL PRS-based Rx-Tx or SL PRS based RSRP)

  - Proposal 3: (Ericsson)

    - Measurement period may be different for different SL PRS resource pool types (dedicated or shared).

    - SL measurement period requirement is defined as a function of the number of samples.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 1-1-4: Impact of SL-DRX

- Proposals:

  - Option 1: (CATT, vivo, OPPO, Huawei, Qualcomm)

    - RAN4 to prioritize non-DRX case for the measurement period for SL PRS-based measurements.

  - Option 2: (Ericsson)

    - SL PRS-based measurement period depends on SL DRX cycle length.

  - Option 3: (Nokia)

    - RAN4 to study the requirement for both SL-DRX and non-SL-DRX case.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 1-1-5: Impact of other channels/signals

- Proposals:

  - Option 1: (OPPO)

    - Not consider sharing mechanism between SL-PRS and S-SSB measurement.

  - Option 2: (Huawei)

    - RAN4 to discuss possible sharing between SL PRS measurement and other SL measurements based on RAN1 agreements.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 1-1-6: Impact of network coverage change

- Proposals:

  - Option 1: (CATT)

    - The measurement period requirements still apply when there is change of network coverage scenario.

  - Option 2: (Huawei)

    - RAN4 to discuss the impact of transition (coverage status change) after requirements for non-transition case are stable.

  - Option 3 (Ericsson):

    - RAN4 to discuss the impact of a change in coverage status (e.g., going between in-coverage, out-of-coverage, partial coverage, or changing the SL coverage range) on SL positioning measurements, including measurement performance, measurement procedure, UE behaviour, etc.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 1-1-7: Impact of Uu link connection

- Proposals:

  - Option 1: (Ericsson)

    - RAN4 to discuss the impact of Uu link distortion on SL positioning measurements, e.g., due to handover, RLF or RR reestablishment in the Uu connection to gNB.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 1-1-8: initiation/cease of SL PRS tx

- Proposals:

  - Option 1 (Ericsson, Huawei):

    - RAN4 to discuss requirements for initiation/cease of SL transmissions for positioning.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

### Sub-topic 1-2 Timing related requirements

#### Issue 1-2-1: Timing error limit requirements of SL UE for positioning (both anchor UE and target UE)

- Proposals:

  - Option 1: (CATT, vivo, Huawei, Qualcomm)

    - Reuse the existing Te requirements for SL positioning in Rel-18 and also no new transmit timing adjustment requirements for SL positioning are needed

  - Option 2: (LGE)

    - More stringent timing error limit requirement is needed for SL positioning.

  - Option 3: (OPPO)

    - Reuse the existing Te requirement for SL-PRS RSRP, RSRPP, AoA/ZoA and RTOA measurement.

    - A more stringent Te requirement can be considered for Tx anchor UEs for RSTD measurement.

    - FFS whether to reuse the exiting Te requirement for Rx-Tx time difference measurement.

- Recommended WF

Agreements:

- Discuss the SL positioning requirements based on existing sidelink Te requirements.

- If any technical issues identified, the agreements can be revisited.

#### Issue 1-2-2: Impact due to the Tx/Rx timing difference of multiple anchor Ues

- Proposals:

  - Option 1: (LGE)

    - Consider all possible synchronization reference sources such as GNSS, gNB and SyncRef UE to overcome the distance difference between synchronization source and anchor UEs.

  - Option 2: (Huawei)

    - Simultaneous Tx/Rx capability is up to RAN1 discussion.

  - Option 3: (Nokia)

    - The timing compensation due to different propagation delay needs to be agreed at RAN1 discussion.

    - RAN4 needs to investigate the transmit time accuracy from different anchor UEs.

- Recommended WF

Agreements:

- SL timing related positioning measurement requirements are defined under the RAN4 assumption that there is no timing misalignment among multiple anchor UEs due to different synchronization reference sources.

#### Issue 1-2-3: The scenarios on synchronization reference resource change

- Proposals:

  - Option 1: (Ericsson)

    - RAN4 to discuss the following scenarios with the synchronization reference source change:

      - Scenario A: Impact on an SL measurement due to the synch change in the same SL UE performing the measurement:

        - Receiving UE for unidirectional measurements

        - Receiving and transmitting UE for bidirectional measurements

      - Scenario B: Impact on an SL measurement due to the synch change in another SL UE (tx or ) involved in the same measurement:

        - Transmitting UE for unidirectional or bidirectional measurements

      - Scenario C: Impact on SL measurements at multiple SL UEs which are sharing the same common synchronization reference source, due to the change of the common synchronization source,

      - Scenario D: Impact on an SL positioning result due to the change in another SL UE involved in the same positioning session but not necessarily the same SL measurement (multi-RTT scenario).

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 1-2-4: UE behavior and the impact on requirements when synchronization reference source change occurs

- Proposals:

  - Proposal 1: (CATT)

    - The measurement period requirements are not impacted due to synchronization reference source change.

  - Proposal 2: (Ericsson):

    - At least SL PRS timing measurements are impacted by the synchronization reference source.

  - Proposal 3: (Ericsson)

    - When the synchronization reference source changes during the on-going SL measurement or right before the SL measurement:

      - Alt 1: Delaying/postponing the start of the synch change until the SL measurement is complete

      - Alt 2: Dropping or restarting the impacted SL measurement

  - Proposal 4: (Huawei)

    - RAN4 to discuss the impact of transition (sync source change) after requirements for non-transition case are stable.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

### Sub-topic 1-3 Others

#### Issue 1-3-1: Applicable CBW for SL positioning:

- Proposals:

  - Option 1: (Qualcomm, Nokia)

    - RAN4 to specify RRM requirement for sidelink positioning up to 100MHz CBW in Rel-18.

  - Option 2: (LGE)

    - RAN4 to specify RRM core requirement for sidelink positioning up to 100MHz CBW in Rel-18. And RRM accuracy requirement follows the limitation of RF spec.

  - Option 3: (Huawei)

    - The measurement period requirements are agnostic to the BW of SL PRS. FFS the applicable PRS BW for defining accuracy requirements.

- Recommended WF

Agreements:

None. No discussion.

#### Issue 1-3-2: Report mapping

- Proposals:

  - Option 1: (vivo)

    - For SL-PRS RSRP, SL-PRS RSRPP, SL-PRS AOA/ZOA, SL-PRS RSTD, SL-PRS RTOA, the report mapping requirements including reporting range, resolution step can be same as Rel-16/Rel-17 report mapping requirement.

  - Option 2: (Ericsson)

    - The measurement ranges for SL positioning measurements (except for SL-PRS AOA/ZOA) can be different compared to those specified for Uu positioning measurements. RAN4 to discuss the relevant range for each SL positioning measurement.

  - Option 3: (Qualcomm)

    - Reuse the UL AoA measurement report mapping in 38.133 for SL AoA measurements.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

## Carrier Phase Positioning (Agenda 8.23.3.6)

### Sub-topic 2-1 Measurement period requirements

#### Issue 2-1-1: Measurement period requirements for DL RSCP/DL RSCPD:

- Proposals

  - Option 1: (CATT)

    - When DL RSCP/RSCPD is reported together with legacy positioning measurement, the existing Rel-17 measurement requirements for single carrier can be reused.

  - Option 2: (OPPO, Huawei, Ericsson, ZTE, Nokia)

    - When DL RSCP/RSCPD is reported together with legacy positioning measurement, the existing requirements for the associated legacy measurement should apply.

  - Option 3: (Qualcomm)

    - Do not define requirements for DL RSCP reported with RSTD.

    - Do not define requirements for DL RSCPD reported with UE Rx-Tx.

  - Option 4: (Qualcomm)

    - Measurement period requirements for CPP shall take into account the time window indication by the LMF.

- Recommended WF

Agreements:

None. No discussion.

#### Issue 2-1-2: Gap conditions for DL carrier phase measurement requirements:

- Proposals

  - Option 1: (OPPO, Nokia)

    - For DL carrier phase measurement in RRC_CONNECTED state, MG-based measurement is supported.

  - Option 2: (Ericsson)

    - RAN4 to not define measurement delay requirements for the case when CPP measurement is performed by UE outside of the MG or within PPW in RRC_CONNECTED state.

- Recommended WF

Agreements:

- For DL carrier phase measurement in RRC_CONNECTED state, MG-based measurement requirements are defined.

#### Issue 2-1-3: Applicable RF frequencies for DL carrier phase measurement requirements:

- Proposals

  - Option 1: (OPPO)

    - Not consider multiple RF frequencies for RSCP/RSCPD measurement if a single RF frequency is defined for a PFL.

  - Option 2: (Huawei, Qualcomm)

    - RAN4 waits for RAN1 conclusion on support of multiple CP measurements for multiple RF frequencies per PFL before discussing the impact on RAN4 requirements.

    - RF frequencies for target TRP and reference TRP in RSCPD measurement is up to RAN1.

  - Option 3: (Ericsson)

    - RAN4 to not define measurement requirements for DL carrier phase measurement for multiple RF frequencies.

    - For RSCPD measurement RF frequencies for target TRP and reference TRP should belong to same PFL.

    - When RSCPD measurement is reported together with RSTD measurement, then RSTD measurement is performed on the same target TRP and reference TRP on which RSCPD measurement is performed.

  - Option 4: (Nokia)

    - In case the network operates multiple PFL’s to support RSTD or UE Rx-Tx time difference measurements, the same PFL per each cell should be configurable for the paired DL CP measurement, i.e. DL RSCPD or DL RSCP, respectively.

    - The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

- Recommended WF

Agreements:

- For RSCP measurement:

  - The measurement is associated with the specific RF frequency.

  - This specific RF frequency can be the center frequency of the DL PFL by default.

- For RSCPD measurement:

  - RF frequencies for target TRP and reference TRP should belong to same PFL.

  - RAN4 can define the requirements starting from the same RF frequency for target and reference TRP.

#### Issue 2-1-4: Impact of UE mobility in RRC_CONNECTED:

- Proposals

  - Option 1: (CATT)

    - The mobility impact in RRC_CONNECTED in Rel-17 can also be reused.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 2-1-5: Impact of collisions with other signals/channels in RRC_INACTIVE:

- Proposals

  - Option 1: (CATT)

    - The collision handling in RRC_INACTIVE state in Rel-17 can also be reused.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

#### Issue 2-1-6: Applicable requirements for PRU:

- Proposals

  - Option 1: (CATT)

    - The measurement period requirements defined in RAN4 apply to both UE and PRU(s).

  - Option 2: (Huawei)

    - RAN4 not to define specific requirements for PRU

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

### Sub-topic 2-2 Measurement reporting requirements

#### Issue 2-2-1: Measurement reporting requirements for DL carrier phase measurement:

- Proposals

  - Option 1: (Huawei)

    - When RSCP or RSCPD is reported together with other measurements, measurement reporting requirements for other measurements apply for CP measurements.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

### Sub-topic 2-3 Others

#### Issue 2-3-2: Report mapping for UL RSCP:

- Proposals

  - Option 1: (Huawei)

    - Same as DL RSCP/RSCPD.

  - Option 2: (Nokia)

    - Define the reporting range [0, 2 𝜋] for UL RSCP.

- Recommended WF

  - Need discussion.

Agreements:

None. No discussion.

# Topic #3: [107][220] NR_pos_enh2_part3

## LPHAP use case (Agenda 8.23.3.3)

### Sub-topic 1-1: eDRX in INACTIVE

#### Issue 1-1-2: PRS requirements with RAN eDRX <= 10.24s, if defined

- Proposals

  - Option 1 (CMCC, OPPO, HW, E///, QC, Nokia):

    - The requirements for RRC_INACTIVE state in R17 can be reused as baseline

  - Option 1a (QC):

    - When CN eDRX <= 10.24 s, TDRX in the measurement period formula is modified to follow the RAN2 definition of UE DRX cycle in Rel-17 TS 38.304.

    - When CN eDRX > 10.24 s, TDRX in the measurement period formula is set to the maximum of the DRX cycles inside and outside of the CN PTW, according to the RAN2 definition of UE DRX cycle in Rel-17 TS 38.304.

- Recommended WF

  - Moderator would like to clarify that the “eDRX” in the context of sub-topic 1 refers to RAN eDRX or eDRX_INACTIVE

  - Agree on option 1, and further discuss option 1a

Agreements:

- The requirements for RRC_INACTIVE state in R17 can be reused as baseline. Details are FFS.

#### Issue 1-1-3: Baseline for PRS requirements with RAN eDRX > 10.24s

- Proposals

  - Option 1 (ZTE, OPPO, HW, QC, Nokia):

    - the requirements for RRC_INACTIVE state in R17 can be reused as baseline

  - Option 2 (CATT):

    - the requirements for eDRX cycle > 10.24 sec in Rel-18 eRedCap WI is used as a baseline

- Recommended WF

  - Discuss the options. Check if option 1 could be agreed based on majority view.

Agreements:

- The requirements for RRC_INACTIVE state with DRX cycle up to 2.56 s in R17 can be used as baseline for RAN eDRX cycle > 10.24 s. Details are FFS.

#### Issue 1-1-4: Number of samples for PRS requirements with eDRX > 10.24s

- Proposals

  - Option 1 (CATT, ZTE, OPPO, HW, E///, Nokia):

    - Both normal and reduced sample numbers are applicable for PRS measurement delay requirements in RRC_INACTIVE state

  - Option 1a (ZTE):

    - Re-use the conditions for reduced sample number from Rel-17 requirements for RRC_INACTIVE

  - Option 2 (CATT):

    - RAN4 to prioritize the measurement requirements with reduced samples

- Recommended WF

  - Agree on option 1, and further discuss option 1a.

Agreements:

- Both normal and reduced sample numbers are applicable for PRS measurement delay requirements in RRC_INACTIVE state

#### Issue 1-1-6: Scaling of legacy requirements related to eDRX

- Proposals

  - Option 1 (ZTE):

    - The legacy measurement period can be used as baseline and define or update a scaling factor related to eDRX in the period requirement calculation.

  - Option 2 (Nokia):

    - RAN4 should reuse the existing framework of requirements rather than introducing a new scaling factor.

  - Option 3 (E///):

    - RAN4 to postpone discussion on the need for any scaling factor related to eDRX in the period requirement calculation until issues related to eDRX > 10.24s are settled.

- Recommended WF

  - Discuss the options

Agreements:

None. No discussion.

#### Issue 1-1-7: eDRX and PRS/SRS alignment

- Proposals

  - Option 1 (OPPO):

    - RAN4 to study the conditions of alignment between eDRX and PRS/SRS configuration, e.g. the distance between PTW and PRS/SRS resource should be less than Tms.

  - Option 2 (E///):

    - RAN4 to wait for progress in RAN2 on alignment between eDRX cycle and PRS configuration before evaluating its impact on PRS measurement requirements.

  - Option 3 (Nokia):

    - Further study whether and how to take the alignment between eDRX and PRS configuration into account based on RAN2 progress

- Recommended WF

  - Discuss the options

Agreements:

None. No discussion.

#### Issue 1-1-8: RRM measurement requirements

- Proposals

  - Option 1 (CATT):

    - RAN4 not to define cell reselection requirement for the determination of validation area.

  - Option 2 (HW):

    - RAN4 to define RRM measurement requirements for positioning specific needs with RRM measurement not limited to PTW, details are FFS.

- Recommended WF

  - Discuss the options

Agreements:

None. No discussion.

### Sub-topic 1-2: SRS positioning validity area

#### Issue 1-2-1: Impact of SRS positioning validity area

- Proposals

  - Option 1 (HW):

    - RAN4 to update the applicability condition of UE Rx-Tx requirements “SRS is configured on the PCell” to e.g. “SRS configuration is valid for the PCell”.

    - RAN4 to wait for further RAN1 progress on SRS UL timing before discussing applicability condition of UE Rx-Tx requirements related to UL timing change.

  - Option 2 (QC, Nokia):

    - Wait for further progress in RAN1 and RAN2 to determine the RRM impact of SRS configuration enhancements in RRC_INACTIVE.

- Recommended WF

  - Discuss the options

Agreements:

None. No discussion.

### Sub-topic 1-3: PRS measurement in IDLE

#### Issue 1-3-1: NRx,TEG,i  and NRxBeam,i in measurement period requirements

- Proposals

  - Option 1 (CATT, HW, E///, Nokia):

    - Reuse values from Rel. 17 for NRx,TEG,i, and NRxBeam,i in positioning measurement delay requirements for RRC_IDLE state.

  - Option 2 (Nokia):

    - Study whether the values have to be adjusted if the number of Rx beams for PRS measurement is reduced in RRC_IDLE

- Recommended WF

Agreements:

- Reuse values from Rel. 17 for NRx,TEG,i, and NRxBeam,i in positioning measurement delay requirements for RRC_IDLE state subject to RAN1/RAN2 agreement on UE capability for reduced beam sweeping factor in RRC_IDLE.

#### Issue 1-3-2: Number of samples in measurement period requirements

- Proposals

  - Option 1 (CATT, CMCC, HW, E///, Nokia):

    - Both normal and reduced sample numbers are applicable for PRS measurement delay requirements in RRC_IDLE state

- Recommended WF

  - Agree on option 1

Agreements:

None. No discussion.

#### Issue 1-3-3: Tavailable for PRS requirements

- Proposals

  - Option 1 (QC):

    - When eDRX <= 10.24 s, TDRX in the measurement period formula is modified to follow the RAN2 definition of UE DRX cycle in Rel-17 TS 38.304.

    - When eDRX > 10.24 s,

      - TDRX in the measurement period formula is modified to follow the RAN2 definition of UE DRX cycle in Rel-17 TS 38.304

      - the measurement period starts on the first eDRX cycle inside the next PTW that contains PRS resources in the assistance data. The measurement period can extend beyond the end of the PTW.

- Recommended WF

  - Discuss option 1

Agreements:

None. No discussion.

#### Issue 1-3-3: Reporting delay requirements

- Proposals

  - Option 1 (CATT, E///):

    - The reporting delay for PRS measurements performed in RRC_IDLE doesn’t need to account for extra delay to transition to RRC_CONNECTED state

  - Option 2 (HW, QC, Nokia):

    - The reporting delay for PRS measurements performed in IDLE needs to account for extra delay to transition to CONNECTED state.

- Recommended WF

  - Check whether it is agreeable to re-use the same wording related to the extra delay for transition to RRC_CONNECTED in Rel-17 reporting delay requirements in RRC_INACTIVE for RRC_IDLE.

For RSTD measurements performed by the UE in RRC_INACTIVE state, The measurement reporting delay excludes all of the following: - additional delay caused other LPP signalling on the DCCH, - delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration, - any delay caused by unavailability of UL resources to transmit the measurement report, - any transmission delay needed by SDT, - the time needed to transition to RRC_CONNECTED state to report the measurements.

Agreements:

- Reuse the same wording related to the extra delay for transition to RRC_CONNECTED in Rel-17 reporting delay requirements in RRC_INACTIVE for RRC_IDLE.

#### Issue 1-3-3: Transition requirements

- Proposals

  - Option 1 (E///):

    - When UE changes its state from RRC_IDLE to RRC_CONNECTED, or cell reselection occurs while performing PRS measurement then the following rules apply:

      - If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the RSTD measurement period, then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

      - If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period, then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

      - If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

      - If cell reselection occurs during the UE Rx-Tx time difference measurement period, then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

- Recommended WF

  - Discuss option 1

Agreements:

None. No discussion.

#### Issue 1-3-4: Accuracy requirements

- Proposals

  - Option 1 (CATT):

    - Accuracy requirements in RRC_INACTIVE state also apply to positioning measurements in RRC_IDLE.

  - Option 2 (Nokia):

    - The accuracy requirements on the reduced sample number should be discussed as a part of performance requirement.

- Recommended WF

Agreements:

None. No discussion.