# 3GPP TSG-RAN WG4 Meeting # 107                                                            R4-2310244

Incheon, Korea, 22th May 2023 - 26th May 2023

Source:	China Unicom

Title:	TP for TR 38.896 to include n8 PC2 RSD values

Agenda item:			7.22.2

Document for:	Approval

1. Introduction

A text proposal for TR 38.896  to update the latest analysis[1] from contributing companies on n8 single band PC2 RSD requirements.

2. Reference

[1] R4-2306544	WF on FDD HPUE Basket WIs, China Unicom

3. Text Proposal

---Start of changes---

5.1	NR band n8

5.1.1	UE maximum output power

Table 5.1.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n8 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.1.2	A-MPR requirements

FFS.

5.1.3	Reference sensitivity requirements

For PC2 single band n8, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n8 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.1.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source |  |  |  | 5 MHz (dB) |  | 10 MHz (dB) | 15 MHz (dB) |  | 20 MHz (dB) |  | 25 MHz (dB) |  | 30 MHz (dB) |  | 35 MHz (dB) | 40 MHz (dB) |  | 45 MHz (dB) |  | 50 MHz (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | Skyworks(R4-2300652, R4-2307239) |  |  |  | 0.3 |  | 0.4 | 0.7 |  | 1.9 |  | 2.1 |  |  |  | 2.6 |  |  |  |  |  |  |
|  | Apple(R4-2300362) |  |  |  | 0 |  | 1.2 | 0.9 |  | 1.9 |  | N/A |  | N/A |  | 4.4 | N/A |  | N/A |  | N/A |  |
|  | Huawei, HiSilicon(R4- 2300715, R4-2307916) |  |  |  | 1.2 |  | 1.2 | 1.4 |  | 3.3 |  | 3.4 |  | - |  | 3.8 | - |  | - |  | - |  |
|  | Murata(R4-2305393, R4-2308436) |  |  |  | 0.4 |  | 0.3 | 0.3 |  | 2.4 |  | 2.8 |  | - |  | 2.9 | - |  | - |  | - |  |
|  | Qualcomm(R4-2302709) |  |  |  | 0.8 |  | 0.6 | 0.7 |  | 1.8 |  | - |  | - |  | 1.8 |  |  |  |  |  |  |
| Average |  |  |  |  | 0.5 |  | 0.7 | 0.8 |  | 2.3 |  | 2.8 |  | - |  | 3.1 |  |  |  |  |  |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.1.3-2:

Table 5.1.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source |  |  |  | 5 MHz (dB) |  | 10 MHz (dB) | 15 MHz (dB) |  | 20 MHz (dB) |  | 25 MHz (dB) |  | 30 MHz (dB) |  | 35 MHz (dB) | 40 MHz (dB) |  | 45 MHz (dB) |  | 50 MHz (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | Skyworks(R4-2300652, R4-2307239) |  |  |  | 0.8 |  | 1.0 | 1.5 |  | 5.1 |  | 5.6 |  |  |  | 6.6 |  |  |  |  |  |  |
|  | Apple(R4-2305364) |  |  |  | 0.5 |  | 0.5 | 0.9 |  | 6.2 |  | N/A |  | N/A |  | 8.1 | N/A |  | N/A |  | N/A |  |
|  | Huawei, HiSilicon(R4- 2300715, R4-2307916) |  |  |  | 3.1 |  | 3.2 | 3.8 |  | 6.2 |  | 6.5 |  | - |  | 7.1 | - |  | - |  | - |  |
|  | Murata(R4-2305393, R4-2308436) |  |  |  | 1.8 |  | 1.8 | 2.2 |  | 5.6 |  | 6.1 |  | - |  | 6.8 | - |  | - |  | - |  |
|  | Qualcomm(R4-2302709) |  |  |  | 1.5 |  | 1.5 | 2.2 |  | 6.5 |  | - |  | - |  | 5.9 |  |  |  |  |  |  |
|  | MTK (R4-2219869) |  |  |  | 0 |  | 0 | 1 |  | 5.4 |  | - |  | - |  | 7.2 | - |  | - |  | - |  |
|  | ZTE, CU (R4-2215893) |  |  |  | 0.5 |  | 0.6 | 1.2 |  | 5.0 |  | - |  | - |  | 6.7 | - |  | - |  | - |  |
|  | Spreadtrum(R4-2304084) |  |  |  | 2.2 |  | 2.2 | 3.8 |  | 6.5 |  |  |  |  |  | 7.3 |  |  |  |  |  |  |
| Average |  |  |  |  | 1.3 |  | 1.4 | 2.1 |  | 5.8 |  | 6.1 |  | - |  | 7.0 |  |  |  |  |  |  |

---End of changes---