3GPP RAN WG4 Meeting #107		R4-2310254

Incheon, South Korea, May 22nd – 26th, 2023

Title:	WF on UE RF requirements for CA_n5-n8

Agenda item:	7.30.2.1

Release:	Rel-18

Source:	Skyworks Solutions, Inc., Qualcomm, Huawei

Document for:	Approval

# 1	Background

To operate CA_n5-n8, the overlap of n5 DL and n8 UL needs to be managed. There are three option of UL support by the UE to manage this:

- Implementation 1: Support UL in band n5 only (which can be supported by legacy UEs and can thus be a baseline support)

- Implementation 2: Support non-concurrent n5 Rx and n8 Tx (optional support pending input from RAN2 on managing 1DL and 2UL configuration)

- Implementation 3: Only support an UL range restricted to 904-915MHz for n8 (based on implemented a dedicated RF filter at least for band n8 UL)

This way forward covers the RF requirements for the different UL configuration support and the signaling aspects to support them.

# Way forward on UE RF Requirement

## Support of Option 1: UL in n5 only and full band support

There were 3 inputs to the meeting:

- No MSD for a 3 antenna architecture

- 1.3 MSD to the lowest n8 5MHz channel (two-antenna architecture)

- 6.6dB MSD (two-antenna architecture) to the lowest restricted spectrum band n8 5MHz channel. After further discussion the value was re-considered and 5.2dB proposed

Since this particular option is not requiring the UL of n8 to use the restricted spectrum the convention would be to use the lowest n8 possible channel.

On the other hand, this n5 1UL is a fall back of option 2 and 3 and it is better to have a frequency that can be tested for all options and thus use the lowest 5MHz in the restricted n8 frequency range

Since the MSD provided are anyhow related to the transmitter noise floor, they are valid for any 5MHz channel in band n8. Thus, an average of the two values (using corresponding linear values) can be used with the 5MHz channel as agreed in last meeting way forward.

The average value based on the 1.3 and 5.2dB two-antenna values proposed is 2.8dB (based on linear average).

<WF1 on RF requirement for option 1 (UL in n5 only and full band support)>

Applicable to UEs supporting CA_n5-n8 DL and n5 UL only

Note: Option 1 is a fallback UL configuration for Option 2 and option 3 and thus the associated RF requirements are needed even if this option is down selected

Based on the average of the two-antenna data provided, the following 1UL cross band MSD is specified:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 951.5 | 5 | [2.8] | >ACLR2 |

## Support of Option 2: 2UL CA_n5-n8 with n5 DL and full band support

There were 2 inputs to the meeting on the IMD3 related MSD, both proposing 25dB MSD but with sightly different frequencies.

One of the proposed test point as a better centering of the MSD and can be used for the MSD test point.

<WF2 on RF requirement for option 2 (non-concurrent n8 UL and n5 DL and full band support >

Applicable to UEs supporting n5 DL and CA_n5-n8 UL or CA_n5-n8 DL and n5 UL

The following 2UL IMD3 MSD is specified pending RAN2 feasibility answer:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 845 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 (RBstart=0) | 935 | 25 | FDD | IMD3 |

## Support of Option 3: UL/DL CA_n5-n8 with n8 UL restricted to 904-915MHz

In this case there are two potential issues:

- 1UL n8 cross-band MSD into n5 with n8 UL using the restricted spectrum (n8R)

- Near miss IMD3 of CA_n5-n8R into n5

<WF3 on 1UL n8R MSD into n5>

Applicable to UEs supporting n5 DL and CA_n5-n8 UL or CA_n5-n8 DL and n5 UL

The following 1UL n8R cross-band MSD is specified if this option is confirmed:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | [12.3] | >ACLR2 |

<WF4 on 2UL CA_n5-n8R IMD3 MSD into n8>

Applicable to UEs supporting n5 DL and CA_n5-n8 UL or CA_n5-n8 DL and n5 UL

The following 2UL IMD3 MSD is specified if this option is confirmed:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 (RBstart=0) | 891.5 | N/A | FDD | N/A |
|  | n8 | 906.5 | 5 | 25 (RBstart=0) | 951.5 | [1.6]X | FDD | Near miss IMD3 |

# Way forward on capturing UE UL/DL support

The network will need to be aware of the UE UL support capability to be able to schedule the UL and DL accordingly. This is in addition to the question raised to RAN2 about the feasibility of a n5DL and CA_n5-n8 UL configuration for the option 2.

Multiple solutions were proposed:

- Option 1: different BCS are specified depending on UL configuration support

- Option 2: dedicated UE capability

- Option 3: notes with a single BCS

The following concerns/ideas are raised in the discussion, they can be considered in the future discussion.

<WF5 on signalling the different UL support options>

- If new approach will be introduced. The impact to the general CA/DC work should be considered. The potential new approach will not be used for any other CA by default

- If introduced, UE capability signalling will target to solve issues in DL/UL configuration for overlap or neighbour spectrums

- The UL/DL UE support capability description should target RAN4 specification only

# References

R4-2309359 Considerations on CA_n5-n8, Qualcomm, RAN4#107

R4-2309004 Discussion on CA_n5-n8, Xiaomi, RAN4#107

R4-2307123 Possible issue to differentiate restricted operations with BCS, Nokia, Nokia Shanghai Bell, RAN4#107

R4-2307603 UE RF requirements for dedicated filter for CA_n5-n8R, China Telecom, RAN4#107

R4-2308575 Discussion on RF requirements for CA_n5-n8, Huawei, HiSilicon, RAN4#107

R4-2309295 Input on CA_n5-n8 options and MSD, Skyworks Solutions Inc., RAN4#107