3GPP TSG-RAN WG4 Meeting # 107	R4-2307058

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for TR 38.872 on CA_n28-n105 1UL and 2UL configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc., Qualcomm, Huawei, Spark NZ, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the following CA_n28-n105 REFSENS aspects in the TR: 5.5.3 Common for 1 band UL and 2 bands UL of CA_n28-n105 5.5.4 Specific for 2 bands UL of CA_n28-n105 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.5.3 Common for 1 band UL and 2 bands UL of CA_n28-n105: - Delta T and Delta R - Cross band MSD for n105 from n28 UL - Cross band MSD for n28 from n105 UL 5.5.4 Specific for 2 bands UL of CA_n28-n105 - 2UL IMD3/5 issue in n28DL - 2UL IMD3/5 issue in n105DL - 2UL IMD3/5 issue in DTV channels - Reason to remove 2UL configuration for CA_n28-n105 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot capture study conclusions on CA_n28-n105 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3, 5.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change>>

### 5.5.3	Common for 1 band UL and 2 bands UL of CA_ n28-n105

#### 5.5.3.0	General

This band combination is similar to CA_n71-n85 with the two UL bandwidths having no frequency gap between them. CA_n71-n85 is being specified for 1UL only as 2 UL showed significant MSD challenges.

#### 5.5.3.1	Operating bands for CA

Table 5.5.3.1-1: CA band combination of band n28+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

#### 5.5.3.2	Channel bandwidths per operating band for CA

Based on WF R4-2306470, it was agreed that only 1UL configuration will be supported, the 2UL issues will be described in section 5.5.4. Thus, BCS0 for CA_n28A-n105A can be specified as below.

Table 5.5.3.2-1: Supported bandwidths per CA band combination of band n28+n105

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n105A | - | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 251, 301, 351 |  |
| NOTE 1: These n105 channel bandwidths are valid in DL only |  |  |  |  |

#### 5.5.3.3	UE co-existence studies

Table 5.5.3.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n28-n105.

Table 5.5.3.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |  | 5th Harmonic |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 |  | 2812 | 2992 |  | 3515 | 3740 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 |  | 2652 | 2812 |  | 3315 | 3515 |

Based on the above table, there is no UL harmonic issue for CA_n28-n105.

Table 5.5.3.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2nd Harmonic |  |  |  | 3rd Harmonic |  |  | 4th Harmonic |  |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | UL Low Band Edge |  | UL High Band Edge |  |  | DL Low Band Edge |  |  | DL High Band Edge |  |  | DL Low Band Edge |  |  | DL High Band Edge | DL Low Band Edge |  | DL High Band Edge | DL Low Band Edge |  | DL High Band Edge | DL Low Band Edge |  | DL High Band Edge |
| n28 | 703 |  | 748 |  |  | 758 |  |  | 803 |  |  | 1516 |  |  | 1606 |  | 2274 |  | 2409 | 3032 |  | 3212 | 3790 |  | 4015 |
| n105 | 663 |  | 703 |  |  | 612 |  |  | 652 |  |  | 1224 |  |  | 1304 |  | 1836 |  | 1956 | 2448 |  | 2608 | 3060 |  | 3260 |

Based on the above table, there is no harmonic mixing issue for CA_n28-n105.

Since the two bands are very close, cross-band isolation may be an issue as shown in Figure 5.5.3.3-3 below, the Table 5.5.3.3-4 provides the analysis for the potential IMD order of the UL configuration and its image for the largest channel bandwidths.

Figure 5.5.3.3-3: IMD landscape for 1UL and 2UL configurations of CA_n28-n105

Table 5.5.3.3-4: Cross band isolation issues

| UL Band |  | UL Low Band Edge |  | UL High Band Edge |  | DL Band |  | DL Low Band Edge | DL High Band Edge | UL to DL distance |  | Maximum UL BW |  | Source of interference (IMD order of UL RBs and their image) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 703 |  | 748 |  | n105 |  | 612 |  | 652 |  | 51 |  | 30 |  | ACLR2 (IMD5) |
| n105 | 663 |  | 703 |  | n28 |  | 758 |  | 803 |  | 55 |  | 20 |  | ACLR3 (IMD7) |

As shown in the above table, the band n28 30MHz CBW UL ACLR2 overlaps with the band n105 DL while the band n105 20MHz CBW UL ACLR3 overlaps with band n28 DL. Both MSDs need to be evaluated.

#### 5.5.3.4	∆TIB,c and ∆RIB,c values

For CA_n28-n105 the 2 antenna implementation should drive the TIB,c and RIB,c values, three companies provided input that is summarized in the two tbles below.

Table 5.5.3.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 Proposed values by company A / B / C |  |
| CA_n28-n105 | 1.0 / 0.7 / 0.8 | 1.0 / 0.7 / 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.5.3.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 Proposed values by company A / B / C |  |
| CA_n28-n105 | 1.0 / 0.4 / 0.2 | 1.0 / 0.4 / 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.5.3.5	REFSENS evaluation

For CA_n28-n105, the 2 antennas implementation n28 and n105 MSDs related to cross band isolation described in Table 5.5.3.3-4 must be evaluated.

Three companies provided their evaluation for both cross band MSDs.

For n28 MSD due to n105 UL, one company suggested that no MSD was needed while the two other companies’ input can be found in Table 5.5.3.5-1a and b respectively.

Table 5.5.3.5-1a n28 MSD due to cross band isolation for CA_n28-n105 with n105 UL (company A)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 3.7 | >ACLR2 |

Table 5.5.3.5-1b n28 MSD due to cross band isolation for CA_n28-n105 with n105 UL (company B)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 10.8 | >ACLR2 |

Similarly, for n105 MSD due to n28 UL, the three companies’ input can be found in Table 5.5.3.5-1a, b and c.

Table 5.5.3.5-2a n105 MSD due to cross band isolation for CA_n28-n105 with n28 UL (company A)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 17 | ACLR2 |

Table 5.5.3.5-2b n105 MSD due to cross band isolation for CA_n28-n105 with n28 UL (company B)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 12.7 | ACLR2 |

Table 5.5.3.5-2c n105 MSD due to cross band isolation for CA_n28-n105 with n28 UL (company C)

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 10.1 | ACLR2 |

### 5.5.4	Specific for 2 bands UL of fallback CA_n28-n105

#### 5.5.4.1	Maximum output power for inter-band CA

Power class 3 was assumed for UL CA_n28-n105, but as discussed in the next clauses, it was decided that the two UL configuration will not be supported due to significant IMD3 MSD and potential co-existence issues with DTV channels.

#### 5.5.4.2	UE co-existence studies

Table 5.5.4.2-1 lists Band n28 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.5.4.2-1 IMD interference analysis for CA_n28-n105 with 2 ULs

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1406 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2109 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2812 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3515 | 3515 | 3740 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4218 | 4218 | 4488 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4921 | 4921 | 5236 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 85 | 0 | 1366 | 1451 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 578 | 703 | 703 | 833 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2029 | 2154 | 2069 | 2199 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 170 | 0 | 2732 | 2902 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1241 | 1406 | 1406 | 1581 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2692 | 2857 | 2772 | 2947 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2329 | 2109 | 2109 | 1904 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3475 | 3695 | 3355 | 3560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 918 | 703 | 703 | 493 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3435 | 3650 | 3395 | 3605 |

From the above table, it can be found that both n28 DL and n105 DL are subject to MSD due to dual-uplink IMD3 and IMD5 of 2UL CA_n28-n105 configuration. Furthermore, the DTV channels below 608 MHz are also victims of the CA_n28-n105 IMD3 and higher orders and thus the coexistence between UL CA_n28-n105 and DTV channels may be compromised.

#### 5.5.4.3	REFSENS evaluation

Based on the above IMD analysis, the IMD3 and IMD5 MSD test points in Table 5.5.4.2-1 were discussed. Some estimations of the IMD3 MSD ranged between 25 dB and 44dB depending on filters’ performance and antenna coupling assumptions. Based on this large MSD and the potential coexistence issue with DTV channels, it was agreed together with the band combination proponent, that 2UL configuration will not be supported. This agreement is captured in way forward R4-2306470.

Table 5.5.4.3-1 potential MSD test points due to IMD3 and IMD5 if 2UL configuration would be specified.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n105 | n28 | 745.5 | 5 | 25 | 800.5 | 2UL not specified | FDD | IMD3 |
|  | n105 | 690.5 | 5 | 25 | 639.5 | N/A | FDD | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | IMD3 |
|  | n105 | 694.5 | 5 | 25 | 643.5 | 2UL not specified | FDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 2UL not specified | FDD | IMD5 |
|  | n105 | 687.5 | 5 | 25 | 636.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | IMD5 |
|  | n105 | 690 | 5 | 25 | 639 | 2UL not specified | FDD | N/A |

## <<End of Change>>