3GPP TSG-RAN WG4 Meeting # 107									       R4-2310256

Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR |  | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.872 to introduce CA_n5-n105 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Huawei, HiSilicon, Skyworks Solutions Inc., Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-05-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the study results of CA_n5-n105 into TR 38.872 based on operators’ request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the spectrum allocation of CA_n5-n105 in section 5.4.1. - Add the potential UE RF architecture to support CA_n5-n105 in section 5.4.2. - Add the RF requirement study for CA_n5-n105 with 1UL configuration in section 5.4.3. - Add the RF requirement study for CA_n5-n105 with 2UL configuration in section 5.4.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study results of CA_n5-n105 would be missing in TR 38.872. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2, 5.4.3, 5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2309006 |  |  |  |  |  |  |  |  |

## << Start of change 1>>

## 5.4	CA_n5-n105

### 5.4.1	General

In this configuration the uplink is also CA_n5A-n105A, but there are two fall back options:

- DL_n5A-n105A_UL_n5A_BCS0

- UL_n5A-n105A_UL_n105A_BCS0

The aggregated spectrum allocation for CA_n5A-n105A is illustrated in Figure 5.4.1-1.

Figure 5.4.1-1: Aggregated spectrum allocation for CA_n5A-n105A

## << End of change 1>>

## << Start of change 2>>

### 5.4.2	UE RF architecture assumption

The UE RF architectures with 2 antenna and 3 antenna are assumed in the feasibility study for CA_n5-n105. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

#### 5.4.2.1	UE RF architecture with two antennas

For CA_n5-n105 with 2-antenna implementation, there are two potential UE RF architectures as shown in Figure 5.4.2.1-1.

#### Figure 5.4.2.1-1a: Potential UE architecture with 2 antenna to support CA_n5-n105

#### Figure 5.4.2.1-1b: Potential UE architecture with 2 antenna to support CA_n5-n105

The topology for 2-antenna architecture illustrated in Figure 5.4.2.1-1a is listed as below.

-	1) n105UL+n105DL+n5UL+n5DL quadplexer on antenna 1

-	2) n5DL+n105DL duplexer on antenna 2

The topology for 2-antenna architecture illustrated in Figure 5.4.2.1-1b is listed as below.

-	1) n5UL+n5DL+n105DL triplexer on antenna 1

-	2) n105UL+n105DL+n5DL triplexer on antenna 2

#### 5.4.2.2	UE RF architecture with three antennas

Figure 5.4.2.2-1 illustrates the potential UE RF architecture for CA_n5-n105 with 3-antenna implementation.

Figure 5.4.2.2-1: Potential UE architecture with 3 antenna to support CA_n5-n105

The topology for 3-antenna architecture illustrated in Figure 5.4.2.2-1 is listed as below:

1) n105UL+n105DL duplexer on antenna 1

2) n5UL+n5DL duplexer on antenna 2

3) n5DL+n105DL duplexer on antenna 3

## << End of change 2>>

## << Start of change 3>>

### 5.4.3	Common for 1 band UL and 2 bands UL of CA_n5-n105

#### 5.4.3.1	Operating bands for CA

#### The operating bands for CA_n5-n105 are provided in Table 5.4.3.1-1.

Table 5.4.3.1-1: CA band combination of band n5+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

#### 5.4.3.2	Channel bandwidths per operating band for CA

#### The supported channel bandwidths for CA_n5-n105 are provided in Table 5.4.3.2-1.

Table 5.4.3.2-1: Supported bandwidths per CA band combination of band n5+n105

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

#### 5.4.3.3	Identify the potential MSD issues due to harmonic/harmonic mixing/cross band isolation

Table 5.4.3.3-1/2 summarizes frequency ranges where harmonics/harmonics mixing occur for CA_n5-n105.

Table 5.4.3.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Table 5.4.3.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on the above analysis, there is no need to study MSD due to harmonics and harmonics mixing for CA_n5-n105.

#### 5.4.3.4	∆TIB,c and ∆RIB,c values

Two-antenna architecture with dual triplexer can be used as a baseline to study Delta Tib and Rib.

By referring to R4-2308581, it seems that two-antenna solutions with quadplexer or triplexer are not very challenging for CA_n5-n105 as the frequency gap between band n5 UL part and band n105 DL part is larger than 100MHz. Thus, general values for low band combinations can be used for Delta Tib and Rib.

Table 5.4.3.4-1: ΔTIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.5 |

Table 5.4.3.4-2: ΔRIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0 |
|  | n105 | 0 |

By referring to R4-2309299, the two antenna implementation should drive the TIB,c and RIB,c values for CA_n5-n105. Based on other LB-LB cases, and especially DC_5_n71, a Delta T of 0.5dB for both bands can be used. For Delta Rib, CA_n5-n12 values can be reused.

Table 5.4.3.4-3: ΔTIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.5 |

Table 5.4.3.4-4: ΔRIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.3 |

By referring to R4-2309356, the following values are derived for Delta Tib and Rib:

Table 5.4.3.4-5: ΔTIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.5 |
|  | n105 | 0.5 |

Table 5.4.3.4-6: ΔRIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n105 | n5 | 0.2 |
|  | n105 | 0.2 |

#### 5.4.3.5	REFSENS exception evaluation for one UL band

By referring to R4-2308581, the following evaluation for MSD due to cross band isolation is provided for CA_n5-n105.

Table 5.4.3.5-1 MSD due to cross band isolation n5 to n105

|  | parameters | n105 main path | n105 diversity path |
| --- | --- | --- | --- |
| transmit power for n5, dBm |  | 23 | 23 |
| Tx band n5 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n5 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n105 receiver signal at ANT port, dBm |  | 13 | 13 |
| n105 filter rejection at 814~849MHz, dB | 40 |  |  |
| signal After n105 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 11.8 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162.2 |  |  |
| Rx band n105 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.67 | -95.67 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n5 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -20.01 |  |
| n5 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.01 |  |
| n5 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n5 PA noise PSD at PA output port, dBm/RxBW |  | -63.47 |  |
| n5 Tx filter rejection at 612~652MHz, dB | 30 |  |  |
| n5 PA noise power at Rx n105 ant port at 612~652MHz, dBm |  | -103.47 | -103.47 |
| Total noise level at ANT port |  | -94.79 | -94.79 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -95.79 | -95.79 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -96.80 |  |  |
| MSD | 0.40 |  |  |

Table 5.4.3.5-2 MSD due to cross band isolation n105 to n5

|  | parameters | n5 main path | n5 diversity path |
| --- | --- | --- | --- |
| transmit power for n105, dBm |  | 23 | 23 |
| Tx band n105 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n105 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n5 receiver signal at ANT port, dBm |  | 13 | 13 |
| n5 filter rejection at 663~703MHz, dB | 40 |  |  |
| signal After n5 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 11 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -163 |  |  |
| Rx band n5 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -96.47 | -96.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n105 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -20.01 |  |
| n105 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.01 |  |
| n105 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n105 PA noise PSD at PA output port, dBm/RxBW |  | -63.47 |  |
| n105 Tx filter rejection at 869~894MHz, dB | 30 |  |  |
| n105 PA noise power at Rx n105 ant port at 869~894MHz, dBm |  | -103.47 | -103.47 |
| Total noise level at ANT port |  | -95.43 | -95.43 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -96.43 | -96.43 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -97.44 |  |  |
| MSD | 0.56 |  |  |

Based on the evaluation above, the REFSENS degradation can be neglected. Thus, there is no need to specify MSD due to cross band isolation for CA_n5-n105.

By referring to R4-2309299, the 1UL cross band interference is as follows for the IMD orders of the allocated UL RBs and its image:

- For the n105 20MHz maximum UL CBW, the IMD interference order to n5 is >9

- For the n5 20MHz maximum UL CBW, the IMD interference order to n105 is >9

Given this, there is no MSD due to IMDs. Nonetheless, the transmitter noise floor must be considered.

With the architecture aspects discussed above and the need to support multiple LBLB cases, conservative values should be considered:

- 45dB cross band Tx-Rx isolation for the triplexers

- 40dB Tx rejection in the other DL band

- 10dB antenna couplingf

- 4dB post PA losses

- Transmitter noise floor of -125dBm/Hz.

Based on these assumptions, the MSD with MRC combining is calculated in Table 5.4.3.5-3.

Table 5.4.3.5-3: MRC calculations for band n5 and n105 cross-band MSD.

|  | band | n105 |  | n5 |  |
| --- | --- | --- | --- | --- | --- |
| CBW/NRB | [MHz]/# | 5 | 25 | 5 | 25 |
| transmitter noise floor | [dBm/CBW] | -58.5 |  | -58.5 |  |
| 5MHz CBW REFSENS | [dBm/CBW] | -97.2 |  | -98.0 |  |
| RX noise floor wo Tx noise: Main/Div | [dBm/CBW] | -93.2 | -93.2 | -94.0 | -94.0 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 45 | 40 | 45 | 40 |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -99.5 | -104.5 | -99.5 | -104.5 |
| noise degradation due to Tx noise: Main/Div | [dB] | 0.9 | 0.3 | 1.1 | 0.4 |
| # noise degradation due to Tx noise: Main/Div | # [lin] | # 1.2 | # 1.1 | # 1.3 | # 1.1 |
| REFSENS degradation after MRC | uncor [dB] | 0.6 |  | 0.7 |  |
| REFSENS degradation after MRC | cor [dB] | 1.1 |  | 1.3 |  |

1UL cross-band MSD for CA_n5-n105 is proposed in Table 5.4.3.5-4.

Table 5.4.3.5-4: CA_n5-n105 1UL cross-band MSD

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 1.3 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 1.1 | >ACLR2 |

By referring to R4-2309356, one aspect that needs to be pointed out is that n105 is a very new band, with very challenging frequency arrangements from RF filtering point of view. Hence, especially given the band is new and implementations are just under development, we don’t see it possible to optimize the CA characteristics at the expense of attenuation at CH36 and n105 TX/RX isolations. Please note that the numbers are not derived from dedicated triplexers, but instead from what can be achieved by matching DPX and RX filters together, without optimizations which may offer improved performance over time.

The MSD with MRC combining is calculated in Table 5.4.3.5-5 and Table 5.4.3.5-6.

Table 5.4.3.5-5 MSD due to cross band isolation n105 to n5

Table 5.4.3.5-6 MSD due to cross band isolation n5 to n105

1UL cross-band MSD for CA_n5-n105 is proposed in Table 5.4.3.5-7.

Table 5.4.3.5-7: CA_n5-n105 1UL cross-band MSD

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.6 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 5.6 | >ACLR2 |

## << End of change 3>>

## << Start of change 4>>

### 5.4.4	Specific for 2 bands UL of CA_n5-n105

#### 5.4.4.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n105

#### 5.4.4.2	UE co-existence studies

Table 5.4.4.2-1 lists Band n5 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.4.4.2-1: Band n5 and Band n105 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Based on the above analysis, there is no IMD issue for CA_n5-n105 with 2UL configuration.

Table 5.4.4.2-2 lists the protected bands required for CA_n5-n105 with 2UL configuration.

Table 5.4.4.2-2: Band n5 and Band n105 UL IMD products

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n105 | E-UTRA Band 1, 3, 4, 5, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 66, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 41, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

#### 5.4.4.3	REFSENS exception evaluation for two UL band

It is not required to study the MSD due to two UL IMD interference since there is no two UL IMD issue for CA_n5-n105.

## << End of change 4>>