3GPP RAN WG4 Meeting #107		R4-2310318

Incheon, South Korea, May 22nd – 26th, 2023

Title:	WF on MSD test point for n71-n85 depending on max BW and legacy/new BW support

Agenda item:	7.1.1.1

Release:	Rel-18

Source:	T-Mobile USA, Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

To finalise CA_n71-n85 BCS4/5 request, the main aspect left open is the cross-band MSD of band n85 due to the new largest Band n71 UL CBW. The latest RAN approved maximum symmetrical UL/DL CBW of 30MHz has been used so far in deriving the cross-band MSD. However, the band combination proponent has now requested 35MHz symmetrical UL/DL CBW for n71.

Separately the Release 18 specification will have to handle two types of UEs: Legacy UEs that only support up to 20MHz UL CBW and UEs that will support all or part of the new symmetrical UL CBW.

This way forward provides input on MSD test point and values when available depending on UE UL CBW.

# Way forward

<WF1 applicable band n71 cross band MSD due to n85 UL>

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

<WF2 applicable band n85 cross band MSD due to n71 UL applicable to UEs not supporting optional maximum symmetrical CBW >

- Applicable to legacy UEs or UEs supporting optional symmetrical UL/DL CBW for band n71 but not the maximum one

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

<WF3 band n85 cross band MSD due to n71 UL applicable to UEs supporting optional maximum symmetrical CBW to be evaluated for RAN4#108>

- Applicable to UEs supporting the maximum optional symmetrical UL/DL CBW in band n71

- If max CBW is 35MHz

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | TBDX | ACLR1 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

- If max CBW is 30MHz

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 683 | 30 | 15 | 20 (Rbstart=140) | 730.5 | 5 | TBDX | ACLR2 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

# References

R4-2309363 New test point for CA_n71-n85 cross band MSD, Skyworks Solutions Inc. RAN4#107`