3GPP TSG-RAN WG4 #107 Meeting 	R4- 2310378

Incheon, Republic of Korea, 22nd – 26th May, 2023

Agenda Item:	8.31.2.1.2

Title:	TP for TR 38.786 on the MPR/A-MPR simulation assumptions

Source: 	Meta Ireland

Document for:	Approval

Background

This contribution provides text proposal on the MPR/A-MPR simulation assumptions for SL-U operation in unlicensed band based on the agreed WF [1].

Text Proposal

<<<<<<<<<<< Start of changes in section 6 >>>>>>>>>>

6	Transmitter characteristics for NR SL evolution

6.1	Tx requirements for NR SL single carrier operation in unlicensed band

6.1.1	Maximum output power for NR SL-U

6.1.2	UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

- Basic RF parameters for MPR of single carrier SL-U UE

  - Waveforms: CP-OFDM for SL-U

  - Supported CBW: 20/40/60/80/100MHz.

  - Modulation: QPSK/16-QAM/64-QAM/256-QAM

  - SCS: 15/30/60kHz

  - C-IMD: 45dBc or 60dBc

  - EVM and impairments

| Modulation | SystemEVM | PA only | Image | PA+image |
| --- | --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 28dB | 10.0% |
| 16QAM | 12.5% | 8% | 28dB | 8.0% |
| 64QAM | 8% | 4% | 28dB | 5.65% |
| 256QAM | 3.5% | 1.8% | 34dB | 2.69% |

  - IBE and impairment exceptions are same as NR-U

- Reuse IBE mask from TS38.10101 Table 6.4F.2.3-1

- Image/carrier exceptions position and specific test according to interlaces.

  - ACLR: 27dBc for both PC3/PC5 UE with NR MBW

  - SEM according to 38.101-1 clause 6.5F.2.2

  - PA calibration & PA configuration:

- PC3 (both single PA and dual PAs with 20+20): 1dB MPR and DFT-s-OFDM QPSK with 100RB0 20MHz waveform with 30dB ACLR

- PC5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

  - MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission.

  - The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

  - Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-2: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - 1 RB per user - All users have the same power per RB - Total power of all users equals 23 or 26dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For multiple RB-sets, RAN4 only considers contiguous RB sets. - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed - MPR will be derived by non-contiguous PSFCH RB allocation (N>1) - At least, the worst cases with possible RBstart and Ngap need to be checked. ( Ngap = RBend – RBstart ) For example: The worst case N gap is (106-1 =105*15kHz*12=) 18.9MHz for 20MHz, 15kHz SCS - IMD problem by dual PSFCH in SEM/SE region shall be considered to derive MPR level according to all supporting CBW and SCS. - N (Number of users) is up to 5 and RBs except for RBstart and RBend can be inserted between RBstart and RBend randomly. - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision. |

For S-SSB transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-3e: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, UE may transmit S-SSB repetition in the contiguous multiple RB sets. RBstart: All the possible cases LCRB: 11 RB |

6.1.2.1	MPR for SL-U operation in single CC

<Editor Note> Detail MPR requirements will be added

6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

6.1.3.1	A-MPR for SL-U with NS_28

6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.1.2	A-MPR for S-SSB transmission

6.1.3.1.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.2	A-MPR for SL-U with NS_29

6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.2.2	A-MPR for S-SSB transmission

6.1.3.2.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.3	A-MPR for SL-U with NS_30

6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.3.2	A-MPR for S-SSB transmission

6.1.3.3.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.4	A-MPR for SL-U with NS_31

6.1.3.4.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.4.2	A-MPR for S-SSB transmission

6.1.3.4.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.5	A-MPR for SL-U with NS_53

6.1.3.5.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.5.2	A-MPR for S-SSB transmission

6.1.3.5.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.6	A-MPR for SL-U with NS_54

6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.6.2	A-MPR for S-SSB transmission

6.1.3.6.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.7	A-MPR for SL-U with NS_58

6.1.3.7.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.7.2	A-MPR for S-SSB transmission

6.1.3.7.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.8	A-MPR for SL-U with NS_59

6.1.3.8.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.8.2	A-MPR for S-SSB transmission

6.1.3.8.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.9	A-MPR for SL-U with NS_60

6.1.3.9.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.9.2	A-MPR for S-SSB transmission

6.1.3.9.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.3.10	A-MPR for SL-U with NS_61

6.1.3.10.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.10.2	A-MPR for S-SSB transmission

6.1.3.10.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

6.1.4	Configured transmitted power for NR SL-U

<<<<<<<<<<< End of changes in section 6 >>>>>>>>>>

References

- R4-2306633 “WF on MPR simulation assumptions for SL-U in single CC,” Meta Ireland

- R4-2306631 “WF on SLU single CC Tx requirements,” Xiaomi

- R4-2306632 “WF on SLU single CC Rx requirements,” OPPO

- RP-230077   “Revised WID on NR sidelink evolution,” OPPO