3GPP TSG-RAN WG4 Meeting #107	R4-2310410

Incheon, South Korea, May 22 – May 26, 2023

Source:	Skyworks Solutions, Inc.

Title:	Triple Beat MSD for DC_3A-8B_n78A

Agenda item:			7.1.1.1 band combinations with UL configurations including intra-band ULCA with IMD or triple beat issues

Document for:	Approval

# 1. Introduction

This document is the revision of R4-2309741 in which the Band 3 MSD analysis due to triple-beat (TB) interference for DC_3A-8B_n78A is analysed.

# 2. Discussion

## 2.1 MSD test point

At RAN4 #106bis-e, interested companies were invited to evaluate the B3 TB MSD test point of Table 1 for uplink (UL) DC_8B_n78A.

Table 1: Band 3 TB MSD test point proposal for DC_3A-8B_n78A

| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8B_n78A | 3 | N/A | 5 | N/A | 1845 | [FFS] | FDD | IMD3 |
|  | 8 | 900 | 10 | 1 (RBSTART=20) | 945 | N/A | FDD | N/A |
|  |  | 910 | 10 | 1 (RBSTART=45) | 955 | N/A | FDD | N/A |
|  | n78 | 3660 | 10 | 25 (RBSTART=0) | 3660 | N/A | TDD | N/A |

The corresponding IMD landscape is shown in Figure 1, with the following notations:

- f1: centre frequency of band n78 UL resource block (RB) allocation,

- f2: centre frequency of band 8 PCC UL RB allocation,

- f3: centre frequency of band 8 SCC UL RB allocation.

According to WF [1], B3 MSD analysis is required due to triple beat product “3” (TB3) expected at 1845MHz and adjacent band groups between downlink (DL) band 3, band 8 and band n78.

Figure 1: IMD landscape for Table 1 DC_3A-8B_n78A MSD test point

Key observations:

Observation 1: For the B8 UL RB configurations of  Table 1,

- The band 8 DL-CA carrier is affected by dual UL IMD5 and IMD7 interference.

- The band 3 downlink 5MHz CBW carrier at 1845MHz is affected only by the TB IMD. It is assumed that the n78 receiver is able to reject the two adjacent IMD3 products.

Observation 2: For near-contiguous UL RB allocations in CA_8B:

- There is a possibility that DL B3 is affected by the simultaneous overlap of three IMD3 products:

- IMD3 [2f2-f1]

- IMD3 [2f3-f1]

- IMD3 [f1-f2-f3] – 1st order “TB3”,

- The B3 MSD may exceed the dual-UL IMD MSD.

- The DL B8 carrier becomes free of dual UL IMD interference.

## 2.2 MSD analysis

This document assumes the diplexer reference architecture of Figure 2 and the RF front-end parameters of Table 2.

Figure 2: DC_3-8_n78 reference architecture and band 3 LNA centric coupling paths (BPF: bandpass filter, LPF: lowpass filter, HTF: harmonic trap filter).

Table 2: Front-end component isolation for MSD calculation.

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to LNA (in) | 60 | PCB isolation (LNA forward mixing) |
| Diplexer | 20 | High/Mid band isolation |
| Diplexer | 15 | Low/Mid band isolation |
| Band 3 Rx filter rejection at n78 TX and B8 Tx | 45¦40 | Main ¦ Diversity |

The triple-beat contributions from the UL-CA B8 PA and the band n78 PA are ignored for the following reasons:

- It has been shown in previous contributions that TB interference is dominated by the UL-CA PA since the TB product increases 2:1 with the UL-CA output power, while the increase is 1:1 for the band n78 PA,

- For this combination, it may be assumed that the B8 PA TB interference is rejected by the harmonic trap filter (HFT).

The LNA TB contributions are evaluated for the following coupling paths:

- B3 primary LNA:

  - conducted path [1]: from B8 PA output via L/M-H diplexers + B3 duplexer;

  - conducted path [2]: from n78 PA output via H/M diplexer + B3 duplexer;

  - path [4]: B8 PA output via limited PCB isolation;

  - path [5] n78 PA output via limited PCB isolation;

- B3 diversity LNA:

  - conducted path [3]: via antenna-to-antenna isolation, diplexer and B3 bandpass filter (BPF)

  - path [6]: from B8 PA output via limited PCB isolation;

  - path [7]: from n78 PA via limited PCB isolation;

Figure 3 summarises the antenna referred LNA TB interference levels for 5MHz channel bandwidth (CBW) at 1845MHz and the evaluated MSD.

Figure 3: LNA triple beat IMD interference levels referred to the primary and diversity antenna ports and corresponding MSD for Band 3 5MHz CBW.

To prevent band 8 self-desense, it is proposed to change the uplink CA_8B RB allocation to ensure the dual uplink IMD order affecting the downlink band is greater than order 9.

Proposal: For DC_3A-8B_n78A, adopt the MSD test point of Table 3.

Table 3: Band 3 TB MSD test point for DC_3A-8B_n78A

| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8B_n78A | 3 | N/A | 5 | N/A | 1845 | 4.1 | FDD | IMD3 |
|  | 8 | 900 | 10 | 1 (RBSTART=35) | 945 | N/A | FDD | N/A |
|  |  | 910 | 10 | 1 (RBSTART=30) | 955 | N/A | FDD | N/A |
|  | n78 | 3660 | 10 | 25 (RBSTART=0) | 3660 | N/A | TDD | N/A |

# 3. Conclusion

This paper presents the TB MSD analysis for DC_3A-8B_n78A. We make the following observations and propose 4.1dB MSD driven solely by the triple beat contributions of primary and diversity LNAs.

Observation 1: For the B8 UL RB configurations of  Table 1,

- The band 8 DL-CA carrier is affected by dual UL IMD5 and IMD7 interference.

- The band 3 downlink 5MHz CBW carrier at 1845MHz is affected only by the TB IMD. It is assumed that the n78 receiver is able to reject the two adjacent IMD3 products.

Observation 2: For near-contiguous UL RB allocations in CA_8B:

- There is a possibility that DL B3 is affected by the simultaneous overlap of three IMD3 products:

- IMD3 [2f2-f1]

- IMD3 [2f3-f1]

- IMD3 [f1-f2-f3] – 1st order “TB3”,

- The B3 MSD may exceed the dual-UL IMD MSD.

- The DL B8 carrier becomes free of dual UL IMD interference.

Proposal: For DC_3A-8B_n78A, adopt the MSD test point of Table 3.

Table 3: Band 3 TB MSD test point for DC_3A-8B_n78A

| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8B_n78A | 3 | N/A | 5 | N/A | 1845 | 4.1 | FDD | IMD3 |
|  | 8 | 900 | 10 | 1 (RBSTART=35) | 945 | N/A | FDD | N/A |
|  |  | 910 | 10 | 1 (RBSTART=30) | 955 | N/A | FDD | N/A |
|  | n78 | 3660 | 10 | 25 (RBSTART=0) | 3660 | N/A | TDD | N/A |

# References

[1]	R4-2220556 WF on triple beat rules and MSD for inter-band with 2UL with intra-band ULCA, 3GPP TSG-RAN WG4 Meeting # 105 , Toulouse, France, Murata Manufacturing Co Ltd., Skyworks Solutions Inc.