3GPP TSG-RAN WG4 Meeting #108	R4-2311447

Toulous, France, 21– 25 Aug., 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on BS RF requirement of network energy savings

Agenda Item:	8.34.3

Document for:	Approval

Introduction

There was extensive discussion on the TAE requirement of SSB-less operation for FR1 co-located inter-band CA of the R18 WI Network energy savings for NR during the last two meetings. The following four options were captured in the agreed WF [1]:

Issue 1-1: TAE

- Further discuss the following values for TAE to guarantee the SSB-less feature performance

  - Option 1: 2.x µs, i.e., CP size for 30kHz SCS

  - Option 2: 260ns

  - Option 3: 65ns

  - Option 4: 3µs

- In the next meeting, encourage companies to provide the achievable values.

This paper we further discuss these options one by one and give our proposals.

We also provide our opinions on Cell DTX/DRX, spatial and power domains techniques, based on the progress that RAN1 has made on these topics so far.

Discussion

Achievable TAE

There are three main scenarios for inter-band CA for FR1 and co-located cells, as shown in the Figure 1:

- Figure 1 Deployment scenarios for inter-band CA for FR1 and co-located cells

Inter-band CA in Case A is implemented with a common baseband and common RF part. This can be seen in a BS that supports multi-band operations.

In Case B, Inter-band CA is implemented using a common baseband but different RF parts. Typically, each RF part operates within a single band.

In Case C, Inter-band CA is implemented using different basebands and RF parts for each band. Generally, each RF part operates within a single band.

The baseband, CPRI/eCPRI interfaces, and RUs throughout the entire link contribute to the TAE. Digital component delay in the chain can be calculated and compensated precisely. TAE of baseband is less than 20ns, TAE of baseband to RF interface is about 10ns, the TAE value of RU mainly depends on the analog filter. Generally speaking, as the deployment complexity increases from case A to case B to case C, the uncertainty of latency becomes greater and greater.

The TAE in case A can be controlled in the same way as intra-band contiguous CA, ensuring it does not exceed 260ns.

For case B, compared to case A, an additional TAE can occur when different interface from baseband to RF parts. The TAEs in star topology, chain topology and ring topology in the BB and RU connection modes are slightly different.

For case C, the CA component carriers are from different basebands and independent RUs. Compared to case B, the baseband clock may vary. Apart from that, the time delay between different RUs is not easily correctable, which can cause additional misalignment between the CCs, so a larger TAE could be expected.

Observation 1: The simpler the deployment, the smaller the latency uncertainty. BS that support multi-band operation may provide the simplest solution for achieving low TAE inter-band CA.

For FR1 NR BS, TAE defined in the TS 38.104 is following:

For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

While for E-UTRA BS, TAE defined in the TS 36.104 is following:

For inter-band carrier aggregation, with or without MIMO or TX diversity, TAE shall not exceed 260ns.

During the NR specification discussion, the TAE for inter-band CA has been relaxed to 3µs. It is because that the synchronization performance may degrade in certain extreme non-co-located scenarios. For example, when the different component carriers use different clock source and both clock sources are faulty, it may experience poor clock synchronization performance.

However, in FR1 co-located scenarios, the TAE is much better than 3µs in practical deployment scenario.

Observation 2: For FR1 NR BS, TAE 3µs for inter-band carrier aggregation is considering extreme non-co-located scenarios. However, in FR1 co-located scenarios, the TAE is much better than 3µs in practical deployment scenario.

As we have proposed at previous meetings, it is feasible to make TAE within 260ns for most scenarios for FR1 co-located inter-band CA.

Many RUs in the network that support NR also support LTE, which makes it natural for them to support the 260 ns inter-band CA TAE.

Currently, BSs supporting multi-band operation are widely available in the market, including n28+n20 (700M+800M), n28+n5/n26 (700M+850M), n28+n8 (700M+900M), n5/26+n8 (850M+900M), n3+n1(1.8G+2.1G), n2+n1(1.9G+2.1G ) , n1+n7 (2.1G+2.6G) and so on. Therefore, the TAE could be controlled in the same way as intra-band contiguous carrier aggregation, ensuring it does not exceed 260ns.

In the case of inter-band CA implemented with RRU support the multi-band operation, the TAE could be controlled within 260 ns.

Proposal 1: For option 2 (260ns), MSR BS which support NR and LTE, and BS supporting multi-band operation, FR1 co-located inter-band CA TAE can be within 260ns.

Some companies prefer to maintain the current 3µs TAE for NR inter-band CA to ensure the NES features are compatible with existing BS and UE RF requirements. 
If the RTD (RTD = TAE of BS + UE receiving time difference) of the two carriers for inter-band CA is greater than CP, the UE cannot correctly demodulate the original signals without performing fine synchronization again.

If SSB-less operation for energy saving FR1 inter-band co-located CA can work, RTD should be less than CP corresponding to the largest SCS among the CCs. For FR1, it is 1.71 µs.

As we know, RTD include TAE and air propagation difference. It can be assumed that in co-located scenario, air propagation difference can be omitted. So the TAE should be less than CP.

Proposal 2: TAE value for SSB-less operation for FR1 co-located inter-band CA should be less than CP corresponding to the largest SCS among the CCs. For FR1 SCS 60 kHz, CP is 1.71 µs.

Considering that it is possible to adopt 15 /30 /60 kHz SCS for FR1, the smallest CP is 1.71 µs.

Table 1 CP for different SCS in FR1

| SCS | Symbol Duration | CP for Long Symbols | CP for other Symbols |
| --- | --- | --- | --- |
| 15kHz | 66.67 µs | 5.2 µs | 4.69 µs |
| 30kHz | 33.33 µs | 2.86 µs | 2.34 µs |
| 60kHz | 16.67 µs | 1.69 µs | 1.17 µs |

Observation 3: 3µs is larger than CP for 30/60kHz FR1 SCS.

Proposal 3: Option 4 (3µs）is not feasibility for SSB-less operation, since it is larger than CP for 30/60kHz SCS.

Observation 4: Option 1 (2.x µs, i.e., CP size for 30 kHz SCS) is larger than CP for 60 kHz FR1 SCS.

Some company think the achievable FR1 inter-band CA TAE requirement in co-located scenario could be optimized down to at least 65 ns. We don't want to deny the possibility, but it's important to note that it may bring unnecessary burden during the design and produce, as well as the testing process. For example, high-precision instruments may be necessary to compensate for line loss and conduct testing during mass production.

Proposal 4: Option 3(65ns) may bring unnecessary burden during the design and mass production, as well as the testing process. For example, high-precision instruments may be necessary to compensate for line loss and conduct testing during mass production.

How to handle the requirements of SSB-less operation

In RAN4#106bis, A WF was agreed as following [2]:

Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation

- Agreement:

  - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS.

SSB-less operation for network energy saving is an optional feature and can be declared by operators. As the WF had agreed, it would have no impact to the legacy RF requirements of inter-band CA TAE defined in TS38.104.

Proposal 5: SSB-less operation for FR1 co-located inter-band CA is an optional feature and can be declared.

There are two ways to handle SSB-less operation for FR1 inter-band co-located CA TAE requirements:

Option 1: Instead of defining feature-specific TAE requirement in the RF session, it can be defined in the RRM session, similar to how SSB-less operation for intra-band contiguous CA has been handled.

Option 2: Feature-specific TAE requirements can be defined directly in the RF session in clause 6.5.3.2 of TS38.104as follows:

For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

For FR1 co-located inter-band carrier aggregation supporting SSB-less operation (NW enabling energy savings), with or without MIMO, TAE shall not exceed [ ].

The time alignment error requirements for NB-IoT are specified in TS 36.104 [13] clause 6.5.3.

Proposal 6: There are two ways to handle SSB-less operation for FR1 inter-band co-located CA TAE requirements:

Option 1: Instead of defining feature-specific TAE requirement in the RF session, it can be implicitly defined in the RRM session, similar to how SSB-less operation for intra-band contiguous CA has been handled.

Option 2: Feature-specific TAE requirements can be defined directly in the RF session in clause 6.5.3.2 of TS38.104.

In our opinion, both options are acceptable and will not have any impact on legacy equipment.

RF requirements for Cell DTX/DRX

The Cell DTX (discontinuous transmission) network saving technology involves switching the transmission between turn on and turn off for both the BS and the UE. This is similar to TDD, but without the switch between transmission and reception.

DTX is usually implemented through various shutdowns, such as shutdown of a carrier, a PA, a TRX channel, or others. These requirements for DTX are related to the switch response time of the PA, TX channel. However, this is not a significant issue as it occurs within the transmitter period or frequency band and does not interfere with others.

We checked RAN1 progress on Cell DTX/DRX, and found that in the Reply LS, RAN1 has made some agreements [3]:

- Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective.

- RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback.

  - subject to UE capability

  - RAN1 asks RAN2 to consider the additional support of a MAC CE based indication.

So far we do not see any RF impact and hold the opinion that there is no need to define DTX/DRX relative requirements at this stage.

Proposal 7: For Cell DTX/DRX, there is no need to define switch time and spectrum as it operates within its own transmitter period or frequency band and does not cause interference with other signals.

RF requirements for spatial and power domains techniques

Regarding spatial and power domains techniques, it is possible to selectively activate or deactivate individual or multiple antenna elements linked to a logical antenna port with flexibility.

In terms of spatial and power domains techniques, it is possible to selectively enable or disable individual or multiple antenna elements that are linked to a logical antenna port. This functionality is akin to Cell DTX/DRX to some extent, it is only occurred during the transmitter period or frequency band without causing interference to other signals.

In the FL summary RAN1 had reached some agreement about spatial and power domains techniques [4]:

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

Discuss the signaling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

- Whether there is a need for transition time per adaptation (for UE)

Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

So far we do not see any RF impact and hold the opinion that there is no need to define spatial and power domains techniques relative requirements at this stage.

Proposal 8: For Spatial and power domain techniques, there is no need to define new requirement within its own transmitter period or frequency band and does not cause interference with other signals.

Conclusions

In this contribution, we discuss the TAE requirement for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells. Besides that, we give two optional solutions on how to handle the requirements of SSB-less operation.

We have also checked the progress of RAN1 on Cell DTX/DRX, spatial and power domains techniques. Our observations and proposals are as follows:

Observation 1: The simpler the deployment, the smaller the latency uncertainty. BS that support multi-band operation may provide the simplest solution for achieving low TAE inter-band CA.

Observation 2: For FR1 NR BS, TAE 3µs for inter-band carrier aggregation is considering extreme non-co-located scenarios. However, in FR1 co-located scenarios, the TAE is much better than 3µs in practical deployment scenario.

Proposal 1: For option 2 (260ns), MSR BS which support NR and LTE, and BS supporting multi-band operation, FR1 co-located inter-band CA TAE can be within 260ns.

Proposal 2: TAE value for SSB-less operation for FR1 co-located inter-band CA should be less than CP corresponding to the largest SCS among the CCs. For FR1 SCS 60 kHz, CP is 1.71 µs.

Observation 3: 3µs is larger than CP for 30/60kHz FR1 SCS.

Proposal 3: Option 4 (3µs）is not feasibility for SSB-less operation, since it is larger than CP for 30/60kHz SCS.

Observation 4: Option 1 (2.x µs, i.e., CP size for 30 kHz SCS) is larger than CP for 60 kHz FR1 SCS.

Proposal 4: Option 3(65ns) may bring unnecessary burden during the design and produce, as well as the testing process. For example, high-precision instruments may be necessary to compensate for line loss and conduct testing during mass production.

Proposal 5: SSB-less operation for FR1 co-located inter-band CA is an optional feature and can be declared.

Proposal 6: There are two ways to handle SSB-less operation for FR1 inter-band co-located CA TAE requirements:

- Option 1: Instead of defining feature-specific TAE requirement in the RF session, it can be implicitly defined in the RRM session, similar to how SSB-less operation for intra-band contiguous CA has been handled.

- Option 2: Feature-specific TAE requirements can be defined directly in the RF session in clause 6.5.3.2 of TS38.104.

Proposal 7: For Cell DTX/DRX, there is no need to define switch time and spectrum as it operates within its own transmitter period or frequency band and does not cause interference with other signals.

Proposal 8: For Spatial adaptation domain techniques, there is no need to define new requirement within its own transmitter period or frequency band and does not cause interference with other signals.

Reference

[1] R4-2310381, WF on RF feasibility study of Network Energy Savings for NR, Huawei, RAN4#107, May 22-26, 2023

[2] R4-2306639, WF on RF feasibility study of Network Energy Savings for NR, Huawei, RAN4#106bis, April 2023

[3] R1-2306246, Reply LS on Cell DTX/DRX activation/deactivation, RAN1#113, May 2023

[4] R1-2306030, FL summary#1 for SD and PD adaptation for R18 NES, Moderator (Huawei), May 2023