3GPP TSG-RAN WG4 Meeting #108 	R4-2311738

Toulouse, France, August 21-25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Advanced Receivers Blind Detection - Simulation results

Agenda item:	8.18.1.1

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenarios with blind detection of interference parameters.

Simulation assumptions

Table 1 provides an overview of the already agreed simulation assumptions taken from the WF [1]:

Table 1: Agreed simulation assumptions

| Reference receiver for BD simulation | R-ML |
| --- | --- |
| Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE | 2Rx UE: 1+1 4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for rank 1+1 |
| Antenna configuration | For rank 1+1: cover 2T2R For rank 2+2: 4T4R |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium Rank 2+2: ULA Low |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 Random PMI selection for the target UE Cover both orthogonal and random PMI selection (same as Rel-17 approach) for the co-scheduled UEs in phase I |
| Transmission PDSCH power ratio of co-scheduled UEs | 0 dB : Precoder scaling factor sqrt(1/2) |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | 2 co-scheduled UEs |
| DMRS sequence for the co-scheduled UE | For BD simulations, assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | MCS 13 for rank 1 and rank 2 |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth for target UE. Full OFDM symbol allocation |
| ZP-CSI-RS configuration | Single port, density 0.5, l_0=3, k_0=0, full CHBW, Aperiodic. Triggered using DCI in every MU PDSCH slot |

Based on the information in Table 1, a blind detection study of interference is pursued with 2 co-scheduled UE for the following parameters:

- Presence of co-scheduled UEs (limited to 2 interference UEs).

- DMRS port and sequence detection (DMRS det): DMRS ports 1000 to 1003.

- FDRA detection (FDRA det) of each co-scheduled UE: full and partial CHBW.

- Modulation order detection (MO det) for each co-scheduled UE. Only required for R-ML.

We provide the simulation results for several relevant cases, summarized in Table 2.

Table 2 - Simulation Overview

| Case | # Co-scheduled UE | Rank target UE | Rank Co-scheduled UE | MCS target UE | Modulation order co-scheduled UE | MIMO | Channel model | FDRA allocation of Co-scheduled UE | Precoder selection for the Co-scheduled UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 2 | 1 | 1 for each Co-UE | MCS13 | Co-UE1: QPSK Co-UE2: 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 | Co-UE1: 0~25 PRBs Co-UE2: 26~51 PRBs | random |
| [17] | 2 | 2 | Co-UE1: 1 on port 1002 Co-UE2: 1 on port 1003 | MCS13 | Co-UE1: QPSK Co-UE2: 16QAM | 4Tx4Rx ULA Low | TDLA30-10 | Full CHBW (52 PRBs) | random |
| Note1: All tests consider the use of MCS index Table 1. Note2: All cases consider blind detection of the co-scheduled UEs FDRA, DMRS port allocation as well as modulation order. |  |  |  |  |  |  |  |  |  |

Blind Detection Simulation Results

This section displays our current simulation results. Normalized throughput curves are presented below along with a black reference line to highlight the SNR @ 70% of maximum throughput for each receiver. Results for the following are summarised in Table 3.

- genie R-ML receiver with full knowledge of all the co-UEs information,

- R-ML with blind detection of co-UEs DMRS ports, FDRA and modulation order

- R-ML with aided blind detection. Aided blind detection relies on ZP-CSI-RS being scheduled in MU-MIMO slot for blind detection of modulation order.

Table 3 – Summary of receiver performance with blind parameter detection (Cases 16, Case [17]).

| Case | # Co-Sch UE* | Rank target UE | Rank Co-Sch UE | MCS Target UE | MO* Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-scheduled UE | Precoder Co-Sch UE | Nokia |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | SNR@70%maxTP (dB) |  |  | Gap Genie to R-ML BD | Gap Genie to R-ML Aided BD |
|  |  |  |  |  |  |  |  |  |  | Genie R-ML | R-ML BD | R-ML Aided BD3 |  |  |
| 16 | 2 | 1 | 1 for each co-UE | MCS13 | Co-UE1: QPSK Co-UE2: 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 | Co-UE1: 0~25 PRBs Co-UE2: 26~51 PRBs | random | TBA | TBA | TBA | TBA | TBA |
| [17] | 2 | 2 | Co-UE1: 1 on port 1002 Co-UE2: 1 on port 1003 | MCS13 | Co-UE1: QPSK Co-UE2: 16QAM | 4Tx4Rx ULA Low | TDLA30-10 | Full CHBW (52 PRBs) | random | 12.6 | 17.2 | 12.8 | 4.6 | 0.2 |
| Note1: All tests consider the use of MCS index Table 1. Note2: All cases consider blind detection of the co-scheduled UEs FDRA, DMRS port allocation as well as modulation order. Note3: Aided BD R-ML relies on ZP-CSI-RS being scheduled in MU-MIMO slot for blind detection of modulation order. Note*: Co-scheduled UE (Co-Sch UE) and Modulation order (MO). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Results for the case [17] are presented in this document, while the results for case 16 will be presented during the RAN4 #108 meeting. We expect similar trends for case 16 as in case [17].

Some observations follow below:

R-ML blind detection has a significant performance gap to genie R-ML and cannot achieve 90% of maximum throughput with 2 co-scheduled UEs.

R-ML with aided blind detection can achieve 90% of maximum throughput and has marginal performance gap to genie receiver with 2 co-scheduled UEs.

Case [17]:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [17] | 2 | 2 | Co-UE1: 1 on port 1002 Co-UE2: 1 on port 1003 | MCS13 | Co-UE1: QPSK Co-UE2: 16QAM | 4Tx4Rx ULA Low | TDLA30-10 | random |

Figure 1 – Case [17] 2 co-UEs: 4T4R Rank 2+1+1 TDLA30-10, low correlation, MCS13-QPSK-16QAM, random precoder. R-ML BD, aided BD are blind detection of co-UEs FDRA, DMRS ports and modulation order.

Conclusion

This paper presents Nokia's simulation results for the study on blind detection for detecting interference parameters.

In the paper, the following Observations were made:

- R-ML blind detection has a significant performance gap to genie R-ML and cannot achieve 90% of maximum throughput with 2 co-scheduled UEs.

R-ML with aided blind detection can achieve 90% of maximum throughput and has marginal performance gap to genie receiver with 2 co-scheduled UEs.

- References

- R4-2309892 WF on advanced receiver for MU-MIMO scenario