# 3GPP TSG-RAN WG4 Meeting # 108                            R4-2311831

# Toulouse, France, August 21 – August 25, 2023

# Source:        ZTE Corporation

# Title:           Discussion on demodulation for FR2-1 UL 256QAM

# Agenda Item:   8.6.4.1

# Document for:  Approval

- Introduction

In RAN 97 meeting, a revised WID [1] for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved, the objective for demodulation is:

UL 256QAM

  - Specify the BS demodulation performance

In this contribution, we give some discussions on simulation assumption and initial simulation results for reference.

- Discussion

## 2.1 Simulation assumption for FR2-1 UL 256QAM

With regard to waveform for demodulation requirements, CP-OFDM can be considered for UL 256QAM as DFT-s-OFDM waveform only investigates the demodulation performance of QPSK modulation with lower MCS which can be checked in TS 38.104[2].

With regard to MCS for demodulation requirements, TS 38.104 defines UL 256qam requirement for FR1 with a code rate of 682.5/1024 which corresponds to MCS20 in Table 5.1.3.1-2 in TS 38.214[3].

With regard to transmission layer and antenna configuration for demodulation requirements, 1Tx antenna configuration with 1 layer transmission is to be considered for FR2-1 UL 256qam. For Rx antenna configuration, 2Rx can be considered and other Rx antenna configuration such as 4Rx and 8Rx needs further discussion.

With regard to mapping type for PUSCH, it can be found that only the requirements of type B mapping are defined for FR2, therefore it is preferred to only consider type B mapping with 10 OFDM symbols scheduled for UL 256QAM demodulation requirements.

With regard to channel model, TS 38.104 defines UL 64qam requirement with TDLA30-75 Low for FR2 in Table 11.2.2.1.2-1[3]. TDL-A with 35ns Doppler shift can be considered for FR2 UL 256qam performance.

With regard to DMRS configuration, both 1 and 1+1 DMRS can be considered just since FR2 64qam considers both.

With regard to phase noise, the new phase noise models agreed for MPR evaluation can be considered[4].

With regard to SCS, 60kHz, 120kHz can be considered and 120kHz is preferred as 120kHz SCS is used for evaluation in RF session.

With regard to BW, 50MHz, 100MHz and 200MHz can be considered for performance evaluation as defined by FR2 64qam for these bandwidths.

Based on the above analysis, the parameters listed in table 2-1 can be considered for FR2-1 UL 256QAM demodulation requirements.

Table 2-1 Simulation assumption for PUSCH demodulation for 256QAM

| Parameter | FR2-1 |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1, 1+1 |
| symbols length | 10 |
| start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 20 in table 5.1.3.1-2 in TS 38.214[3] (R=682.5/1024) |
| Carrier frequency (GHz) | 30 |
| Propagation condition | TDL-A 30ns delay spread, 35Hz Doppler frequency |
| SCS and BW | 60kHz: 100MHz 120kHz: 100MHz |
| PTRS | Disabled,KPT-RS =2, LPT-RS =1 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Number of HARQ transmissions and RV sequence | 4 , {0,2,3,1} |
| PN model for Tx and Rx | Tx：New phase noise as agreed for MPR simulation Rx: example 2 for BS in TR 38.803 |
| Testing metric | SNR @70% of maximum throughput |

Proposal 1: To consider the parameters in table 2-1 for FR2-1 UL 256QAM demodulation requirements.

Table 2-1 Simulation assumption for PUSCH demodulation for 256QAM

| Parameter | FR2-1 |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1, 1+1 |
| symbols length | 10 |
| start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 20 in table 5.1.3.1-2 in TS 38.214[3] (R=682.5/1024) |
| Carrier frequency (GHz) | 30 |
| Propagation condition | TDL-A 30ns delay spread, 35Hz Doppler frequency |
| SCS and BW | 60kHz: 100MHz 120kHz: 100MHz |
| PTRS | Disabled,KPT-RS =2, LPT-RS =1 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Number of HARQ transmissions and RV sequence | 4 , {0,2,3,1} |
| PN model for Tx and Rx | Tx：New phase noise as agreed for MPR simulation Rx: example 2 for BS in TR 38.803 |
| Testing metric | SNR @70% of maximum throughput |

## 2.2 Simulation results

## The following table shows the operating SNR based on the simulation assumption listed in table 2-1 and the throughput curves are shown in table A-1.

Table 2-2 Simulation results of PUSCH demodulation for 256QAM

| Mapping Type | Channel | SCS [kHz] /CBW [MHz] | DMRS Config | PTRS | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| Type B | TDLA30-35Low | 120/100 | DMRS 1+1 | disabled | 18.5 |
|  |  |  |  | enabled | 19.6 |
|  |  |  | DMRS 1 | disabled | 18.6 |
|  |  |  |  | enabled | 21.0 |
|  |  | 60/100 | DMRS 1+1 | disabled | 21.1 |
|  |  |  |  | enabled | 22.0 |
|  |  |  | DMRS 1 | disabled | 16.7 |
|  |  |  |  | enabled | 18.5 |

- Conclusion

Based on the discussion, the following proposal is given:

Proposal 1: To consider the parameters in table 2-1 for FR2-1 UL 256QAM demodulation requirements.

Table 2-1 Simulation assumption for PUSCH demodulation for 256QAM

| Parameter | FR2-1 |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1, 1+1 |
| symbols length | 10 |
| start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 20 in table 5.1.3.1-2 in TS 38.214[3] (R=682.5/1024) |
| Carrier frequency (GHz) | 30 |
| Propagation condition | TDL-A 30ns delay spread, 35Hz Doppler frequency |
| SCS and BW | 60kHz: 100MHz 120kHz: 100MHz |
| PTRS | Disabled,KPT-RS =2, LPT-RS =1 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Number of HARQ transmissions and RV sequence | 4 , {0,2,3,1} |
| PN model for Tx and Rx | Tx：New phase noise as agreed for MPR simulation Rx: example 2 for BS in TR 38.803 |
| Testing metric | SNR @70% of maximum throughput |

- Reference

RP-222228 Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia, Xiaomi

3GPP TS 38.104 V18.2.0

3GPP TS 38.214 V17.6.0

R4-2310260 WF on UL 256QAM requirements, Xiaomi

- Annex

Table A-1 BS PUSCH demodulation for FR2-1 UL 256QAM