3GPP TSG-RAN WG4 # 108	R4-2311999

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.18.1.2

Source:	MediaTek Inc.

Title:	Simulation results of Advanced receiver to cancel inter-user interference for MU-MIMO

Document for:	Discussion

# Introduction

In RAN#99, the status report of Rel-18 WI of NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined [1].  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. This work is split into two phases where the first phase studies the performance gain, reference receiver assumption, interference modelling, testability, required signalling overhead, as well as impact on other WGs. The initial receiver candidates are E-MMSE-IRC and R-ML. In the second phase it is expected to specify PDSCH demodulation requirements under MU-MIMO scenario with advanced receiver.

# Simulation parameters and results

In this chapter we are going to discuss simulation parameters in Chapter 2.1, simulation results in Chapter 2.2 and simulation analysis in Chapter 2.3.

# 2.1 Simulation parameters

In this chapter we are going to discuss simulation assumptions from WF [2] and show simulation results with short analysis.

The number of co-scheduled UEs

  - In the phase I study for the advanced receiver with genie aided knowledge of the required information

    - Only consider 1 co-scheduled UE

MCS for the target UE

  - MCS 13 for rank 1 and rank 2

  - MCS 17 for rank 2

  - MCS 4 for rank 1

Modulation order for the co-scheduled UE

  - For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose)

    - For rank 1+1: QPSK

    - For rank 2+2: 64QAM

    - For rank 2+2: QPSK

    - For rank 2+2: 16QAM

PDSCH resource allocation for the target and co-scheduled UE

  - In the phase I study for the advanced receiver with genie aided knowledge of the required information

    - Only consider the scenario with full FDRA for both target and co-scheduled UEs

Phase I Simulation result collection

  - Companies are encouraged to bring phase I simulation results on the agreed cases to the Aug RAN4 meeting

  - The updated simulation result collection template will be shared before the Aug RAN4 meeting

We have provided simulation results of 2Rx UE in MCS4 1+1 and MCS13 1+1 scenarios and 4Rx UE in MCS13 2+2 and MCS17 2+2 scenarios. Also, we are assuming the same scrambling ID for DMRS sequence for all UEs. For Modulation order of the co-scheduled UE, we have simulated all options of QPSK, 16-QAM and 64-QAM to give more complete summary of results. We have simulated only single co-scheduled UE scenarios for this meeting.

For Rank 1+1 we have simulated TDLC300-100 with low and medium correlation and for Rank 2+2 we have simulated TDLA30-10 and TDLC300-100 with low correlation. We have simulated both random and orthogonal precoding options for all scenarios to give more complete summary of results. We have assumed full bandwidth allocation of all UEs.

Furthermore, we have assumed the same QCL assumption in all DMRS ports. We have assumed all needed co-scheduled UE information as known. The evaluation metric is SNR at 70% of maximum throughput.

In this document we compare performance of 3 receiver types as agreed in WF [2], namely

- MMSE-IRC (Rel-17 reference receiver)

- E-MMSE-IRC (Rel-18 advanced receiver)

- R-ML (Rel-18 advanced receiver)

# 2.2 Simulation results

Simulation configuration of target UE rank 1 using MCS4 with co-scheduled UE rank 1 simulations in 2RX are shown in Table 2-1. The corresponding simulation results with random orthogonal precoding are shown in Table 2-2 and Figure 2-1. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-3 and Figure 2-2.

Simulation configuration of target UE rank 1 using MCS13 with co-scheduled UE rank 1 simulations in 2RX are shown in Table 2-4. The corresponding simulation results with random orthogonal precoding are shown in Table 2-5 and Figure 2-3. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-6 and Figure 2-4.

Simulation configuration of target UE rank 2 using MCS13 with co-scheduled UE rank 2 simulations in 4RX are shown in Table 2-7. The corresponding simulation results with random orthogonal precoding are shown in Table 2-8 and Figure 2-5. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-9 and Figure 2-6.

Simulation configuration of target UE rank 2 using MCS17 with co-scheduled UE rank 2 simulations in 4RX are shown in Table 2-10. The corresponding simulation results with random orthogonal precoding are shown in Table 2-11 and Figure 2-7. Furthermore, simulation results with random non-orthogonal precoding are shown in Table 2-12 and Figure 2-8.

Table 2-1: Test configurations for MU-MIMO with Rank1 target UE using MCS4 and Rank1 co-scheduled UE in 2RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank1 CoUE Rank1 | FDD: 10/15 | QPSK, 0.30 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLC300-100 | 2 x 2 ULA Low or ULA Medium | 70 | Test dependent |

Table 2-2: Simulation results for MU-MIMO with Rank1 target UE using MCS4 and Rank1 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLC-Low | 1.8dB | 1.9dB | 2.3dB | 0.5dB | 0.4dB |
| coUE:16-QAM TDLC-Low | 1.9dB | 1.9dB | 2.3dB | 0.4dB | 0.4dB |
| coUE:64-QAM TDLC-Low | 2.0dB | 1.9dB | 2.3dB | 0.3dB | 0.4dB |
| coUE:QPSK TDLC-Medium | 4.0dB | 4.5dB | 5.2dB | 1.2dB | 0.7dB |
| coUE:16-QAM TDLC-Medium | 4.5dB | 4.5dB | 5.2dB | 0.7dB | 0.7dB |
| coUE:64-QAM TDLC-Medium | 4.7dB | 4.5dB | 5.2dB | 0.5dB | 0.7dB |

Figure 2-1: Simulation results for MU-MIMO with Rank1 target UE using MCS4 and Rank1 co-scheduled UE with orthogonal precoding.

| TDLC300-100 Low correlation | TDLC300-100 Medium correlation |
| --- | --- |

Table 2-3: Simulation results for MU-MIMO with Rank1 target UE using MCS4 and Rank1 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLC-Low | 2.5dB | 2.6dB | 3.2dB | 0.7dB | 0.6dB |
| coUE:16-QAM TDLC-Low | 2.7dB | 2.6dB | 3.2dB | 0.5dB | 0.6dB |
| coUE:64-QAM TDLC-Low | 2.8dB | 2.6dB | 3.2dB | 0.4dB | 0.6dB |
| coUE:QPSK TDLC-Medium | 4.5dB | 5.3dB | 6.1dB | 1.6dB | 0.8dB |
| coUE:16-QAM TDLC-Medium | 5.3dB | 5.3dB | 6.1dB | 0.8dB | 0.8dB |
| coUE:64-QAM TDLC-Medium | 5.5dB | 5.3dB | 6.1dB | 0.6dB | 0.8dB |

Figure 2-2: Simulation results for MU-MIMO with Rank1 target UE using MCS4 and Rank1 co-scheduled UE with non-orthogonal precoding.

| TDLC300-100 ULA Low correlation | TDLC300-100 ULA Medium correlation |
| --- | --- |

Table 2-4: Test configurations for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE in 2RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank1 CoUE Rank1 | FDD: 10/15 | 16QAM, 0.48 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLC300-100 | 2 x 2 ULA Low or ULA Medium | 70 | Test dependent |

Table 2-5: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLC-Low | 11.0dB | 12.7dB | 13.4dB | 2.4dB | 0.7dB |
| coUE:16-QAM TDLC-Low | 12.3dB | 12.7dB | 13.4dB | 1.1dB | 0.7dB |
| coUE:64-QAM TDLC-Low | 12.5dB | 12.7dB | 13.4dB | 0.9dB | 0.7dB |
| coUE:QPSK TDLC-Medium | 14.8dB | 20.4dB | 20.8dB | 6.0dB | 0.4dB |
| coUE:16-QAM TDLC-Medium | 17.7dB | 20.4dB | 20.8dB | 3.1dB | 0.4dB |
| coUE:64-QAM TDLC-Medium | 19.1dB | 20.4dB | 20.8dB | 1.7dB | 0.4dB |

Figure 2-3: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with orthogonal precoding.

| TDLC300-100 Low correlation | TDLC300-100 Medium correlation |
| --- | --- |

Table 2-6: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLC-Low | 12.0dB | 14.8dB | 15.9dB | 3.9dB | 1.1dB |
| coUE:16-QAM TDLC-Low | 14.0dB | 14.8dB | 15.9dB | 1.9dB | 1.1dB |
| coUE:64-QAM TDLC-Low | 14.3dB | 14.8dB | 15.9dB | 1.6dB | 1.1dB |
| coUE:QPSK TDLC-Medium | 15.9dB | 23.1dB | 23.8dB | 7.9dB | 0.7dB |
| coUE:16-QAM TDLC-Medium | 19.5dB | 23.1dB | 23.8dB | 4.3dB | 0.7dB |
| coUE:64-QAM TDLC-Medium | 21.3dB | 23.1dB | 23.8dB | 2.5dB | 0.7dB |

Figure 2-4: Simulation results for MU-MIMO with Rank1 target UE using MCS13 and Rank1 co-scheduled UE with non-orthogonal precoding.

| TDLC300-100 ULA Low correlation | TDLC300-100 ULA Medium correlation |
| --- | --- |

Table 2-7: Test configurations for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE in 4RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank2 + CoUE Rank2 | FDD: 10/15 | 16QAM, 0.48 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLA30-10 or TDLC300-100 | 4 x 4 ULA Low | 70 | Test dependent |

Table 2-8: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 11.0dB | 12.4dB | 12.4dB | 1.4dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 12.1dB | 12.4dB | 12.4dB | 0.3dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 12.3dB | 12.4dB | 12.4dB | 0.1dB | 0.0dB |
| coUE:QPSK TDLC-Low | 13.1dB | 14.5dB | 16.0dB | 2.9dB | 1.5dB |
| coUE:16-QAM TDLC-Low | 14.3dB | 14.5dB | 16.0dB | 1.7dB | 1.5dB |
| coUE:64-QAM TDLC-Low | 14.4dB | 14.5dB | 16.0dB | 1.6dB | 1.5dB |

Figure 2-5: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

Table 2-9: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 13.5dB | 16.1dB | 16.1dB | 3.0dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 15.3dB | 16.1dB | 16.1dB | 1.1dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 15.9dB | 16.1dB | 16.1dB | 0.3dB | 0.0dB |
| coUE:QPSK TDLC-Low | 15.8dB | 19.3dB | 26.6dB | 10.8dB | 7.3dB |
| coUE:16-QAM TDLC-Low | 18.0dB | 19.3dB | 26.6dB | 8.6dB | 7.3dB |
| coUE:64-QAM TDLC-Low | 18.7dB | 19.3dB | 26.6dB | 7.9dB | 7.3dB |

Figure 2-6: Simulation results for MU-MIMO with Rank2 target UE using MCS13 and Rank2 co-scheduled UE with non-orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

Table 2-10: Test configurations for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE in 4RX.

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Co-scheduled UE precoder | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Target UE Rank2 + CoUE Rank2 | FDD: 10/15 | 64QAM, 0.43 | Random QPSK, 16QAM or 64QAM symbols | Orthogonal/ Non-orthogonal | TDLA30-10 or TDLC300-100 | 4 x 4 ULA Low | 70 | Test dependent |

Table 2-11: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with orthogonal precoding.

| Orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 15.2dB | 16.6dB | 16.6dB | 1.4dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 16.5dB | 16.6dB | 16.6dB | 0.1dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 17.0dB | 16.6dB | 16.6dB | -0.4dB | 0.0dB |
| coUE:QPSK TDLC-Low | 18.0dB | 19.9dB | 31.8dB | 13.8dB | 11.9dB |
| coUE:16-QAM TDLC-Low | 19.7dB | 19.9dB | 31.8dB | 12.1dB | 11.9dB |
| coUE:64-QAM TDLC-Low | 20.4dB | 19.9dB | 31.8dB | 11.4dB | 11.9dB |

Figure 2-7: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

Table 2-12: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with non-orthogonal precoding.

| Non-orthogonal precoding | R-ML | E-MMSE-IRC | MMSE-IRC | Gain of R-ML | Gain of E-MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| coUE:QPSK TDLA-Low | 18.7dB | 21.2dB | 21.2dB | 2.5dB | 0.0dB |
| coUE:16-QAM TDLA-Low | 20.8dB | 21.2dB | 21.2dB | 0.4dB | 0.0dB |
| coUE:64-QAM TDLA-Low | 21.6dB | 21.2dB | 21.2dB | -0.4dB | 0.0dB |
| coUE:QPSK TDLC-Low | 22.5dB | 26.3dB | N/A | N/A | N/A |
| coUE:16-QAM TDLC-Low | 25.0dB | 26.3dB | N/A | N/A | N/A |
| coUE:64-QAM TDLC-Low | 25.9dB | 26.3dB | N/A | N/A | N/A |

Figure 2-8: Simulation results for MU-MIMO with Rank2 target UE using MCS17 and Rank2 co-scheduled UE with non-orthogonal precoding.

| TDLA30-10 ULA Low correlation | TDLC300-100 ULA Low correlation |
| --- | --- |

# 2.3 Simulation analysis

We can do several observations from these simulations.

Observations from MCS4 1+1 scenario:

Observation #1: In MCS4 1+1 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.4dB in TDLC300-100 low correlation and 0.7dB in TDLC300-100 medium correlation.

Observation #2: In MCS4 1+1 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.6dB in TDLC300-100 low correlation and 0.8dB in TDLC300-100 medium correlation.

Observation #3: In MCS4 1+1 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.3dB and 0.5dB in TDLC300-100 low correlation and between 0.5dB and 1.2dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #4: In MCS4 1+1 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.4dB and 0.7dB in TDLC300-100 low correlation and between 0.6dB and 1.6dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observations from MCS13 1+1 scenario:

Observation #5: In MCS13 1+1 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.7dB in TDLC300-100 low correlation and 0.4dB in TDLC300-100 medium correlation.

Observation #6: In MCS13 1+1 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 1.1dB in TDLC300-100 low correlation and 0.7dB in TDLC300-100 medium correlation.

Observation #7: In MCS13 1+1 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.9dB and 2.4dB in TDLC300-100 low correlation and between 1.7dB and 6.0dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #8: In MCS13 1+1 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 1.6dB and 3.9dB in TDLC300-100 low correlation and between 2.5dB and 7.9dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observations from MCS13 2+2 scenario:

Observation #9: In MCS13 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 1.5dB in TDLC300-100 low correlation.

Observation #10: In MCS13 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 7.3dB in TDLC300-100 low correlation.

Observation #11: In MCS13 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.1dB and 1.4dB in TDLA30-10 low correlation and between 1.6dB and 2.9dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #12: In MCS13 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.3dB and 3.0dB in TDLA30-10 low correlation and between 7.9dB and 10.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observations from MCS17 2+2 scenario:

Observation #13: In MCS17 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 11.9dB in TDLC300-100 low correlation.

Observation #14: In MCS17 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation.

Observation #15: In MCS17 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 1.4dB in TDLA30-10 low correlation and between 11.4dB and 13.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #16: In MCS17 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 2.5dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Common observations:

Observation #17: E-MMSE-IRC gain over MMSE-IRC does not depend on modulation order of co-scheduled UE.

Observation #18: R-ML gain over MMSE-IRC increases when modulation order of co-scheduled UE decreases.

Observation #19: In TDLA30-10, MMSE-IRC and E-MMSE-IRC are practically equivalent.

# Conclusion

In this paper we provided the view on the advanced receiver to cancel inter-user interference for MU-MIMO. The following observations and proposals are made:

Observation #1: In MCS4 1+1 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.4dB in TDLC300-100 low correlation and 0.7dB in TDLC300-100 medium correlation.

Observation #2: In MCS4 1+1 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.6dB in TDLC300-100 low correlation and 0.8dB in TDLC300-100 medium correlation.

Observation #3: In MCS4 1+1 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.3dB and 0.5dB in TDLC300-100 low correlation and between 0.5dB and 1.2dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #4: In MCS4 1+1 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.4dB and 0.7dB in TDLC300-100 low correlation and between 0.6dB and 1.6dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #5: In MCS13 1+1 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.7dB in TDLC300-100 low correlation and 0.4dB in TDLC300-100 medium correlation.

Observation #6: In MCS13 1+1 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 1.1dB in TDLC300-100 low correlation and 0.7dB in TDLC300-100 medium correlation.

Observation #7: In MCS13 1+1 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.9dB and 2.4dB in TDLC300-100 low correlation and between 1.7dB and 6.0dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #8: In MCS13 1+1 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 1.6dB and 3.9dB in TDLC300-100 low correlation and between 2.5dB and 7.9dB in TDLC300-100 medium correlation depending on co-scheduled UE modulation order.

Observation #9: In MCS13 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 1.5dB in TDLC300-100 low correlation.

Observation #10: In MCS13 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 7.3dB in TDLC300-100 low correlation.

Observation #11: In MCS13 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.1dB and 1.4dB in TDLA30-10 low correlation and between 1.6dB and 2.9dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #12: In MCS13 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between 0.3dB and 3.0dB in TDLA30-10 low correlation and between 7.9dB and 10.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #13: In MCS17 2+2 scenario with orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and 11.9dB in TDLC300-100 low correlation.

Observation #14: In MCS17 2+2 scenario with non-orthogonal random precoding E-MMSE-IRC gain over MMSE-IRC is 0.0dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation.

Observation #15: In MCS17 2+2 scenario with orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 1.4dB in TDLA30-10 low correlation and between 11.4dB and 13.8dB in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #16: In MCS17 2+2 scenario with non-orthogonal random precoding R-ML gain over MMSE-IRC varies between -0.4dB and 2.5dB in TDLA30-10 low correlation and undefined in TDLC300-100 low correlation depending on co-scheduled UE modulation order.

Observation #17: E-MMSE-IRC gain over MMSE-IRC does not depend on modulation order of co-scheduled UE.

Observation #18: R-ML gain over MMSE-IRC increases when modulation order of co-scheduled UE decreases.

Observation #19: In TDLA30-10, MMSE-IRC and E-MMSE-IRC are practically equivalent.

# Reference

- RP-230313, “Status report for NR demodulation requirement evolution”, China Telecom, Intel Corporation

- R4-2309892, “WF on advanced receiver for MU-MIMO scenario”, China Telecom