3GPP TSG-RAN WG4 Meeting # 108										                   R4-2312430

Toulouse, France, August 21 – August 25, 2023

Title: 					Discussion on measurement requirements for eRedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	8.31.2

Document for:	Discussion

Introduction

The revised WID on enhanced support of reduced capability NR devices has been approved in [1]. Techniques for further UE complexity reduction have been studied in the study item documented in TR 38.865. The WID has the following objectives:

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In RAN4# 107 meeting, it concluded that no CGI reading related requirements are specified for R18 RedCap when SIB1 is scheduled with BW > 5 MHz. Thus this contribution provides the analysis on measurement requirements due to inactive enhanced eDRX configuration

Discussion

- Neighbor cell measurement requirements with inactive eDRX

In RAN2#121bis-e meeting, the following progress is made for T calculation. In general, T determination is settled down although there are some small open issues, e.g., whether RAN PTW is larger than CN PTW, or whether RAN PTW length is always the same as CN PTW length.

T calculation In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

Based on RAN2 progress, the summary of T is provided as below.

If RAN PTW length and CN PTW length is different, then there are three kinds of T (shown in Figure 1):

- In non-overlapped PTW: T1. As per current RAN2 agreement, T1= RAN configured DRX cycle;

- In overlapping PH, T2-1 is for overlapping part of CN and RAN PTW, and T2-2 is for the non-overlapping part with PTW.

- If CN PTW length is larger than RAN PTW (shown in Figure 1 case 1), as per current RAN2 agreement,

T2-1 = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information};

T2-2 = min {CN configured DRX cycle, default paging cycle broadcast in system information};

- If RAN PTW is larger than CN PTW as shown in Figure 1 case 2 (whether this configuration is valid depends on RAN2 decision), as per latest RAN2 agreement:

T2-1 = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information};

T2-2 = RAN configured DRX cycle;

Note: The case RAN PTW is larger than CN PTW will be decided in RAN2

| Case 1. CN PTW length is larger than RAN PTW |  |
| --- | --- |
| Case 2. RAN PTW is larger than CN PTW (if valid) |  |

Figure 1 ‘T’ in RAN and CN paging coincide PH and non-coincide PH if RAN PTW length and CN PTW length is different

If RAN PTW length and CN PTW length is always the same (whether the case is valid depends on RAN2 decision), then there are two kinds of T (shown in Figure 2):

- In non-overlapped PTW: T1. As per latest RAN2 agreement, T1= RAN configured DRX cycle;

- In overlapping PH: T2. As per latest RAN2 agreement, T2 = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

Figure 2. ‘T’ in RAN and CN paging coincide PH and non-coincide PH if RAN PTW length and CN PTW length is always the same (if valid)

As in R17 RedCap, the RRM requirements of serving cell are specified based on T (paging DRX) which are applied for both inside PTW and outside PTW. For R18 eRedCap, T is constrained to within PTW. In RAN4#107 meeting, it is agreed that the measurement requirements for serving cell are based on T. The open issues are mainly on measurement requirements of neighbour cells in inactive eDRX.

As per RAN2 agreement, Rel-18 enhanced INACTIVE eDRX cycle shall be not larger than the IDLE eDRX cycle. It means that it shall be inactive PTW (RAN paging PTW) at the non-overlapping PH. It is straight forward that UE follows T1 which is RAN configured DRX cycle to perform measurement, as shown with dotted line in Figure 3.

Proposal 1: At non-coincided PH where IDLE and INACTIVE eDRX PTW do not coincide, UE performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW (i.e., RAN PTW).

Figure 3.  UE performs measurement at non- coincided PH

As in R17 RedCap, the measurement period of intra-frequency/inter-frequency measurement depends on fixed inactive eDRX cycles. The reason why T is not used for neighbour cell measurement is that paging DRX T is different within idle_PTW and outside idle_PTW. Measurements as per changed T would complicate UE implementation. In RAN4# 107 meeting for R18 Redcap with inactive eDRX it is agreed that neighbour cell measurement is to be specified in step of RAN configured DRX cycle (i.e., T1) [2].

Measurement period when configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements - The detection/measurement/evaluation delay requirement are specified in the step of: - RAN configured DRX cycle, if eDRX_Inactive 20.48sec

The above agreement for R18 eRedcap is aligned with the R17 Redcap measurement principle- using fixed measurement step.

At the coincided PH, CN paging PTW and RAN paging PTW are overlapped. According to the above agreement, UE shall perform measurements in step of RAN configured DRX cycle either. The question is where UE to perform measurements when the PTWs are partially overlapping. There are two possible options:

Option 1: UE performs neighbour cells measurement within RAN Paging PTW (shown in Figure 4.a)

Option 2: UE performs neighbour cells measurement within CN paging PTW (shown in Figure 4.b)

According to option 2, the measurement window length during coincided PH and non-coincided PH is different. Then from UE implementation perspective, performing measurement in varied window length would impact UE sampling algorithms and complicate implementation. Therefore an unified measurement window (option 1) is more preferred. In option 1, regardless within coincided PH or non-coincided PH, UE always performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW.

Figure 4.  UE performs measurement at coincided PH

Proposal 2: At coincided PH where IDLE and INACTIVE eDRX PTW are partial overlapped, UE only performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW (i.e., RAN PTW).

Based on proposal 1 and proposal 2,

Proposal 3: When UE configured with inactive eDRX>10.24s, UE performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW (i.e., RAN PTW).

Transition requirements

Another open issue is whether to define requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec). For R17 RedCap, the following requirements are specified:

“For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.”

We have no strong view whether the transmission requirements need to be specified. If it is to be specified, then if inactive eDRX configuration is changed, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

Proposal 4: UE needs to satisfy the less stringent requirements for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(>10.24s).

Conclusions

This contribution provides analysis on the measurement requirements for RedCap UE when eDRX is configured. According to the analysis, we propose the following proposals.

Proposal 1: At non-coincided PH where IDLE and INACTIVE eDRX PTW do not coincide, UE performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW (i.e., RAN PTW).

Proposal 2: At coincided PH where IDLE and INACTIVE eDRX PTW are partial overlapped, UE only performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW (i.e., RAN PTW).

Proposal 3: When UE configured with inactive eDRX>10.24s, UE performs neighbour cell measurement based on the RAN configured DRX cycle within inactive PTW (i.e., RAN PTW).

Proposal 4: UE needs to satisfy the less stringent requirements for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(>10.24s).

References

- RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson

- R4-2310154, WF on R18 RedCap RRM requirements, Ericsson