3GPP TSG-RAN4 Meeting #108	R4-2312561

Toulouse, France，August 21 – August 25, 2023

Agenda item:			8.3.2.1

Source:	vivo

Title:	Discussion on CA congifuration of CA_n5A-n28A-n105A

Document for:	Discussion

- Introduction

In RAN#100 meeting, the SI [1] has been approved to complete the study of 3 bands combination: CA_n5A-n28A-n105A. The specific following UL configuration need to be studied: UL_ n5A-n28A, UL_ n5A-n105A, single UL in either n28 or n105. Power class 3 (PC3) is considered, and the following aspects need be studied:

    - UE RF architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna and multiple low band narrowband antennas.

    - Evaluate the MSD requirements if needed.

    - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators.

In this paper, we’d like to preliminary discuss the multiple potential UE RF architecture and MSD requirements for CA_n5A-n28A-n105A combination.

- Discussion

  - UE RF architecture assumption

The spectrum range of CA_n5-n28-n105 is shown in Figure 1, as can be observed that n28UL is adjacent to n105UL. Uplink transmission of CA_n28A-n105A will produce some apparent interference issues, like 2UL IMD products, cross band interference and so on. Therefore in the SI [1], UL combination of CA_n28A-n105A has been excluded.

Figure 1. CA_n5A-n28A-n105A spectrum range

    - UE RF architecture with two antennas

The potential RF architectures with two antennas for CA_n5A-n28A-n105A are showed below. The UL configuration of CA_ n28A-n105A will not be supported for this band combination in the SI [1]. To avoid interference issues, the related antenna structures covering both n28UL and n105UL in the same TRx are excluded. As can be seen in the figure 2, both of the Main TRx and Diversity Rx for architecture (a) and (b) need to cover the whole spectrum range from n105DL to n5DL, which is as long as 282MHz. It takes a huge challenge to the bandwidth ratio for a typical antenna design in a smartphone. And the design of pentaplexer need to aggregate five closely spaced spectrum ranges together, considering not only the strict filter isolation but also appropriate insertion loss. Therefore, it seems difficult to consider 2-antenna structure as the baseline architecture.

| (a) | (b) |
| --- | --- |

Figure2. Potential UE architecture with 2 antennas, supporting CA_n5A-n28A-n105A

    - UE RF architecture with four antennas

The assumption RF architecture with four antennas for CA_n5A-n28A-n105A is showed below. Three main TRx antennas belong to three different frequency bands, avoiding combination of different signals. Hence the stress of multiplexer design can be reduced, and the frequency range covered by each antenna is shorter as well. However, the use of four antennas will inevitably take up more space of the smartphone, which should be further considered in practical application.

Figure3. Potential UE architecture with 4 antennas, supporting CA_n5A-n28A-n105A

    - UE RF architecture with three antennas

The potential RF architectures with three antennas for CA_n5A-n28A-n105A are showed below. The UL configuration of CA_ n28A-n105A will not be supported for this band combination in the SI [1]. To avoid interference issues, the related antenna structures covering both n28UL and n105UL in the same TRx are excluded. Compared with 2-antenna structure, 3-antenna structure could avoid complex pentaplexer, and the use of triplexer and quadplexer can ease the requirement of filter isolation and insertion loss to some extent. However, the challenge of wide spectrum range from n5DL to n105DL seems still unavoidable. Comparing with 4-antenna structure, 3-antenna structure occupies less space in the smart phone, reducing the difficulty of application. Therefore, 3-antenna architecture could be considered as a baseline structure for the study of CA_n5A-n28A-n105A.

Observation1：3-antenna architecture could be a baseline structure for CA_n5A-n28A-n105A.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure4. Potential UE architecture with 3 antennas, supporting CA_n5A-n28A-n105A

  - 2DL/2UL configurations

2DL_2UL cases are fallback configurations of CA_n5A-n28A-n105A, containing DL_n5A-n28A_UL_n5A-n28A and DL_n5A-n105A_UL_n5A-n105A, the interference issues have been evaluated minutely yet, such as harmonics and harmonic mixing, 1UL cross-band MSD and 2UL cross-band MSD, related requirements and text configurations are recorded in TR38.872 and will not be listed here.

  - 3DL configurations

    - Harmonics and harmonic mixing

The following table1 is UL harmonic products of CA_n5-n28-n105, and as can be seen there is no related harmonic interference need to consider further.

Table 1. Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |

The following table2 is DL harmonic products of CA_n5-n28-n105, and as can be seen there is no related harmonic mixing interference need to consider further.

Table 2. Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 | 3790 | 4015 |

    - 1UL cross-band MSD

Based on TR 38.872, the present 1UL cross-band MSD test configuration could be a reference considering the impacts from band n5 UL to n28DL, n5 UL to n105DL, n105 UL to n5DL.

Observation2: The present 1UL cross-band MSD test configuration of CA_n5-n105, CA_n5-n28 and CA_n28-n105 could be a reference.

Table 3: CA_n5A-n28A-n105A 1UL cross-band MSD

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | TBD | ACLR2 |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | TBD | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | TBD | >ACLR2 |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | TBD | >ACLR2 |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | TBD | ACLR2 |

    - IMD products

      - UL_n5A-n28A

Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis is listed below. As table 4 shows, there is no IMD issues for band n5 or band n28, however the two-tone 3rd order IMD products of UL_n5A-n28A will fall into Rx frequencies of band n105.

Table 4. Band n5 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 869 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 166 | 76 | 1527 | 1617 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 1035 | 537 | 672 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2486 | 2230 | 2365 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1724 | 1904 | 1240 | 1420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3175 | 3355 | 2933 | 3113 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 332 | 3054 | 3234 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2168 | 1943 | 2773 | 2548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 371 | 1201 | 976 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3636 | 3861 | 3999 | 4224 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3757 | 3982 | 3878 | 4103 |

      - UL_n5A-n105A

Band n5 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis is listed below. As table 5 shows, there is no IMD issues for band n5, band n28, or band n105 with UL_n5-n105 configuration.

Table 5. Band n5 and Band n105 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Observation3: For CA_n5A-n28A-n105A, the two-tone 3rd order IMD products of UL_n5A-n28A will fall into Rx frequencies of band n105. The following test configuration could be a reference.

Table 6. 3DL/2UL IMD3 MSDs for CA_n5A-n28A-n105A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5A-n28A-n105A | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n28 | 723.5 | 5 | 25 | 778.5 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | 25 | 620.5 | TBD | FDD | IMD3 |

- Conclusion

Observation1：3-antenna architecture could be a baseline structure for CA_n5A-n28A-n105A.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Observation2: The present 1UL cross-band MSD test configuration of CA_n5-n105, CA_n5-n28 and CA_n28-n105 could be a reference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | TBD | ACLR2 |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | TBD | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | TBD | >ACLR2 |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | TBD | >ACLR2 |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | TBD | ACLR2 |

Observation3: For CA_n5A-n28A-n105A, the two-tone 3rd order IMD products of UL_n5A-n28A will fall into Rx frequencies of band n105. The following test configuration could be a reference.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5A-n28A-n105A | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n28 | 723.5 | 5 | 25 | 778.5 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | 25 | 620.5 | TBD | FDD | IMD3 |

- References

- RP-231141 SID revision: Study on enhancement for sub-1GHz NR band combinations, #RAN 100.