3GPP TSG-RAN WG4 # 108	R4-2312614

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.4.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#107 meeting, a WF related to 8Rx UE PDSCH requirements was agreed [1]. In this contribution, we provide simulation results and discuss outstanding issues.

- Discussion

We provide our simulation results in Table 1 based on the assumptions from Annex A.1 in [1].

Table 1. SNR point of 70% maximum throughput for rank 2, 4, 8

| Rank | MCS | Propagation Condition | Channel Correlation | TDD SNR (dB) | FDD SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 12.5 | 12.6 |
| 4 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 12.5 | 12.7 |
| 8 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 21.2 | 21.4 |

Since RAN4 already define high MCS/code rate in SDR tests, we propose to define 8Rx PDSCH requirements with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS17 (rank8, ULA low, TDLA 30-10)

Proposal 1: Introduce PDSCH requirements for 8Rx UE with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS17 (rank8, ULA low, TDLA 30-10)

- Conclusion

In this contribution, we provide simulation results based on agreed simulation assumptions. According to simulation results, we propose to define 8Rx PDSCH requirements with the following MCS.

Proposal 1: Introduce PDSCH requirements for 8Rx UE with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS17 (rank8, ULA low, TDLA 30-10)

- Reference

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon