3GPP TSG-RAN WG4 Meeting #108 	R4-2313058

Toulouse, France meeting, August 21st – 25th, 2023

Agenda Item: 8.10.2.4

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option B-1-2

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#99 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option B-1-2 - Specify support of BM/RLM/BFD based onSSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to Interoperability Device Testing (IoDT) availability). - The interruption related requirements will be decided and specified in RAN4. 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - Note: further check in RAN #101 on whether any work is required to support L3 intra-frequency measurements without gaps for Option B-1-2 The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

Therefore, in this contribution paper, we provide analysis and discussion on how to specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions.

# Discussion on RRM requirements

RAN4 has discussed the requirements applicability for Option B-1-2 in the previous meeting R4#107 RAN4 reached the following way forward:

#### Issue 5-2: Interruption requirements for supporting option B-1-2 <Agreement >: - Define the following interruption requirements for Option B-1-2 - Interruption length - Option 1: [0.5ms] for FR1 and [0.25ms] for FR2 - FFS: Interruption ratio - FFS: Interruption location #### Issue 5-4: Applicable of conclusions for Option B-1-1 to Option B-1-2 <Agreement >: - When CD-SSB is outside the active BWP, the requirements for SSB-based RLM/BFD/BM for UE supporting option B-1-2 with interruptions are FFS. #### Issue 5-6: Clarification on existing timing requirements for supporting Option B-1-2 <Agreement >: - Follow conclusion of Issue 3-3 for option B-1-1.

Regarding Issue 5-2:

The existing RAN4 requirements specify that the UE is not required to perform BM/RLM/BFD outside the active BWP. However, in the previous RAN plenary #99/100, a new WI was introduced to allow the UE to perform BM/RLM/BFD measurements based SSB outside the active BWP with interruption. RAN4 has not introduced interruption requirements for L1/BM/RLM/BFD for existing NR and hence new interruption requirements shall be defined for BM/RLM/BFD measurements. However, specification in 38.133 already cover interruption requirements for L3 measurements in the form on NCSG or NFG. Therefore, RAN4 shall leverage the interruption requirements from L3 measurements to define the interruption requirements for BM/RLM/BFD measurements.

In previous meetings RAN4 agreed that RAN4 shall define requirements for interruption length. In general, the interruption length is based on the RF retune and baseband preparation duration as defined in NCSG requirements; hence, the interruption length can be the same as these defined for NCSG. This means, when a UE signals that interruption is needed for gap-less measurements the interruption length can be VIL=1 ms in FR1 and VIL=0.75 ms in FR2 (i.e. one VIL before and after the SMTC).

- RAN4 shall leverage the interruption requirements (NCSG and NFG) from L3 measurements to define the interruption requirements for RLM/BFD/BM measurements.

- RAN4 shall define the interruption length requirements the same as these defined for NCSG in Rel-17, (i.e. VIL=1 ms in FR1 and VIL=0.75 ms in FR2).

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 shall leverage the interruption requirements (NCSG and NFG) from L3 measurements to define the interruption requirements for RLM/BFD/BM measurements.

Proposal 2: RAN4 shall define the interruption length requirements the same as these defined for NCSG in Rel-17, (i.e. VIL=1 ms in FR1 and VIL=0.75 ms in FR2).

# References

- RP-231486, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Taipei, Taiwan, June 2023.

- R4-2310176, “WF on NR BWP operation without restriction”, vivo, Incheon, KR, May 2023.