3GPP TSG-RAN WG4 Meeting # 108																R4-2313934

Toulouse, France, Aug 21 – Aug 25, 2023

Title:	[Draft] LS on report quantity parameter setting for CQI reporting with 1Tx

Response to:

Release:	Release 15

Work Item: 			NR_newRAT-Perf

Source:		RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name:	Osamu Yamashita

E-mail Address:	Osamu.Yamashita@anritsu.com

Send any reply LS to:			3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R4-2311202 Report quantity parameter setting for CQI reporting with 1Tx

1. Overall Description:

In RAN4 #108 RAN4 discussed on a test parameter called reportQuantity for demodulation CQI reporting with 1Tx and noticed that PMI specification is not defined in TS 38.214 cl.5.2.2.2.1 in a case of the 1 antenna port. It is not clear on how PMI for Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured.

After the further investigation in RAN1 specification TS 38.212, we noticed that there is a description at clause 6.3.1.1.2 about CSI sequence generation in UCI over PUCCH. According to the sentence, the bitwidth for PMI with 1 CSI-RS port is 0. Therefore, there may be a chance that we configure the report quantity as ‘cri-RI-PMI-CQI’ and UE will not report the PMI. Though, the intended behavior for UEs not supporting the “csi-ReportWithoutPMI” capability when dealing with zero length PMI is not clear, and neither is it clear why the zero length PMI is not specified in UCI over PUSCH. 
Furthermore, another issue was identified with TS38.331 on codebookConfig IE that it is undefined for one CSI/antenna port. Since the codebookConfig IE is optional according to TS 38.331, there is a possibility that under 1Tx scenario where no precoder is applied, gNB can omit the codebookconfig field, which means no signaling of precoder Type information and codebook restriction. RAN4 also analyzed that at least for PUSCH part, codebook type will be undefined if CodebookConfig is not signalled (as it is conditioned on codebookType) and consequently PMI bitwidth will be undefined.

We do note that all the CSI tests with 1Tx in the latest RAN4 UE demodulation performance requirements assume the CSI report via PUCCH, and all the test cases configure “PMI with no codebook config”. Since it may not be aligned with RAN1’s intention as codebook config is conditioned on codebookType, RAN4 kindly asks RAN1 to take the above information into account and clarify how PMI for Type I codebook is reported from the UE when cri-RI-PMI-CQI is configured with 1Tx.

2. Actions:

To RAN1 group.

ACTION:	RAN4 kindly asks RAN1 to take the above information into account and clarify how PMI for Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured with 1Tx. Including what are the consequences of not configuring the optional codebookConfig IE.

To RAN2 group.

ACTION:	RAN4 kindly asks RAN2 to take the above information into account and clarify how RAN4 should configure codebook for 1Tx.

3. Date of Next RAN4 Meeting:

TSG-RAN WG4#108bis	                October 9 - 13, 2023		Xiamen, China

TSG-RAN WG4#109	                November 13 - 17, 2023	Chicago, US