3GPP TSG-RAN WG4 Meeting #108	R4-2314339

Toulouse, France, , 21st ‒ 25th August, 2023

Title: 	WF on support for bandwidth part operation without restriction

Agenda Item:	8.10.3

Source: 	vivo

Document for:	Approval

- Topic #1: General aspects

#### Issue 1-2: Requirements for handover for Option A, C, B-1-1 and B-1-2

- It is RAN4 understanding that it is beneficial defining requirements for handover to a BWP with NCD-SSB directly for UE supporting option C.

- Whether to define the HO requirements is up to RAN plenary discussion.

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option B-1-1 and C

- It is RAN4 understanding that for UE supporting both option B-1-1 and C, if NCD-SSB is configured within the active BWP, then UE is expected to perform RLM/BFD/BM and L3 intra-frequency measurements based on NCD-SSB based on TS 38.331. No additional requirements will be defined for such scenario.

#### Issue 1-4: Requirements/UE behaviour for UE supporting both option B-1-1 and A

- Option 1:

  - RAN4 doesn’t need to specifically discuss requirements for UE supporting both option B-1-1 and A since UE can always measure CD-SSB without gap or interruption.

- Option 2:

  - There is no obvious gain if option A and option B-1-1 are supported simultaneously.

  - If it is supported that UE works with option A and option B-1-1 simultaneously, UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

- Option 3:

  - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour.

- Option 4:

  - If the UE is operating in an active BWP with no NCD-SSB, and no CD-SSB but WITH CSI-RS, then would the UE perform the measurements based on Option B-1-1 or Option A (CSI-RS)?

- Option 5:

  - RAN4 not to define additional requirements or clarify UE behaviour for UE supporting multiple options.

- Option 6:

  - Whether RAN4 should define requirements for a UE supporting multiple options could be discussed later.

#### Issue 1-5: Requirements/UE behaviour for UE supporting both option C and A

- Option 1:

  - For UE supporting both option C and A, when active BWP contains NCD-SSB, RAN4 shall discuss whether RLM/BM/BFD should be based on NCD-SSB or CSI-RS. At least we don’t expect UE needs to monitor both.

- Option 2:

  - UE working with option A and option C simultaneously should be supported and UE behaviour/conditions for RLM/BM/BFD measurements should be defined.

- Option 3:

  - For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour.

- Option 4:

  - RAN4 not to define additional requirements for UE supporting multiple options.

- Option 5:

  - Whether RAN4 should define requirements for a UE supporting multiple options could be discussed later.

- Option 6:

  - A UE supporting Option C must always additionally support at least on other options (A, B-1-1 or B-1-2).

#### Issue 1-6: Requirements/UE behaviour for UE supporting both option B-1-1 and B-1-2

- UE shall not indicate support of both B-1-1 and B-1-2 for the same agreed granularity reporting. Whether and how to capture this in 38.133 is FFS depending on RAN1 conclusion.

- Topic #2: Impact of Option A

#### Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP

- Option 1:

  - No clarification on existing timing requirements when CD-SSB is outside active BWP is needed

- Option 2:

  - In Rel-18, it is clarified in clause 7.1.2 by adding one sentence “the SSB is within the channel BW of the UE”.

- Topic #3: Impact of Option B-1-1

#### Issue 3-1: Applicable conditions of existing RLM/BFD/BM requirements for Option B-1-1

- For a UE capable of supporting Option B-1-1, the UE is required to meet BM/RLM/BFD requirements without interruptions if CD-SSB is within the channel bandwidth of the UE.

- The applicable conditions are to be updated in requirements in clauses 8.1.1, 8.1.2, 8.5.1, 8.5.2, 8.5.5, 9.5.1, 9.8.1.

- Note: additional conditions can be considered subject to RAN1 agreements

#### Issue 3-3: Clarification on existing timing requirements for supporting Option B-1-1

- Option 1:

  - Follow conclusion of issue 2-2 (agreements in the last meeting).

- Option 2:

  - No clarification is needed for the UE to meet the existing UE transmit timing requirements (section 7.1 of TS 38.133) when performing BM/RLM/BFD according to option B-1-1

#### Issue 3-4: Clarification/modification on requirements for intra-frequency measurements without gaps requirements for supporting option B-1-1

- From RAN4 perspective same UE feature group can be used for support of Option B-1-1 RLM/BFD/BM and L3 intra-frequency measurements

- For UE capable of supporting Option B-1-1, the requirements in section 9.2.5 for Intra-frequency measurements without measurement gaps are applicable if CD-SSB is within the channel bandwidth of the UE.

- Note: additional conditions can be considered subject to RAN1 agreements

- FFS:  How would UE report NeedForGap/NeedForGapNCSG with ‘no-gap’ for intra-frequency measurement when supporting of option B-1-1 is indicated already?

- Topic #4: Impact of Option C

#### Issue 4-1: Applicable conditions of existing RLM/BFD/BM requirements for Option C

- Clarify in RLM/BFD/BM measurement requirements in clauses 8.1, 8.5, 9.5 and 9.8 that the SSB and SMTC applies for CD-SSB and NCD-SSB if the UE is capable of supporting Option C. Detailed wording is up to CR stage discussion.

#### Issue 4-3: Clarification on existing timing requirements for supporting Option C

- In Rel-18, it is clarified in clause 7.1.2 by adding one sentence “the SSB includes CD-SSB and NCD-SSB for UE capable of option C”.

#### Issue 4-4: Spec impact on intra-frequency measurement requirements for UE supporting option C

- Spec impact on intra-frequency measurement requirements for UE supporting option C

  - The requirements for L3 intra-frequency measurements are based on the existing requirements in 9.1.

  - The SSB and SMTC in section 9.1.1 applies for both CD-SSB and NCD-SSB for the UE capable of option C, i.e., use the same text as for RedCap.

  - The reference SSB definition for non-RedCap should be captured in clause 9.2.1 with the same definition as for RedCap.

    - Note: alignment with RAN2 terminology is needed

- From RAN4 perspective same UE feature group can be used for support of NCD-SSB based RLM/BFD/BM and L3 intra-frequency measurements

#### Issue 4-5: Requirements for transition between CD-SSB and NCD-SSB for supporting option C

- The intra-/inter-frequency definition and requirements for transition between CD-SSB and NCD-SSB for L3 measurement from RedCap can be re-used for UE supporting option C by defining requirements in clause 9.1.6.

#### Issue 4-6: SSB configuration for UE supporting option C

- FFS:

  - For a non-RedCap UE supporting Option C, the Rel-17 NCD-SSB based L3 measurement and mobility requirements are adopted. When the UE is in a CA mode, a measurement resource on an SCell is assumed as below depending on the state of the cell:

    - When the SCell is deactivated

      - CD-SSB indicated by servingCellMO in ServingCellConfig.

    - During SCell activation

      - NCD-SSB in the first active BWP for AGC/time/freq tracking, etc.

    - When the SCell is activated and the first active BWP is in a dormant state

      - NCD-SSB in the first active BWP for L1 and L3 measurements

- Topic #5: Impact of Option B-1-2

#### Issue 5-2: Interruption requirements for supporting option B-1-2

- Introduce interruption length and interruption ratio requirements for Option B-1-2

#### Issue 5-2a: Interruption length (RF tuning time) for option B-1-2

- Option 1:

  - Interruption length is 0.5ms for FR1 and 0.25ms for FR2.

- Option 2:

  - Interruption length is 1ms for FR1 and 0.75ms for FR2.

- Option 3:

  - Interruption length is max (slot length of the cell, 0.5ms) for FR1 and max(slot length of the cell, 0.25ms) for FR2.

#### Issue 5-2b: Interruption length requirements for option B-1-2

- Option 1: Interruption length in number of slots (based on interruption length of 0.5ms for FR1 and 0.25ms for FR2)

Interruption length in NR CA:

Table 2: Interruption due to RLM and RLP measurements on other active serving cell in inter-band NR CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 3: Interruption due to RLM and RLP measurements on other active serving cell in intra-band NR CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + NSSB |
| 1 | 0.5 | 1 + NSSB |
| 2 | 0.25 | 2 + NSSB |
| 3 | 0.125 | 4 + NSSB |
| 5 | 0.03125 | 16 + NSSB |
| 6 | 0.015625 | 32 + NSSB |
| NOTE: NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR |  |  |

Interruption length in NR DC:

- For synchronous inter-band NR-DC, the interruption length is the same as in table 2.

- For asynchronous inter-band NR-DC, the interruption length is shown in table 4.

Table 4: Interruption due to RLM and RLM on other active serving cell in asynchronous inter-band NR-DC

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 4 |
| 3 | 0.125 | Aggressor cell is on FR2 | 5 |
|  |  | Aggressor cell is on FR1 | 6 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 18 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 34 |

Interruption length in EN-DC and NE-DC:

For inter-band EN-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band EN-DC or

- 2 subframes for asynchronous inter-band EN-DC.

For synchronous intra-band EN-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

For inter-band NE-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band NE-DC or

- 2 subframes for asynchronous inter-band NE-DC.

For synchronous intra-band NE-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

Where, NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR.

- Option 2: Interruption length in number of slots (based on interruption length of 1ms for FR1 and 0.75ms for FR2)

#### Issue 5-2c: Interruption ratio for option B-1-2

- Option 1:

  - For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%].

- Option 1a:

  - In addition to the maximum length of each interruption, the maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined:

  - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %.

  - The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %.

  - The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

- Option 2:

  - For UE supporting option B-1-2, the probability of missed ACK/NACK is 1% for ALL RLM/BFM/BM(L1-RSRP/L1-SINR) measurements based on SSB outside active BWP.

- Option 2a:

  - The probability of missed ACK/NACK for a UE supporting Option B-1-2, due to interruptions caused by UE performing BM/RLM/BFD measurements based on SSB outside the active BWP, shall not exceed [1.0]%.

- Option 2b:

  - In addition to the maximum length of each interruption, the maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined:

  - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD, CBD and L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 1 %.

- Option 3:

  - X%=interruption length * 2 / L1-RS periodicity, where X% is the interruption ratio, interruption length is 0.5ms in FR1 and 0.25ms in FR2, and L1-RS periodicity is the periodicity of SSB configured for BM/RLM/BFD after taking scaling factor P into account.

#### Issue 5-4: How interruption requirements for supporting option B-1-2 are captured

- FFS:

  - Interruption requirements for RLM/BFD/BM measurements for UE supporting option B-1-2 are captured in section 8.2.

#### Issue 5-5: RLM/BFD/BM requirements for Option B-1-2

- For a UE capable of supporting Option B-1-2, the UE is allowed to cause interruptions to meet CD-SSB based BM/RLM/BFD requirements if CD-SSB is within the channel bandwidth of the UE but outside of active BWP.

  - The interruption requirements are FFS

  - If the existing BM/RLM/BFD requirements can be reused is depending on how interruption ratio is defined.

- The requirements are defined based on the assumption that there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured.

- The applicable conditions are to be updated in requirements in clauses 8.1.1, 8.1.2, 8.5.1, 8.5.2, 8.5.5, 9.5.1, 9.8.1.

- Note: additional conditions can be considered subject to RAN1 agreements

- Draft CR work split

Spec impact are identified for option B-1-1, B-1-2 and C on L1 measurements (RLM/BFD/BM) and L3 intra-frequency measurement. Draft CRs are needed for those changes. Company is welcome to work on the interested changes.

| No. | Requirements to be updated | Company |
| --- | --- | --- |
| 1 | Option B-1-1: RLM in clause 8.1 BFD/CBD in clause 8.5 L1-RSRP measurements in clause 9.5 L1-SINR measurements in clause 9.8 | Apple |
| 2 | Option B-1-1: Intra-frequency measurement in clause 9.2 | Nokia |
| 3 | Option B-1-2: RLM in clause 8.1 BFD/CBD in clause 8.5 L1-RSRP measurements in clause 9.5 L1-SINR measurements in clause 9.8 | MediaTek |
| 4 | Option B-1-2: Interruption requirements in clause 8.2, if needed | CATT |
| 5 | Option C: RLM in clause 8.1 BFD/CBD in clause 8.5 L1-RSRP measurements in clause 9.5 L1-SINR measurements in clause 9.8 | Huawei |
| 6 | Option C: Intra-frequency measurement in clause 9.2 Timing requirements | vivo |
| 7 | Timing requirements for option A, B-1-1 and B-1-2, if needed | vivo |

Table 6-2: Requirements in TS 36.133

| No. | Requirements to be updated | Company |
| --- | --- | --- |
| 1 | Option B-1-2: Interruption requirements in clause 7.32 (EN-DC) and clause 7.36 (NE-DC), if needed | Ericsson |

----EoD---