3GPP TSG-RAN WG4 Meeting #108-bis	R4-2315008

Xiamen, China, 09 – 13 October 2023

3GPP TSG RAN WG1 Meeting #114	R1-2308561

Toulouse, France, August 21st – August 25th 2023

Title:	LS on further clarifications on enhancements to realize increasing UE power high limit for CA and DC

Response to:	Reply LS on enhancements to realize increasing UE power high limit for CA and DC (R4-2310500)

Release:		Rel-18

Work Item:		NR_cov_enh2

Source:		RAN1

To:		RAN4, RAN2

Cc:

Contact person:		Marco Maso, marco (dot) maso (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

# 1	Overall description

RAN1 thanks RAN4 for the Reply LS on enhancements to realize increasing UE power high limit for CA and DC in R4-2310500.

RAN1 discussed about the recommendations and guidance included in R4-2310500 and agreed on the following observations.

Concerning the recommendation of enabling UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when configured duty cycle is exceeded:

- RAN1 understands it as related to a PHR reporting enhancement by means of which Power class fallback ΔPPowerClass is reported by UE with aperiodic PHR as discussed in R4-2303560, i.e., the WF brought to RAN1’s attention by RAN4 with R4-2303701, Reply LS on enhancements to realize increasing UE power high limit for CA and DC.

- The duty cycle exceedance is referred to by RAN4 as “occasion of the report”. RAN1 understands that this expression refers to the event that triggers the aperiodic PHR report, and not to the actual UL resource to send the MAC-CE carrying the report, which would be still subject to UL resource availability as per RAN1 specification.

- RAN1 does not see a RAN1 impact for this enhancement.

Furthermore, RAN1 agreed on respectfully ask to RAN4 the following questions:

- Q1:  It is RAN1 understanding that ΔPPowerClass can be triggered by the cases when the percentage of uplink symbols transmitted in a certain evaluation period is larger than a certain duty cycle as specified in Clause 6.2.4 of TS 38 101-1. Could RAN4 clarify whether all these cases can trigger ΔPPowerClass reporting in PHR MAC CE?

- Q2: In case of duty cycle exceedance, and resulting ΔPPowerClass reporting as per recommendation in R4-2310500, is a further ΔPPowerClass reporting also allowed when UE returns to advertised PC power capabilities?

- Q3: Could RAN4 confirm the correctness of RAN1’s understanding as per observation b) concerning the recommendation of enabling UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when duty cycle is exceeded?

- Q4:  Could RAN4 clarify the meaning of the recommendation related to the combination of the ΔPPowerClass report with full-power MIMO transmission capability reporting corresponding to the current power class?

# 2	Actions

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to provide answers to questions Q1 to Q4.

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to consider the above in its further work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #114bis	October 9th – 13th, 2023,		Xiamen, China

TSG-RAN WG1 Meeting #115	November 13th – 17th, 2023,		Chicago, USA