3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315027

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.27.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On reply LS to R1-2308561 on UL power enhancements for CA and DC

Document for:	Approval

# Introduction

This paper addresses four questions enclosed in RAN1 LS of [1].

# Discussion

Question 1, 2 and 3

In our view, Q1, 2 and 3 have been already addressed by an LS of [2] approved in RAN4#108. Perhaps, it may be better to share further clarification on the Q1. Though “all these cases” in Q1 may not be crystal clear here, “the cases” intended in RAN4 corresponds to following part except for texts in yellow.

Given that what RAN1 referred to in their LS is a clause for configured transmitted power for single carrier operation, it would be better to clarify again that the same principle applies to NR CA and MR-DC.

Possible answer for Q1

RAN4 confirms that the cases to enable UE to report ΔPPowerClass are limited to occasions when maximum transmission power changes originating from a duty cycle mechanism. This principle applies to not only single carrier case, but also multiple carrier case such NR CA and MR-DC.

Regarding Q2 & 3, We believe that the LS approved in RAN4#108 perfectly addressed them. Hence, no detailed answers are required.

Possible answer for Q2 & 3

Regarding Q2 & 3, RAN4’s answers are Yes and No, respectively. The details can be found in R4-2314728.

Question 4

In our understanding, this originates from a t-doc of [3]. Simply put, a suitable ul-FullPowerTransmission can be different depending on relation between ΔPPowerClass value and an advertised power class by a UE. More specifically, assuming a UE supporting a band with PC1.5(29 dBm) for a band, e.g., n41, the UE may support ul-FullPwrMode1-r16 where the UE can achieve full power with TPMI = 2, i.e., 1/√2[1 1]T, since PC1.5 for n41 performance requirements were developed with the assumption that PC2 capable PA x 2 implementation. The same UE, however, may reduce achievable maximum transmission power by e.g., 3 dB from the advertised PC1.5 due to an uplink duty cycle exceedance or return to the achievable maximum transmission power to the advertised PC1.5 from 26 dBm. Though ul-FullPwrMode1-r16 may be suitable during a period of ΔPPowerClass being 0 dB, not ul-FullPwrMode1-r16 but rather ul-FullPwrMode-r16 may be more suitable during a period of  ΔPPowerClass being 3 dB as depictured in Figure 1.

Figure 1: relation between ΔPPowerClass transitions and Fullpowermodes

Consequently, possible RAN4’s answer toward Q4 is as follows.

Possible answer for Q4

Concerning Q4, RAN4’s intention was UE is allowed to report a more suitable mode for ul-FullPowerTransmission depending on ΔPPowerClass. More specifically, if e.g., a UE supports PC1.5 with ul-FullPwrMode1-r16 is allowed to indicate an additional capability, e.g., ul-FullPwrMode1-r16 which is applicable to only ΔPPowerClass = 3 dB or 6 dB duration where achievable maximum transmission power is capped by 26 dBm or 23 dBm, respectively.

# Conclusion

This paper addressed the four questions enclosed in RAN1 LS of [1] and shared following possible answers. A companion LS is drafted in an Annex.

Possible answer for Q1

RAN4 confirms that the cases to enable UE to report ΔPPowerClass are limited to occasions when maximum transmission power changes originating from a duty cycle mechanism. This principle applies to not only single carrier case, but also multiple carrier case such NR CA and MR-DC.

Possible answer for Q2 & 3

Regarding Q2 & 3, RAN4’s answers are Yes and No, respectively. The details can be found in R4-2314728.

Possible answer for Q4

Concerning Q4, RAN4’s intention was UE is allowed to report a more suitable mode for ul-FullPowerTransmission depending on ΔPPowerClass. More specifically, if e.g., a UE supports PC1.5 with ul-FullPwrMode1-r16 is allowed to indicate an additional capability, e.g., ul-FullPwrMode1-r16 which is applicable to only ΔPPowerClass = 3 dB or 6 dB duration where achievable maximum transmission power is capped by 26 dBm or 23 dBm, respectively.

# Reference

- R1-2308561, LS on further clarifications on enhancements to realize increasing UE power high limit for CA and DC, RAN1, TSG-RAN WG1 #114

- R4-2314728, LS on enhancements to realize increasing UE power high limit for CA and DC , TSG-RAN WG4 #108

- R4-2218548, Power Class fallback aspect, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #105

# Annex

3GPP TSG-RAN WG4 Meeting #108bis	R4-23xxxxx

# Xiamen, China, October 09 – October 13, 2023

# Title:	LS reply for further clarifications on enhancements to realize increasing UE power high limit for CA and DC

# Response to:	LS on further clarifications on enhancements to realize increasing UE power high limit for CA and DC (R1-2308561)

# Release:	Rel-18

# Work Item:	NR_cov_enh2-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name: Hiromasa Umeda

#### E-mail Address: hiromasa.umeda@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

RAN4 discussed the four questions included in R1-2308561 and would like to share the answers as follows.

- Q1:  It is RAN1 understanding that ΔPPowerClass can be triggered by the cases when the percentage of uplink symbols transmitted in a certain evaluation period is larger than a certain duty cycle as specified in Clause 6.2.4 of TS 38 101-1. Could RAN4 clarify whether all these cases can trigger ΔPPowerClass reporting in PHR MAC CE?

  - A1: RAN4 confirms that the cases to enable UE to report ΔPPowerClass are limited to occasions when maximum transmission power changes originating from a duty cycle mechanism. It is noted that this principle applies to not only single carrier case, but also multiple carrier case such NR CA and MR-DC.

- Q2: In case of duty cycle exceedance, and resulting ΔPPowerClass reporting as per recommendation in R4-2310500, is a further ΔPPowerClass reporting also allowed when UE returns to advertised PC power capabilities?

  - A2: Yes, it is. The details can be found in R4-2314728.

- Q3: Could RAN4 confirm the correctness of RAN1’s understanding as per observation b) concerning the recommendation of enabling UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when duty cycle is exceeded?

  - A3: No, it isn’t. The details can be found in R4-2314728.

- Q4:  Could RAN4 clarify the meaning of the recommendation related to the combination of the ΔPPowerClass report with full-power MIMO transmission capability reporting corresponding to the current power class?

  - A4: The intention of R4-2314728 was UE is allowed to report a more suitable mode for ul-FullPowerTransmission depending on ΔPPowerClass. More specifically, if e.g., a UE supports PC1.5 with ul-FullPwrMode1-r16 is allowed to indicate an additional capability, e.g., ul-FullPwrMode1-r16 which is applicable to only ΔPPowerClass = 3 dB or 6 dB duration, where achievable maximum transmission power is capped by 26 dBm or 23 dBm, respectively.

2. Actions:

To RAN2 group.

ACTION: RAN4 asks RAN2 to consider the above in its further work.

To RAN1 group.

ACTION: RAN4 asks RAN1 to consider the above in its further work if any.

3. Date of Next RAN WG4 Meetings:

3GPPRAN4#109		13 – 17 November 2023		Chicago, US

3GPPRAN4#110		26 Feb – 01 March 2024		  Athens, GR