3GPP TSG-RAN WG4 Meeting #108bis	R4-2315047

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulations for MIMO evolution BS Demodulation

Agenda item:	5.29.4.2

Document for:	Discussion

Introduction

This document provides simulations to help introduces to RAN4 some of the features defined in the work item “MIMO evolution” within Rel-18 of 5G NR[1], specifically those features which have an impact related to the BS demodulation performance requirements, this is to support our discussions provided in [2].

Discussion

The below simulations support discussions that lead to Proposal 4 in [2]

In the simulations we use the fully coherent codebook (DL Type I), with antenna configuration 8Tx8Rx FR1 and target SINR @ 10% BLER.

The channel utilized is TDLA 30-10 and results are presented for both ULA Low and ULA Medium B.

|  | Stream1 | Stream2 | Stream1 | Stream2 |
| --- | --- | --- | --- | --- |
|  | ULA Low | ULA Low | ULA MedB | ULA MedB |
| Codebook00 | -2,9 | -2,8 | 10,5 | 12,1 |
| Codebook01 | -2,9 | -2,8 | 10,5 | 12,1 |
| Codebook10 | -2,9 | -2,8 | 10,5 | 12,1 |
| Codebook11 | -2,9 | -2,8 | 10,5 | 12,1 |
| Codebook20 | -3,0 | -2,7 | 10,7 | 11,3 |
| Codebook21 | -3,0 | -2,8 | 10,8 | 11,4 |
| Codebook30 | -3,0 | -2,7 | 10,7 | 11,3 |
| Codebook31 | -3,0 | -2,8 | 10,8 | 11,4 |
| Codebook40 | -2,8 | -2,8 | 11,3 | 10,4 |
| Codebook41 | -2,8 | -2,8 | 11,5 | 10,4 |
| Codebook50 | -2,8 | -2,8 | 11,3 | 10,4 |
| Codebook51 | -2,8 | -2,8 | 11,5 | 10,4 |
| Codebook60 | -2,6 | -2,9 | 12,2 | 9,2 |
| Codebook61 | -2,6 | -2,9 | 12,3 | 9,3 |
| Codebook70 | -2,6 | -2,9 | 12,2 | 9,2 |
| Codebook71 | -2,6 | -2,9 | 12,3 | 9,3 |
| min | -3,0 | -2,9 | 10,5 | 9,2 |
| max | -2,6 | -2,7 | 12,3 | 12,1 |
| average | -2,8 | -2,8 | 11,2 | 10,8 |

We note that for ULA low channels the performance for all Precoding matrices is similar (+/-0.4dB) and perf for the 2 streams (2 Codewords) is similar (+/-0.1). However, for Med B correlation, the gaps can be 2-3dB.

In ULA Low channels all streams and codebooks show extremely similar performance, which strongly implies no layer diversity in these channels.

Conclusion

This contribution provides simulation results to support our discussion on 8Tx UL BS Demodulation, the following observations were made:

Observation 1: In ULA Low channels all streams and codebooks show extremely similar performance, which strongly implies no layer diversity in these channels.

References

- RP-223276 “MIMO Evolution for Downlink and Uplink”, RAN#98-e, December 2022.

- R4-2315046 “Discussion on MIMO evolution BS Demodulation”, Nokia, Nokia Shanghai Bell, RAN WG4 #108bis, Xiamen, Oct 23