3GPP TSG-RAN WG4 Meeting #108-Bis	R4-2315069

Xiamen, China, 9th Oct 2023 - 13th Oct 2023

Title:	NTN Doppler shift handling for frequency error

Source:	Qualcomm Incorporated

Agenda item:	8.1

Release:	Rel-18

WI: 	NR_NTN_solutions, LTE_NBIoT_eMTC_NTN_req

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4 responded [2] to RAN5 query [1] about NTN doppler handling in previous meeting. After that, RAN#101 gave guidance [3]. This paper discusses doppler handling in frequency error tests.

# 2. 	Discussion

## 2.1 Background and task

Guidance from RAN for frequency error is as follows:

- RAN tasks RAN4 to discuss and conclude in Oct meeting on

  - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications

    - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test

- No changes to the RAN4 core requirements

Our reading of that if RAN4 is tasked to define how fixed frequency offset should be tested. Fixed here means constant.

Observation 1: RAN4 is tasked to define more in detail how frequency error is tested in fixed Doppler frequency offset conditions.

In practical environment Doppler is rarely constant and never constant for longer periods of time since constant Doppler shift is observed only when location of the observer is perfectly located on the axis of the velocity vector of the signal source. This situation never happens in real satellite system since only cases where UE would be in the path of the satellite would be when UE is located in space, in the orbit of the satellite or the satellite is descending to the earth where UE is located. For NTN UE to compensate the Doppler, the information of the SAN relative velocity to the UE is needed since UE calculates the correct pre-compensation based on the relative velocity.

## 2.2	Possible setup for fixed Doppler

The UE can be informed, via SIB-19 a position and velocity of the SAN that is approaching or escaping the UE perfectly. This is illustrated in the Figure 1.

Figure 1. Illustration of the principle for one the test conditions for fixed Doppler

In order to set the position and velocity vector correctly, the UE location also needs to be known in the test, either by reading the location from the UE or then setting the UE its location.

When the SAN is set to perfectly approach or escape the UE’s location, the ephemeris information can be set to repeat the same trajectory in a same location or then the ephemeris can be set such that SAN moves in the approaching or escaping trajectory and ephemeris information is updated accordingly.

Using an actual motion based ephemeris has problems. The Maximum doppler shift considered is 24 ppm [TR 38.821] and that corresponds to 7.2 km/s velocity. Starting with 35 000 km orbit (writer is aware it is GEO orbit) it would yield 1 h and 21 min of testing time before the distance between UE and SAN would become 0. One option with this approach is to repeat the trajectory every 1h 21 mins.

Repeating the same position with non-zero velocity would make the trajectory nonphysical but this approach enjoys the convenience of stable setup but down side is that UE has to recognise that this is not physical but still needs to perform the pre-compensation and other behaviour expected by the NTN requirements.

Proposal 1: Give guidance to RAN5 about assumptions for frequency error testing with Doppler shift in the lines of above discussion.

Regarding the actual values of Doppler shift for NGSO, the range as written in TR 38.821 is from 0 to 24 ppm. Fixed values can be chosen between those.

## 2.3	Possible ran4 specification changes

The guidance [3] says “no core requirements changes” but it does not rule out specification improvements as long as requirement itself does not change. The current 38.101-5 states that

“Requirement will be verified for at least two cases of which one has zero Doppler conditions.”

The current RF requirements are applicable regardless of capability ntn-ScenarioSupport-r17 and with that, the same UE has to meet the GSO and NGSO Doppler shit pre-compensation requirements. This was conscious choice during the WI and therefore RF requirement specification was made transparent to ntn-ScenarioSupport-r17.  The discussion about this topic has included a lot of aspects comparing NGSO to GSO (and GEO in some cases) and WF [3] gives guidance for NGSO but does not mention GSO. To make the specification clear explicitly for both cases, the following change is proposed, and a CR is provided.

Proposal 2: Detail RAN4 requirements for NTN Frequency error regarding Doppler shift handling with the specification change as below:

“Requirement will be verified for at least two cases of which one has zero Doppler shift conditions. Doppler shift for GSO scenario is no larger than 0.93 ppm and for NGSO no larger than 24 ppm.”

# Conclusion

We discussed guidance from plenary on Doppler shift frequency error and made one observation and two proposals:

Observation 1: RAN4 is tasked to define more in detail how frequency error is tested in fixed Doppler frequency offset conditions.

Proposal 1: Give guidance to RAN5 about assumptions for frequency error testing with Doppler shift in the lines of above discussion.

Proposal 2: Detail RAN4 requirements for NTN Frequency error regarding Doppler shift handling with the specification change as below:

“Requirement will be verified for at least two cases of which one has zero Doppler shift conditions. Doppler shift for GSO scenario is no larger than 0.93 ppm and for NGSO no larger than 24 ppm.”

# Reference

[1]	R4-2311011 (R5-233672), “LS on clarifications for Non-Terrestrial Networks”, RAN5 (Keysight), 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, 21 – 25 August 2023

[2]	R4-2314001, “Reply LS on clarifications for Non-Terrestrial Networks”, RAN4 (Keysight Technologies UK Ltd, THALES), 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, 21 – 25 August 2023

[3]	RP-232682, “Summary of offline discussions for NTN testing”, RAN WG4 Chair, RAN WG5 Chair, 3GPP TSG-RAN Meeting #101, Bangalore, India, September 11-15, 2023

[4]	R4-2315271, “[Draft] CR for 38.101-5 Detailed Freq error Doppler conditions”, Qualcomm, 3GPP TSG-RAN4 Meeting #108-bis, Xiamen, China, 9th Oct 2023 - 13th Oct 2023