3GPP TSG-RAN WG4 Meeting #108bis	R4-2315077

Xiamen, China, October 09 – October 13, 2023

Source:	China Telecom

Title:	Discussion on PDSCH requirements for UE with 8Rx

Agenda Item:	5.3.3.1.2

Document for:	Discussion

- Introduction

In RAN4 #108 meeting, the 8Rx demodulation are discussed and the WF is approved in [1]. In this paper, we give our views on the remaining open issues.

- Discussion

CBW for PDSCH CA requirements

Status in the WF in [1]:

- Option 1: Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

  - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

  - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

- Option 2: Single bandwidth for each CC: 15kHz/10MHz for FDD and 30kHz/40MHz for TDD separately

We have the basic test criteria for CA that the ‘largest aggregated CA bandwidth combination is tested’ which has already been used for 2/4/8 Rx LTE, 2/4 Rx NR demodulation and SDR test specifications. Therefore, for 8Rx CA  requirements, we support to follow the widely used criteria to cover all the existing single CC channel bandwidth configurations, i.e.,

  - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

  - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Proposal 1: For 8Rx CA requirements, cover all the existing single CC channel bandwidth configurations:

  - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

  - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Test scenario

One single test scenario to be defined, with same test configurations as single carrier case except the bandwidth:

- Option 1: Rank8 case

- Option 2: Rank2 case

Since rank 8 transmission requirements have been covered by single CC tests, it is proposed to use rank 2 test for the CA scenario requirements which we believe is more practical even for 8Rx UEs.

Proposal 2: Use rank 2 test for the CA scenario 8Rx UE demodulation requirements.

- Conclusion

Proposal 1: For 8Rx CA requirements, cover all the existing single CC channel bandwidth configurations:

  - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

  - TDD: 15kHz SCS: 5/10/15/20/25/30/40/50 MHz

  - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Proposal 2: Use rank 2 test for the CA scenario 8Rx UE demodulation requirements.

- References

- [1] R4-2313877, WF for 8Rx UE performance requirements, Huawei, HiSilicon, RAN4 #108, Aug, 2023