# 3GPP TSG-RAN WG4 Meeting #108bis	R4-2315082

# Xiamen, China, October 09 – October 13, 2023

Title:	draft reply LS on required DCI signalling for advanced receiver on MU-MIMO scenario

Release:	Rel-18

Response to:	R1-2308598

Work Item:	NR_demod_enh3-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Jingzhou Wu

E-mail Address:	wujingzhou@chinatelecom.com

Attachments:

1. Overall Description:

RAN4 thank RAN1 for the reply LS (R1-2308598) on required DCI signalling for advanced receiver on MU-MIMO scenario.

Regarding the questions for the new DCI-based assistant signalling, RAN4 agreed the following answers:

- Question 1: Whether this new signaling in DCI is introduced in DCI format 1_2 in addition to format 1_1?

- RAN4 answer: Yes, the new DCI could be introduced in DCI format 1_2 in addition to format 1_1

- Question 2: Whether this new signaling in DCI is supported for one or more DL multi-TRP schemes?

- RAN4 answer: No, multi-TRP is not covered in the study thus the new DCI is not supported for one or more DL multi-TRP schemes.

- Question 3: Whether this new signaling in DCI is supported when the RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2?

- RAN4 answer: Yes, the new DCI is supported when the RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2.

- Question 4: Whether the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured?

- RAN4 answer: Yes, the new DCI is supported when the RRC codeBlockGroupTransmission is configured.

- Question 5: Whether the new signaling in DCI is supported when Rel-18 DMRS is configured?

- RAN4 answer: Yes, the new DCI is supported when Rel-18 DMRS is configured.

- Question 6: In the content corresponding to “Bit field mapped to index” =6, whether or not the phrase “In each individual PRB allocated to the target UE, the following condition is satisfied” should be replaced by “In each individual PRB PRG allocated to the target UE, the following condition is satisfied”?

- RAN4 answer: From the R-ML receiver processing point of view, RAN4 think the original wording is more suitable.

- Question 7: For “Bit field mapped to index” =1/2/3/4/5, does “empty PRB without co-scheduled UE” is allowed “in all the PRBs” of the target UE.

- RAN4 answer: Yes, ‘empty PRB without co-scheduled UE’ is allowed “in all the PRBs” of the target UE for “Bit field mapped to index” =1/2/3/4/5.

2.

To RAN WG1 group.

ACTION: RAN4 respectfully request RAN1 to take the above answers into account while designing the DCI-based network assistance signalling for UE advanced receiver to cancel inter-user interference for MU-MIMO scenario.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #109 	13 Nov - 17 Nov 2023

TSG-RAN WG4 Meeting #110 	26 Feb - 01 Mar 2024