3GPP TSG-RAN WG4 Meeting # 108bis                                                          R4-2315098

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion of NCR-MT feature list

Source: 	CATT

Agenda item:	5.28.1

Document for:	Approval

- Background

The NCR-MT feature list, we had the proposal that IAB-MT feature list can be considered as the starting point for NCR-MT in [2]. However, as the LA NCR-MT RF requirements reuse legacy UE requirements as agreed in last RAN4 meeting [3]. This proposal may not be applicable. In this contribution, we reviewed all of R15, R16 and R17 feature list for NCR-MT. The preliminary views are provided.

- Discussion

2.1 R15 feature list

The R15 features are reviewed in Table 1, the difference between UE and NCR-MT are highlighted.

Table 1: Preliminary views on NCR-MT R15 feature list

| Features | Index | Feature group | Mandatory/Optional for UE | Mandatory/Optional for R18 NCR-MT |
| --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | Optional with capability signalling | The same as UE |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | Mandatory without capability signalling | The same as UE |
|  | 1-3 | 64QAM for PUSCH | Mandatory without capability signalling | The same as UE |
|  | 1-4 | 256QAM for PDSCH | Mandatory with capability signalling for FR1 | Optional with capability signalling |
|  |  |  | Optional with capability signalling for FR2 | The same as UE |
|  | 1-5 | 256QAM for PUSCH | Optional with capability signalling (for both FR1 and FR2) | The same as UE |
|  | 1-6 | pi/2-BPSK for PUSCH | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | The same as UE |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 | The same as UE |
|  | 1-8 | Active BWP switching delay | Mandatory to support either type 1 or type 2 with capability signalling | Optional with capability signalling |
|  | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Optional with capability signalling | The same as UE |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | Optional with capability signalling |
|  | 1-11 | 7.5kHz UL raster shift | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory | The same as UE |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. | The same as UE |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. | Optional |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 | Optional |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 | Optional |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 | Optional |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Optional with capability signalling | The same as UE |
|  | 2-7 | Almost contiguous UL CP-OFDM | Optional with capability signalling | The same as UE |
|  | 2-8 | UE power class | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class | The same as UE |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 | The same as UE |
|  | 2-10 | Multiple frequency band indication | Mandatory without capability signalling | The same as UE |
|  | 2-11 | Modified MPR behaviour | Optional with capability signalling | The same as UE |
|  | 2-12 | Multiple NS/P-Max | Mandatory without capability signalling | The same as UE |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Optional with capability signalling. The capability signalling is absent if UE supports 50% | The same as UE |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Optional with capability signalling | The same as UE |
|  | 2-15 | Maximum uplink duty cycle for FR2 | Optional with capability signalling | The same as UE |
|  | 2-16 | PA architectures for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. | The same as UE |
|  | 2-17 | PA architectures for intra-band UL CA | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture | The same as UE |

2.2 R16 and R17 feature list

All of R16 and R17 features are optional for UE, so our understanding is that the same approach can be applied to NCR-MT.

Proposal: All of R16 and R17 features are optional for NCR-MT.

- Summary

This contribution provides our view for NCR feature list. For R15 feature list, we provide our views in Table 1. For R16 and R17 features, we have the following proposal.

Proposal: All of R16 and R17 features are optional for NCR-MT.

Reference

[1] TS 38.822

[2] R4-2311643, “Discussion on NCR feature list”, CATT, RAN4#108

[3] R4-2313903, “WF for NCR RF requirements”, ZTE, RAN4#108