3GPP TSG-RAN WG4 Meeting # 108bis                                                          R4-2315099

Xiamen, China, October 09 – October 13, 2023

Title: 	Discussion on how to handle BS RF requirements for NES

Source: 	CATT

Agenda item:	5.34.1

Document for:	Approval

- Background

There’s still no agreement on the BS RF TAE requirement in last RAN4 meeting. There’re two options which are defining or not defining the requirements. This contribution provides our further consideration.

- Discussion

The followings are the status of BS RF TAE discussion in last RAN4 meeting,

-------------------------------------------------

Issue 1-1: TAE

Agreement:

- Alternative #1:

  - Do not specify the BS TAE requirements of SSB-less operation for FR1 co-located inter-band CA

  - Define the side condition of RTD to ensure UE performance in RRM part.

- Alternative #2:

  - Specify BS TAE requirements

    - FFS on values

-------------------------------------------------

For the two alternatives, we support alternative 1 then left the BS TAE performance to the implementation.

Proposal 1: The following option is approved for the BS RF NES discussion.

- Alternative #1:

  - Do not specify the BS TAE requirements of SSB-less operation for FR1 co-located inter-band CA

  - Define the side condition of RTD to ensure UE performance in RRM part.

For the side condition of RTD, some analysis is needed. In the BS implementation, windowing is used in the Tx path digital operation to solve the out of band emission issue. So in the time domain, there’re some distortions for each OFDM symbol as shown in Figure 1. In the specification TS 38.104 and TS 38.101, the followings describe how to measure EVM,

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

is estimated so that the EVM window of length W is centred on the measured cyclic prefix of the considered OFDM symbol.

The length W for FR1 is shown in the following tables which are copied from TS 38.104.

Table B.5.2-1: EVM window length for normal CP, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| …… |  |  |  |  |
| 25 | 2048 | 144 | 72 | 50 |
| …… |  |  |  |  |

Table B.5.2-2: EVM window length for normal CP, FR1, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| …… |  |  |  |  |
| 25 | 1024 | 72 | 36 | 50 |
| …… |  |  |  |  |
| 60 | 3072 | 216 | 130 | 60 |
| …… |  |  |  |  |

Figure1: FFT window and Pcell/Scell timing

As shown in the Figure 1, the time tolerance (the lowest FFT processing window)/(the highest FFT processing window) and (the center FFT processing window) is half of the length of W. So for the Scell, when the timing for the center FFT processing window is selected, the TAE allowed is equal to half of W. The timing of the center FFT processing window for Scell can have some offset between Pcell and Scell based on the implementation. But the time tolerance is not changed when the time domain data SNR quality is taken into acount. Scell timing may be advanced or delayed of Pcell timing according to UE FFT processing window chosen strategy. The Scell timing should be selected referring to the center FFT processing window to allow the TAE for both sides.

Observation 1: The lowest time edge and the highest time edge of the FFT processing windows for the one symbol is the length of W, which is the EVM window.

Observation 2: The Scell time TAE compared with Pcell should be less than W/2 to guarantee the sampled data for FFT is complete.

Observation 3: Using center FFT processing window for the Pcell as the Scell FFT window seems the correct implementation to accommodate the advance or delay possibility of TAE.

According to TS 38.104, the time length of W/2 is shown in the following table,

| SCS (kHz) | 15 | 30 | 60 |
| --- | --- | --- | --- |
| CP length (us) | 4.69 | 2.34 | 1.17 |
| W/2 when W_length=40% CP (us) | 0.94 | 0.47 | 0.23 |
| W/2 when W_length=50% CP (us) | 1.17 | 0.59 | 0.29 |
| W/2 when W_length=60% CP (us) | 1.41 | 0.70 | 0.35 |

Observation 4: Half of EVM window W for 40% CP is 230 ns, 470 ns and 940 ns for 60 kHz, 30 kHz and 15 kHz SCS.

It should be noted that the above analysis only considers the EVM window analysis. If other aspects, such as multi-path, system margin, etc, shorter TAE would bring better system performance.

From the above analysis, the following side condition for UE RTD can be provided to RRM part.

Proposal 2: The side condition for UE RTD is that the maximum value of BS TAE is half of EVM window length for 40% CP length, i.e. 230 ns, 470 ns and 940 ns for 60 kHz, 30 kHz and 15 kHz SCS.

- Summary

This contribution provides our analysis and views on BS TAE requirements, UE RF impact and UE received power imbalance. We have the following observations and proposals.

Proposal 1: The following option is approved for the BS RF NES discussion.

- Alternative #1:

  - Do not specify the BS TAE requirements of SSB-less operation for FR1 co-located inter-band CA

  - Define the side condition of RTD to ensure UE performance in RRM part.

Observation 1: The lowest time edge and the highest time edge of the FFT processing windows for the one symbol is the length of W, which is the EVM window.

Observation 2: The Scell time TAE compared with Pcell should be less than W/2 to guarantee the sampled data for FFT is complete.

Observation 3: Using center FFT processing window for the Pcell as the Scell FFT window seems the correct implementation to accommodate the advance or delay possibility of TAE.

Observation 4: Half of EVM window W for 40% CP is 230 ns, 470 ns and 940 ns for 60 kHz, 30 kHz and 15 kHz SCS.

Proposal 2: The side condition for UE RTD is that the maximum value of BS TAE is half of EVM window length for 40% CP length, i.e. 230 ns, 470 ns and 940 ns for 60 kHz, 30 kHz and 15 kHz SCS.

Reference

[1] R4-2314935, “WF on RF requirements for NES”, Huawei, RAN4#108