3GPP TSG-RAN WG4 Meeting # 108-bis	            R4-2315116

Xiamen, CN, Oct 09th – Oct 13rd, 2023

Agenda Item:	5.24.2.4

Source: 	CATT

Title: 	Discussion on enhanced CHO configurations

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on NR Mobility Enhancements RRM requirements (part 2) was approved [1]. In this contribution, some open issues on enhanced CHO configurations are further discussed.

2. Discussion

CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

Tmeasure

In the last meeting, the candidate solutions for Tmeasure are duplicated below:

- Candidate solutions: - Option 1: Tmeasure for CHO with CPA/C shall be updated to T target cell_SSB_index_identify, which is from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. (E///) - Option 2: (Nokia) - Both when CHO-only configuration is provided and when it is not provided, in the definition of Tmeasure_CHOwithCPAC, take into account that the UE continues to evaluate CPA/CPC condition until CHO condition is met even if the condition is met for some candidate PSCell before this. In this case, the duration of Tmeasure_CHOwithCPAC = Tmeasure_CHOwithCPAC_PCell - When CHO-only configuration is not provided, define Tmeasure_CHOwithCPAC to take into account that the UE continues to evaluate CHO and CPA/CPC conditions until they are both met. In this case, the duration of Tmeasure_CHOwithCPAC = max(Tmeasure_CHOwithCPAC_PCell, Tmeasure_CHOwithCPAC_PSCell).

In the last meeting, PCell/ PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4) has been agreed as follows:

- Agreement:

  - When the UE is NOT provided with CHO-only configuration

    - DCHOwithCPAC_PCell = TRRC + TEvent_DU + max(Tmeasure_CHO, Tmeasure_CPA/CPC) + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

      - Tmeasure for CHO is used as baseline for Tmeasure_CHO

      - Tmeasure for CPA/CPC is used as baseline for Tmeasure_CPA/CPC

For Tmeasure_CHO and Tmeasure_CPA/CPC, Tmeasure for CHO and for CPA/CPC is used as baseline, that is, the measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. Compared with Option 1 and Option 2, we prefer the legacy description.

Proposal 1: Tmeasure for CHO and for CPA/CPC is used as baseline, that is, the measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

3. Conclusion

In this paper, we provide our views on enhanced CHO configurations. From this discussion we have derived the following proposal:

Proposal 1: Tmeasure for CHO and for CPA/CPC is used as baseline, that is, the measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

4. Reference

- R4-2314332 WF on NR Mobility Enhancements RRM requirements (part 2), Apple