3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315166

Xiamen, China, October 09 - October 13, 2023

Agenda Item:	5.13.6

Source: 	CMCC

Title: 			 Discussion on measurement accuracy and test cases for ATG

Document for:	Discussion

- Introduction

ATG RRM core requirements were discussed during RAN4#104 to RAN4#108. According to the TU allocation, RAN4 is supposed to start the performance part for the WI. In this contribution, we will provide our views on the measurement accuracy and test cases for ATG.

- Discussion

  - Measurement Accuracy

Basically, there are two factors that affect the measurement accuracy, measurement samples and side conditions.

For ATG normal measurement in both RRC_IDLE and RRC_CONNECTED, the measurement samples are same as in legacy TN network. For ATG enhanced measurement in RRC_IDLE/INACTIVE, the measurement samples are same as in HST requirement.

For the side condition of Es/Iot, we think it can be re-used from TN.

In NTN, the measurement accuracy will not be impact by timing/frequency error of UE pre-compensation. For timing error, comparing with NTN, only UE GNSS error (40m, smaller than 50m in NTN) need to be considered in ATG network. For frequency error, maximum hundreds Hz residual Doppler is on the same level as in TN mobility scenario. Therefore, we think the measurement accuracy will not be impact by timing error of ATG UE timing/frequency pre-compensation.

Based on the observation and discussion above, we think the Es/Iot side condition and measurement accuracy of legacy TN network can be reused for ATG network.

Proposal 1: Reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network for ATG.

  - ATG test cases

Test configuration

In R18 ATG, RAN4 only consider 15kHz SCS for FDD, 30kHz SCS for TDD. Therefore, we propose two general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Proposal 2: Consider two general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Test case coverage

Two types of UE are considered in the core spec, which are UE with omnidirectional antennas and UE with antenna array. UE with omnidirectional antennas are assumed for both TDD and FDD case, UE with antenna array are assumed for TDD case. Therefore, we propose follow this assumption when define the RRM test cases.

Proposal 3: Define FDD and TDD test cases and test requirements for UE with omnidirectional antennas. Define TDD test cases and test requirements for UE with antenna array.

For the exact TCs, based on the core requirements, we think the following TCs should be defined for ATG.

Proposal 4: Define following test cases for ATG:

| Test scope RRC_IDLE state mobility | Test number | Note |
| --- | --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 | Neighbour cell reference BS location is provided |
| Location based cell reselection to intra-frequency neighbour cell | ATG 1-2 |  |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 | Neighbour cell reference BS location is not provided |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 | Neighbour cell reference BS location is provided |
| Location based cell reselection to inter-frequency neighbour cell | ATG 1-5 |  |

| Test scope RRC_CONNECTED state mobility | Test number | Note |
| --- | --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 | N/A |
| Intra-frequency HO with unknown target cell | ATG 2-2 | Neighbour cell reference BS location is provided |
| Inter-frequency HO with known target cell | ATG 2-3 | N/A |
| Inter-frequency HO with unknown target cell | ATG 2-4 | Neighbour cell reference BS location is not provided |
| Intra-frequency location based CHO | ATG 2-5 | Neighbour cell reference BS location is not provided |
| Inter-frequency location based CHO | ATG 2-6 | Neighbour cell reference BS location is provided |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 | N/A |
| Intra-frequency RRC Re-establishment with unknown target cell | ATG 2-8 | Neighbour cell reference BS location is not provided |
| Inter-frequency RRC Re-establishment with known target cell | ATG 2-9 |  |
| Inter-frequency RRC Re-establishment with unknown target cell | ATG 2-10 | Neighbour cell reference BS location is provided |
| 4-step RA type contention based random access test | ATG 2-11 | N/A |
| 4-step RA type Non-Contention based random access test | ATG 2-12 | N/A |
| 2-step RA type contention based random access test | ATG 2-13 | N/A |
| 2-step RA type Non-Contention based random access test | ATG 2-14 |  |
| RRC Connection Release with Redirection | ATG 2-15 | Neighbour cell reference BS location is provided |

| Test scope Timing | Test number | Note |
| --- | --- | --- |
| UE transmit timing test | ATG 3-1 | N/A |
| Timing advance adjustment accuracy | ATG 3-2 | N/A |

| Test scope Signalling characteristics | Test number | Note |
| --- | --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 | N/A |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-3 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-4 | N/A |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-5 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-6 | N/A |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-7 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-8 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-9 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | ATG 4-10 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-11 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode | ATG 4-12 | N/A |
| DCI-based and Timer-based Active BWP Switch | ATG 4-13 | N/A |
| RRC-based Active BWP Switch | ATG 4-14 | N/A |
| UE specific CBW change | ATG 4-15 | N/A |
| Pathloss reference signal switching delay | ATG 4-16 | N/A |

| Test scope Measurement procedure | Test number | Note |
| --- | --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 | Neighbour cell reference BS location is provided |
| Intra-frequency measurements event triggered reporting tests without gap under DRX | ATG 5-2 | Neighbour cell reference BS location is not provided |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-3 | Neighbour cell reference BS location is not provided |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under DRX | ATG 5-4 | Neighbour cell reference BS location is provided |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-5 | Neighbour cell reference BS location is provided |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-6 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | ATG 5-7 | Neighbour cell reference BS location is not provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is used | ATG 5-8 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-9 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is used | ATG 5-10 | Neighbour cell reference BS location is not provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is used | ATG 5-11 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-12 | Neighbour cell reference BS location is provided |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-13 | N/A |
| SSB based L1-RSRP measurement when DRX is used | ATG 5-14 | N/A |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-15 | N/A |
| CSI-RS based L1-RSRP measurement when DRX is used | ATG 5-16 | N/A |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used | ATG 5-17 | N/A |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-18 | N/A |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-19 | N/A |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-20 | N/A |

| Test scope Measurement Performance requirements | Test number | Note |
| --- | --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 | N/A |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 | N/A |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 | N/A |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 | N/A |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 | N/A |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 | N/A |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 | N/A |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 | N/A |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 | N/A |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 | N/A |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 | N/A |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 | N/A |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 | N/A |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 | N/A |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 | N/A |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 | N/A |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 | N/A |

Test case design

UE mobility

For the test of location-based cell re-selection, location-based CHO and UL transmit timing tests, the UE mobility should be assumed with 1200km/h. For the other tests, UE could be assumed with no mobility.

Proposal 5: For the location-based cell re-selection tests, location-based CHO tests and UL transmit timing tests, the UE mobility should be assumed with 1200km/h. For the other tests, UE could be assumed with no mobility.

The number of neighbour cell

For cell re-selection test and intra/inter-frequency measurement test, in the case of FDD configuration, 1 neighbour cell is enough to test UE behavior and requirements for UE with omnidirectional antenna. However, in the case of TDD configuration, we should choose some test cases to set 2 neighbour cells  to test UE behavior and requirements for UE with antenna array.

Proposal 6: For the FDD cell re-selection test and intra/inter-frequency measurement test for UE with omnidirectional antenna, configure 1 neighbour cell. For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells.

Channel model

In the discussion of demod session, the AWGN with residual doppler channel model is used for ATG scenario, we proposal to keep the same design and use the AWGN with residual doppler channel model in RRM test cases.

Proposal 7: Use the AWGN with residual doppler channel model for RRM test cases.

GNSS acquirement

Similar GNSS acquirement method as NTN can be reused. However, the issue is still under discussion of NTN WI, we can wait more progress.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network for ATG.

Proposal 2: Consider two general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Proposal 3: Define FDD and TDD test cases and test requirements for UE with omnidirectional antennas. Define TDD test cases and test requirements for UE with antenna array.

Proposal 4: Define following test cases for ATG:

| Test scope RRC_IDLE state mobility | Test number | Note |
| --- | --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 | Neighbour cell reference BS location is provided |
| Location based cell reselection to intra-frequency neighbour cell | ATG 1-2 |  |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 | Neighbour cell reference BS location is not provided |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 | Neighbour cell reference BS location is provided |
| Location based cell reselection to inter-frequency neighbour cell | ATG 1-5 |  |

| Test scope RRC_CONNECTED state mobility | Test number | Note |
| --- | --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 | N/A |
| Intra-frequency HO with unknown target cell | ATG 2-2 | Neighbour cell reference BS location is provided |
| Inter-frequency HO with known target cell | ATG 2-3 | N/A |
| Inter-frequency HO with unknown target cell | ATG 2-4 | Neighbour cell reference BS location is not provided |
| Intra-frequency location based CHO | ATG 2-5 | Neighbour cell reference BS location is not provided |
| Inter-frequency location based CHO | ATG 2-6 | Neighbour cell reference BS location is provided |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 | N/A |
| Intra-frequency RRC Re-establishment with unknown target cell | ATG 2-8 | Neighbour cell reference BS location is not provided |
| Inter-frequency RRC Re-establishment with known target cell | ATG 2-9 |  |
| Inter-frequency RRC Re-establishment with unknown target cell | ATG 2-10 | Neighbour cell reference BS location is provided |
| 4-step RA type contention based random access test | ATG 2-11 | N/A |
| 4-step RA type Non-Contention based random access test | ATG 2-12 | N/A |
| 2-step RA type contention based random access test | ATG 2-13 | N/A |
| 2-step RA type Non-Contention based random access test | ATG 2-14 |  |
| RRC Connection Release with Redirection | ATG 2-15 | Neighbour cell reference BS location is provided |

| Test scope Timing | Test number | Note |
| --- | --- | --- |
| UE transmit timing test | ATG 3-1 | N/A |
| Timing advance adjustment accuracy | ATG 3-2 | N/A |

| Test scope Signalling characteristics | Test number | Note |
| --- | --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 | N/A |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-3 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-4 | N/A |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-5 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-6 | N/A |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-7 | N/A |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-8 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-9 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | ATG 4-10 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-11 | N/A |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode | ATG 4-12 | N/A |
| DCI-based and Timer-based Active BWP Switch | ATG 4-13 | N/A |
| RRC-based Active BWP Switch | ATG 4-14 | N/A |
| UE specific CBW change | ATG 4-15 | N/A |
| Pathloss reference signal switching delay | ATG 4-16 | N/A |

| Test scope Measurement procedure | Test number | Note |
| --- | --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 | Neighbour cell reference BS location is provided |
| Intra-frequency measurements event triggered reporting tests without gap under DRX | ATG 5-2 | Neighbour cell reference BS location is not provided |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-3 | Neighbour cell reference BS location is not provided |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under DRX | ATG 5-4 | Neighbour cell reference BS location is provided |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-5 | Neighbour cell reference BS location is provided |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-6 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | ATG 5-7 | Neighbour cell reference BS location is not provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is used | ATG 5-8 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-9 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is used | ATG 5-10 | Neighbour cell reference BS location is not provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is used | ATG 5-11 | Neighbour cell reference BS location is provided |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-12 | Neighbour cell reference BS location is provided |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-13 | N/A |
| SSB based L1-RSRP measurement when DRX is used | ATG 5-14 | N/A |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-15 | N/A |
| CSI-RS based L1-RSRP measurement when DRX is used | ATG 5-16 | N/A |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used | ATG 5-17 | N/A |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-18 | N/A |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-19 | N/A |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-20 | N/A |

| Test scope Measurement Performance requirements | Test number | Note |
| --- | --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 | N/A |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 | N/A |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 | N/A |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 | N/A |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 | N/A |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 | N/A |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 | N/A |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 | N/A |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 | N/A |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 | N/A |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 | N/A |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 | N/A |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 | N/A |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 | N/A |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 | N/A |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 | N/A |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 | N/A |

Proposal 5: For the location-based cell re-selection tests, location-based CHO tests and UL transmit timing tests, the UE mobility should be assumed with 1200km/h. For the other tests, UE could be assumed with no mobility.

Proposal 6: For the FDD cell re-selection test and intra/inter-frequency measurement test for UE with omnidirectional antenna, configure 1 neighbour cell. For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells.

Proposal 7: Use the AWGN with residual doppler channel model for RRM test cases.

- Reference

- R4-2314307, WF on NR ATG RRM requirements, CMCC