3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315170

Xiamen, China, October 09 - October 13, 2023

Agenda Item:	5.13.7.3

Source: 	CMCC

Title: 			 Discussion on BS demodulation requirements for ATG scenario

Document for:	Discussion

- Introduction

In this contribution, we further provide our views on test PUSCH test scope and TDD patterns related issues.

- PUSCH requirements

Issue 1-2: TDD pattern 30D4S6U

Agreement:

- Wait RAN1 response. If RAN1 response that “Increasing the number of HARQ processes” and “K1 range extension” issue can be solved by existing NTN solution, consider to define corresponding demodulation requirements with the new TDD pattern and note should be added in the specification that this pattern is for ATG scenario only, these requirements should be optional based on UE capability

Issue 1-3: Applicability rule for TDD pattern

- Option 1: the legacy TDD pattern can be skipped if the test of new TDD pattern is passed;

- Other options not precluded

Based on RAN1’s reply LS [2], the functionality corresponding to UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ can be used for Rel-18 NR ATG. New ATG UE features of ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ should be introduced in Rel-18.

Therefore, the TDD pattern 30D4S6U could be supported by UE supports new ATG UE features of ‘Increasing the number of HARQ processes’ and ‘K1 range extension’. We strongly propose to define the test cases to test whether the BS could configure such TDD pattern correctly since it is not typical pattern in legacy. To reduce the test effort, the legacy TDD pattern can be skipped if the test of new TDD pattern is passed.

Proposal 1: For new incremental PUSCH requirements, introduce test cases of new TDD pattern configuration 30D4S6U.

Proposal 2: The legacy TDD pattern can be skipped if the test of new TDD pattern is passed.

Issue 3-3: Test scope for PUSCH

Agreement:

- For new incremental requirements:

  - The same requirements are applicable to TDD with different UL-DL pattern

- For legacy requirements reusing:

  - Consider normal PUSCH demodulation and UCI multiplexing on PUSCH as mandatory

Due to the channel model is different for FDD and TDD, how to define the requirements for FDD and TDD cases is still unclear.

Based on the current average simulation results among companies, the max SNR gap between FDD and TDD is about 0.42dB. We think different channel model and different requirements should be apply for FDD and TDD separately.

- New PUSCH test cases for ATG with FDD can be as follows.

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- |
|  |  | 256QAM (MCS 22 in table 2) |

- New PUSCH test cases with static TDD slot pattern can be as follows (if introduced):

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & 30D4S6U | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- | --- |
|  |  |  | 256QAM (MCS 22 in table 2) |

Proposal 3: Introduce following new dedicated requirements for PUSCH, which are separately for FDD and TDD cases.

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- |
|  |  | 256QAM (MCS 22 in table 2) |

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & 30D4S6U | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- | --- |
|  |  |  | 256QAM (MCS 22 in table 2) |

- Others

Issue 1-5: Specification impact

For ATG BS demodulation requirements

- Option 1: Add a new section for ATG PUSCH demodulation requirements. In this section, clarifications for how to reuse legacy applicability rules and requirements should be added, and new defined PUSCH demodulation requirements could be captured.

- Option 2:

  - New clause for ATG new incremental PUSCH requirements should be introduced in 38.104

  - New clause for ATG applicability of PUSCH requirements should be introduced in 38.141-1 and 38.141-2

  - New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

  - New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- Option 3: Capture ATG demodulation requirement into the same section with legacy requirement to minimize the effort of specification modification with adding the referring statement as “The following requirements in sections of 8.2.1 and 8.2.3 can be applied for BS declared to support ATG scenario”. New dedicated requirement can be added into the corresponding table in section 8.2.1

We prefer Option 2, which is more straightforward and forward compatible.

New clause for ATG new incremental PUSCH requirements should be introduced in 38.104, for instance:

8.2.x Requirements for PUSCH with transform precoding disabled for ATG 11.2.1.x Requirements for PUSCH with transform precoding disabled for ATG

New clause for ATG applicability of PUSCH requirements should be introduced in 38.141-1 and 38.141-2, for instance:

8.1.2.x1 Applicability of PUSCH performance requirements for ATG Unless otherwise stated, for BS (type 1-O) which declaring support ATG, the requirements for PUSCH with transform precoding disabled for ATG in 8.2.x of 38.104, requirements for PUSCH with transform precoding disabled demodulation in Clause 8.2.1 (11.2.1.1) and the requirements for UCI multiplexing on PUSCH in Clause 8.2.3 (11.2.1.3) shall be mandatory tested. 8.1.2.x1.1 Applicability of requirements for TDD with different UL-DL patterns Unless otherwise stated, for 30kHz declared to be supported, if BS (type 1-O) supports multiple TDD UL-DL patterns including TDD pattern 30D4S6U, only the supported TDD UL-DL pattern 30D4S6U shall be used for all tests.

New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2, for instance:

8.1.2.x2 Applicability of PUCCH performance requirements for ATG Unless otherwise stated, for BS (type 1-O) which declaring support ATG, the performance requirements for PUCCH in 8.3 (11.3.1) are applied.

New clause for ATG applicability of PRACH requirements should be introduced in 38.141-1 and 38.141-2, for instance:

8.1.2.x3 Applicability of PRACH performance requirements for ATG Unless otherwise stated, for BS (type 1-O) which declaring support ATG, the performance requirements for PRACH in 8.4 (11.4.1) are applied.

Proposal 4:

- New clause for ATG new incremental PUSCH requirements should be introduced in 38.104

- New clause for ATG applicability of PUSCH requirements and Applicability of requirements for TDD with different UL-DL patterns should be introduced in 38.141-1 and 38.141-2

- New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For new incremental PUSCH requirements, introduce test cases of new TDD pattern configuration 30D4S6U.

Proposal 2: The legacy TDD pattern can be skipped if the test of new TDD pattern is passed.

Proposal 3: Introduce following new dedicated requirements for PUSCH, which are separately for FDD and TDD cases.

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- |
|  |  | 256QAM (MCS 22 in table 2) |

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & 30D4S6U | 1T2R | 64QAM (MCS 28 in table 1) |
| --- | --- | --- | --- |
|  |  |  | 256QAM (MCS 22 in table 2) |

Proposal 4:

- New clause for ATG new incremental PUSCH requirements should be introduced in 38.104

- New clause for ATG applicability of PUSCH requirements and Applicability of requirements for TDD with different UL-DL patterns should be introduced in 38.141-1 and 38.141-2

- New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2

- Reference

[1] R4-2313875, WF for ATG demodulation requirements, CMCC

[2] R1-2308531, Reply LS on UE features for NR ATG, CMCC

- [3] R4-2315171, Simulation results for ATG PUSCH demodulation, CMCC