3GPP TSG-RAN WG4 Meeting # 108bis		R4-2315174

Xiamen, China, October 09 - October 13, 2023

Agenda Item:	5.34.3.1

Source: 	CMCC

Title: 			 Discussion on cell DTX/DRX for network energy saving

Document for:	Discussion

- Introduction

Up to last meeting, RAN1 has achieved key agreements about cell DTX and DRX [1]. In this contribution, we discuss the RRM requirement due to cell DTX/DRX and other RAN1 designs.

- Discussion

  - Cell DTX/DRX

Issue 2-1-1: RRM impacts of Cell DTX/DRX – general

- Proposals

  - Proposal 1: If RAN1 concludes that the RS for RRM measurement (e.g RRM/RLM/BFD/BM) is not impacted by cell DTX, there is no RRM impacts for objective#2 (Huawei)

  - Proposal 2: to indicate RAN1 that: (Apple)

    - Based on network configuration, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX:

    - PRS

    - CSI-RS configured by measObjectNR (for RRM)

    - CSI-RS associated with RLM, BFD, CBD

    - Periodic/Semi-persistent CSI-RS for beam measurement

    - FFS: PDCCH in USS to be decided by RAN1/2

    - FFS: PDCCH in Type-3 CSS to be decided by RAN1/2

To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

  - Proposal 3:  For cell DTX/DRX, the impact to RRM requirements is not necessary. By gNB implementation, adjustment of CSI-RS measurement period by RRC is always possible, and therefore no impact to spec is needed. UE assumes CSI-RS occasions are still available if the CSI-RS periodicity/offset configuration indicates the availability of CSI-RS during cell DTX in-active period (Vivo)

Based on RAN1’s progress, following signaling is not expected to transmit by network or UE during cell DTX and DRX:

From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS

For other signaling/channels, the transmission and reception will not be impacted by cell DTX/DRX condition. Hence, RRM requirements will not be impacted by cell DTX/DRX.

Proposal 1: RRM requirements will not be impacted by cell DTX/DRX.

  - Spatial and power domain techniques

Issue 2-2: RRM impacts of Spatial and power domain techniques

- Proposals

  - Proposal 1: If spatial and power domain adaptation is not considered for BM enhancement in RAN1, no RAN4 RRM impact is expected for objective#3.

- In RAN1#114 meeting, the following conclusion has been achieved:

- No further work on BM enhancements for R18 NES

- Therefore, no RRM impact is expected for spatial and power domain techniques.

Proposal 2: No RRM impact is expected for spatial and power domain techniques.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: RRM requirements will not be impacted by cell DTX/DRX.

Proposal 2: No RRM impact is expected for spatial and power domain techniques.

- Reference

[1] Draft Report of 3GPP TSG RAN WG1 #114 v0.2.0, RAN WG1

[2] R4-2314381, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.