# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315191

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.13.1.4

Source: 	CMCC

Title: 	Discussion on ATG non-synchronized simulation results

Document for:	Discussion

# 1. Introduction

In previous meeting, A WF on ATG remaining issues is approved. According to the timeline, in this meeting, we need to provide detailed simulation results for the non-synchronization operation between TN gNB and ATG gNB. In this contribution, we show our simulation results.

Following list the layout illustration for information.

Fig 4: front view of non-synchronized scenarios case 2

Fig 5: top view of non-synchronized scenarios case 2

# 2. Simulation parameters

Previous agreements in TR is to use free space propagation model for ATG gNB-to TN gNB CLI evaluation. However, as shown in previous meeting, final isolated distance may be larger than 100km. For such long distance, it may be not reasonable to still assume free space modeling. From this point of view, it’s better list other options besides free space model.

During the discussion of SBFD simulation, legacy UMa path loss model is referred with updating hUT as 25m, the height of gNB. If we refer to this principle, then we could use path loss model in TR 38.901 by updating hUT as 30m when evaluate ATG gNB-to- TN gNB CLI.

During ITU terrestrial evaluation, usually ITU-R P.452 model is assumed which take factors like LOS propagation, Diffraction, Tropospheric scatter, ducting/layer reflection, additional clutter losses into consideration. the baseline LOS propagation is to use the sum of free space and attenuation by atmospheric gases, the latter one is several dB and less than free space loss.

Observation 1: we have following three options for path loss model when evaluate ATG gNB-to-TN gNB CLI.

- Option 1: Free space as listed in current TR

- Option 2: Using RMa in TR 38.901 by updating hUT as 30m

- Option 3: Using model in ITU-R P.452

- All above options are OK for us, but considering we don’t have enough time for simulation and final simulation results i.e. isolated distance will not impact final RF requirements and just for information, it’s not mandate all companies to use the same path loss model. Final results are contribution driven. All provided isolation distance simulation results will be captured into final TR for information with explicitly state corresponding path loss model.

- Proposal 1: there is no mandatory path loss model when evaluating ATG gNB-to-TN gNB CLI. All simulation results will be captured into final TR with explicitly state its corresponding path loss model provided final non-synchronized simulation results will only for information without impacting RF requirements.

# 2. Simulation results

2.1 Results based on TR RMa model with updating hUT as 30m

The propagation model between ATG BS and TN BS is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1 (shadow fading and clutter loss are assumed to be zero, since the ATG BS and TN BS are at 30m above the clutter).

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null steering and lead to less interference in our simulation.

2.1.1 Scenario 5: 4GHz ATG DL interfering TN UL

Following table list our simulation results.

Table 1: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | 17 | 11 | 9 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 12 | 7 | 6 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Observation 2: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [6, 17]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 1.

2.1.2 Scenario 7: 4GHz TN DL interfering ATG UL

Following table list our simulation results.

Table 2: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | 16 | 9 | 8 |
|  |  | Average of all users in the whole network | 11 | 7 | 5 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Observation 3: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [5, 16]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 2.

2.1.3 Scenario 14: 2GHz TN DL interfering ATG UL

Following table list our simulation results.

Table 3: Simulation results for Scenario 13 – 2GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | 18 | 12 | 10 |
|  |  | Average of all users in the whole network | 13 | 0 | 6 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Observation 4: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [6, 18]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 3.

2.2. Results based on free space model

Noted: the reason that 30 degree will lead to less isolation distance is because at 30degree direction, antenna pattern is almost null steering and lead to less interference in our simulation.

2.2.1 Scenario 5: 4GHz ATG DL interfering TN UL

Following table list our simulation results.

Table 4: Simulation results for Scenario 5 – 4GHz ATG DL interfering TN UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim | >400 | 50 | 35 |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim | 200 | 30 | 15 |
|  | Subarray | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |
|  |  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |  |

Observation 5: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [15, >400]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 4.

2.2.2 Scenario 7: 4GHz TN DL interfering ATG UL

Following table list our simulation results.

Table 5: Simulation results for Scenario 7 – 4GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | >500 | 75 | 55 |
|  |  | Average of all users in the whole network | >300 | 45 | 25 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Observation 6: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [25, >600]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 5.

2.2.3 Scenario 14: 2GHz TN DL interfering ATG UL

Following table list our simulation results.

Table 6: Simulation results for Scenario 13 – 2GHz TN DL interfering ATG UL

| Company | ATG/ TN BS antenna model | Performance Metric | Isolation distance (km) for 5% throughput loss |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Angle between ATG BS boresight and nearest TN BS boresight in azimuth |  |  |
|  |  |  | 0° | 30° | 60° |
| CMCC | Non-Subarray | 5% in the whole network | >500 | >100 | 80 |
|  |  | Average of all users in the whole network | 500 | 75 | 35 |
|  | Subarray | 5% in the whole network |  |  |  |
|  |  | Average of all users in the whole network |  |  |  |

Observation 7: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [35, >500]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 6.

# 3. Conclusions

In this contribution, we have following observations and proposals.

Observation 1: we have following three options for path loss model when evaluate ATG gNB-to-TN gNB CLI.

- Option 1: Free space as listed in current TR

- Option 2: Using RMa in TR 38.901 by updating hUT as 30m

- Option 3: Using model in ITU-R P.452

- Proposal 1: there is no mandatory path loss model when evaluating ATG gNB-to-TN gNB CLI. All simulation results will be captured into final TR with explicitly state its corresponding path loss model provided final non-synchronized simulation results will only for information without impacting RF requirements.

Observation 2: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [6, 17]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 1.

Observation 3: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [5, 16]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 2.

Observation 4: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [6, 18]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 3.

Observation 5: for 4GHz ATG DL interfering TN UL case, isolation distance is in the range [15, >400]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 4.

Observation 6: for 4GHz TN DL interfering ATG UL case, isolation distance is in the range [25, >600]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 5.

Observation 7: for 2GHz TN DL interfering ATG UL case, isolation distance is in the range [35, >500]km for 0 degree, 30 degree and 60 degree. Detailed simulation results are in table 6.