# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315194

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.20.2

Source: 	CMCC

Title: 	discussion on LP-WUS ACS and ASCS requirements

Document for:	Discussion

# 1. Introduction

In RAN4 #108 meeting, one WF[1] and LS[2] are approved to help further discussion. Besides, it’s approved that both filter suppression performance and link level simulation are allowed to analyze guard band. In this contribution, we focus on the discussion of ACS and ASCS related issues.

- Discussion

2.1 Guard RB evaluation for ACS based on filter characteristics

Last meeting agreement is listed as below for information:

Issue 1-1-3: Required number of guard RBs for LP-WUS ACS Agreement: - Companies provide the analyzed results with the RF impairment assumptions. RAN4 target to make decision on required number of guard RBs next meeting. - RF impairments and power cost impacts can be claimed by companies used in the analysis - For each RF impairment could be reported for different RF architecture - Companies are encouraged to provide the text proposals for RAN4 RF architecture evaluation.

Following list our filter evaluation analysis assumption.

Table 1: LP-WUS evaluation scenarios

| NR RF channel BW | 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1, i.e. For 15kHz SCS, 242.5kHz guardband for 5M CBW and 452.5kHz guard band for 20M CBW For 30kHz SCS, 505kHz guardband for 5M CBW and 845kHz guard band for 100M CBW |
| WUS BW within NR channel | 5.04 MHz |
| WUS RB allocation (Note 1) | 24 RB, i.e. total 4.32MHz |
| WUS placement within NR channel | - Carrier edge for ACS and carrier center for ASCS |
| Guard RB size of LP-WUS | - 0 RB, 0.5 RF, 1RB, 2RBs, 3RB at each side. - Only symmetric guard RB has been considered |
| ACS interferer | 5MHz |
| Filter characteristic | 2rd and 5th order Butterworth Both analog and digital filter can be considered |
| Filter passband BW | 4.32/2=2.16MHz for low-pass filter 4.32MHz for band-pass filter |

Following list the S21 of 2th and 5th order butterworth filters and related parameters.

Figure 1: Frequency response of 2rd Butterworth filters

Figure 2: Frequency response of 5rd Butterworth filters

Following table list ACS value based on low-pass filter for zero-IF envelop detector architecture, in which pass bandwidth is 4.32/2=2.16MHz.

Table 2. evaluated ACS based on low-pass filter with 2.16MHz pass bandwidth

| Filter order |  |  | ACS, BWinterference = 5MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB/ 30kSCS | 0.5RB/ 30kSCS | 1RB/ 30kSCS | 2RB/ 30kSCS | 3RB/ 30kSCS |
| 5th | 20MHz, SCS = 15KHz | 0 ppm | -27.75 | -29.88 | -31.9 | -35.65 | -39.07 |
|  |  | 5 ppm | -27.6 | -29.73 | -31.76 | -35.52 | -38.95 |
|  |  | 10 ppm | -27.44 | -29.58 | -31.61 | -35.39 | -38.83 |
|  |  | 50 ppm | -26.15 | -28.35 | -30.45 | -34.34 | -37.87 |
|  |  | 200 ppm | -21.1 | -23.47 | -25.78 | -30.11 | -34.03 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -35.00 | -36.78 | -38.47 | -41.65 | -44.59 |
|  |  | 5 ppm | -34.87 | -36.65 | -38.35 | -41.54 | -44.49 |
|  |  | 10 ppm | -34.74 | -36.52 | -38.23 | -41.43 | -44.38 |
|  |  | 50 ppm | -33.66 | -35.5 | -37.25 | -40.53 | -43.55 |
|  |  | 200 ppm | -29.36 | -31.4 | -33.35 | -36.97 | -40.27 |
| 2nd | 20MHz, SCS = 15KHz | 0 ppm | -12.62 | -13.28 | -13.93 | -15.18 | -16.34 |
|  |  | 5 ppm | -12.57 | -13.24 | -13.89 | -15.13 | -16.3 |
|  |  | 10 ppm | -12.52 | -13.19 | -13.84 | -15.09 | -16.26 |
|  |  | 50 ppm | -12.12 | -12.8 | -13.47 | -14.74 | -15.93 |
|  |  | 200 ppm | -10.59 | -11.3 | -12.01 | -13.36 | -14.63 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -14.96 | -15.56 | -16.13 | -17.24 | -18.28 |
|  |  | 5 ppm | -14.91 | -15.51 | -16.09 | -17.2 | -18.24 |
|  |  | 10 ppm | -14.87 | -15.47 | -16.05 | -17.16 | -18.21 |
|  |  | 50 ppm | -14.51 | -15.13 | -15.72 | -16.85 | -17.91 |
|  |  | 200 ppm | -13.12 | -13.77 | -14.41 | -15.62 | -16.76 |

Following table is based on analog band-pass filter for RF/IF envelop detector architecture, in which pass bandwidth is 4.32MHz.

Table 3. evaluated ACS based on analog band-pass filter with 4.32MHz pass bandwidth

| Filter order |  |  | ACS, BWinterference = 5MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB/ 30kSCS | 0.5RB/ 30kSCS | 1RB/ 30kSCS | 2RB/ 30kSCS | 3RB/ 30kSCS |
| 5th | 20MHz, SCS = 15KHz | 0 ppm | -17.96 | -19.16 | -20.34 | -22.64 | -24.83 |
|  |  | 5 ppm | -17.88 | -19.08 | -20.26 | -22.56 | -24.75 |
|  |  | 10 ppm | -17.79 | -18.99 | -20.18 | -22.48 | -24.68 |
|  |  | 50 ppm | -17.09 | -18.3 | -19.49 | -21.82 | -24.05 |
|  |  | 200 ppm | -14.46 | -15.67 | -16.89 | -19.3 | -21.63 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -22.23 | -23.35 | -24.44 | -26.55 | -28.56 |
|  |  | 5 ppm | -22.15 | -23.27 | -24.36 | -26.47 | -28.49 |
|  |  | 10 ppm | -22.07 | -23.19 | -24.29 | -26.4 | -28.42 |
|  |  | 50 ppm | -21.41 | -22.54 | -23.66 | -25.8 | -27.84 |
|  |  | 200 ppm | -18.86 | -20.05 | -21.22 | -23.47 | -25.63 |
| 2nd | 20MHz, SCS = 15KHz | 0 ppm | -8.18 | -8.56 | -8.94 | -9.7 | -10.43 |
|  |  | 5 ppm | -8.15 | -8.53 | -8.92 | -9.67 | -10.4 |
|  |  | 10 ppm | -8.12 | -8.51 | -8.89 | -9.64 | -10.38 |
|  |  | 50 ppm | -7.9 | -8.28 | -8.67 | -9.43 | -10.17 |
|  |  | 200 ppm | -7.05 | -7.44 | -7.83 | -8.6 | -9.36 |
|  | 100MHz, SCS = 30KHz | 0 ppm | -9.56 | -9.93 | -10.3 | -11.02 | -11.72 |
|  |  | 5 ppm | -9.53 | -9.91 | -10.27 | -10.99 | -11.69 |
|  |  | 10 ppm | -9.51 | -9.88 | -10.25 | -10.97 | -11.67 |
|  |  | 50 ppm | -9.29 | -9.67 | -10.04 | -10.76 | -11.47 |
|  |  | 200 ppm | -8.47 | -8.85 | -9.23 | -9.97 | -10.7 |

The main reason between table 2 and table 3 is that less pass bandwidth will lead to better filter suppression capability. So table 2 with less pass bandwidth will have better performance.

Observation 1: table 2 and table 3 list filter suppression performance for ACS evaluation.

2.2 Guard RB evaluation for ASCS based on filter characteristics

Following table is based on low-pass filter for zero-IF envelop detector architecture, in which pass bandwidth is 4.32/2=2.16MHz.

Table 4. evaluated ASCS based on low-pass filter with 2.16MHz pass bandwidth

| Filter order |  |  | ASCS, BWinterference = 5MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB/ 15kSCS | 1RB/ 15kSC | 2RB/ 15kSC | 4RB/ 15kSC |  |
| 5th | 20MHz, SCS = 15KHz | 0 ppm | -20.05 | -22.39 | -24.75 | -29.25 |  |
|  |  | 5 ppm | -19.89 | -22.22 | -24.58 | -29.1 |  |
|  |  | 10 ppm | -19.72 | -22.05 | -24.41 | -28.94 |  |
|  |  | 50 ppm | -18.44 | -20.7 | -23.05 | -27.67 |  |
|  |  | 200 ppm | -14.31 | -16.06 | -18.08 | -22.66 |  |
| 2nd | 20MHz, SCS = 15KHz | 0 ppm | -10.71 | -11.42 | -12.12 | -13.47 |  |
|  |  | 5 ppm | -10.65 | -11.37 | -12.07 | -13.42 |  |
|  |  | 10 ppm | -10.6 | -11.31 | -12.02 | -13.38 |  |
|  |  | 50 ppm | -10.19 | -10.9 | -11.61 | -12.99 |  |
|  |  | 200 ppm | -8.66 | -9.36 | -10.07 | -11.49 |  |

Following table is based on analog band-pass filter for RF/IF envelop detector architecture, in which pass bandwidth is 4.32MHz.

Table 5. evaluated ASCS based on analog low-pass filter with 4.32MHz pass bandwidth

| Filter order |  |  | ASCS, BWinterference = 5MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB/ 15kSCS | 1RB/ 15kSC | 2RB/ 15kSC | 4RB/ 15kSC | 0RB/ 15kSCS |
| 5th | 20MHz, SCS = 15KHz | 0 ppm | -14.41 | -15.59 | -16.79 | -19.2 | -21.56 |
|  |  | 5 ppm | -14.33 | -15.51 | -16.71 | -19.12 | -21.48 |
|  |  | 10 ppm | -14.24 | -15.42 | -16.62 | -19.03 | -21.39 |
|  |  | 50 ppm | -13.58 | -14.74 | -15.93 | -18.34 | -20.72 |
|  |  | 200 ppm | -11.24 | -12.29 | -13.39 | -15.73 | -18.13 |
| 2nd | 20MHz, SCS = 15KHz | 0 ppm | -7.25 | -7.64 | -8.02 | -8.79 | -9.54 |
|  |  | 5 ppm | -7.22 | -7.61 | -8 | -8.76 | -9.52 |
|  |  | 10 ppm | -7.2 | -7.58 | -7.97 | -8.73 | -9.49 |
|  |  | 50 ppm | -6.97 | -7.36 | -7.74 | -8.51 | -9.27 |
|  |  | 200 ppm | -6.14 | -6.52 | -6.91 | -7.68 | -8.45 |

The main reason between table 4 and table 5 is that less pass bandwidth will lead to better filter suppression capability. So table 2 with less pass bandwidth will have better performance.

Observation 2: table 4 and table 5 list filter suppression performance for ASCS evaluation.

Proposal 1: it’s suggested to capture table 2-5 into final TR.

2.3 Target ACS when assume the same coverage as legacy network

Last meeting agreement is listed as below for information.

Issue 1-1-2: Target ACS value for LP-WUR receiver Agreement: - The methodology for guard RB is that at first conclude the relationship between guard RB and adjacent channel selectivity. - WUR ACS should be further discussed in the context of the guard RB design and main receiver test requirement.

Following show the target ACS value analysis based on the same interference level and REFSENSE degradation assumption as legacy UE.

| parameter | value |  | comment |
| --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |
| P_interference: Interference power for ACS requirement | =REFSENS Normal_UE + 45.5dB=-52.48dBm/5MHz | -25dBm | We use n41 as example. The same as the value for ACS testing considering such interference mainly comes from adjacent channel gNB power even for LP-WUR case |
| REFSENSE degradation due to adjacent channel interference | +14dB | -56.5dBm | This is also related to target coverage. +14dB is the same as legacy UE. But when target coverage shrinks for other UE device, large REFSENSE degradation is allowed, which will lead to smaller ACS. |
| ACS | 31.7 for 9dB NF assumption 25.7 for 15dB NF assumption 16.7 for 24dB NF assumption | 31.5 for 9dB NF assumption 25.5 for 15dB NF assumption 16.5 for 24dB NF assumption |  |

As analyzed in above table, ACS for 9dB, 15dB, 24dB NF are suggested as 32dB, 26dB and 17dB respectively. Based on the ACS evaluation in table 2 and 3, it seems 5 order butterworth filter is enough with max 2PRB (30kHz SCS) .

Proposal 2: to avoid adjacent channel interference, 5 order butterworth filter is suggested with max 2PRB (30kHz SCS) guard RB.

2.4 Dedicated LP-WUS operation band

LS has been sent from RAN1 to reply the possibility of same CBW of LP-WUS as the carrier CBW. RAN1 currently doesn’t evaluate the case when LP-WUS occupies the whole carrier. As approved in last meeting  it’s RAN4’s responsibility to decide whether to consider dedicated LP-WUS operation band. Considering this is the study item, RAN4’s main responsibility is to identify RF architecture, ACS and guardband assumption. However ACS/guard band requirements are not needed for the case when LP-WUS occupies the whole operation band.  Besides, dedicated operation band will not introduce extra new architecture. Therefore, it seems we can wait for work item to discuss dedicated LP-WUS operation band.

If companies still have strong view of this dedicated LP-WUS operation band in this study item, n28 and n41 could be used as example band which have been commercially deployed by many operators among the world.

Proposal 3: if RAN4 finally approve to define LP-WUS dedicated operation band, band 28 and band 41 are suggested as example band which has been globally deployed by many operators.

# 3. Conclusions

In this contribution, LP-WUR architecture and RF requirements are discussed with following observations:

Observation 1: table 2 and table 3 list filter suppression performance for ACS evaluation.

Observation 2: table 4 and table 5 list filter suppression performance for ASCS evaluation.

Proposal 1: it’s suggested to capture table 2-5 into final TR.

Proposal 2: to avoid adjacent channel interference, 5 order butterworth filter is suggested with max 2PRB (30kHz SCS) guard RB.

Proposal 3: if RAN4 finally approve to define LP-WUS dedicated operation band, band 28 and band 41 are suggested as example band which has been globally deployed by many operators.

# 4. Reference

[1] R4-2314932, WF on LP-WUS, vivo

[2] R4-2314931, Reply LS to RAN1 on low-power wake-up receiver architectures, vivo

[3] R4-2314726, TP to TR 38.869 on LP-WUS receiver architectures, vivo, Huawei, HiSilicon, Qualcomm, Ericsson, Nokia