# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315196

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.6.3.1

Source: 	CMCC

Title: 	Discussion on Beam correspondence requirements

Document for:	Discussion

# 1. Introduction

Last meeting, WF is approved [1]. In this contribution, we focus on the discussion of beam correspondence requirement for initial access and INACTIVE state.

- Discussion

  - tolerance requirement discussion

Last meeting agreement is listed as below:

- WF - Power tolerance and spherical coverage requirement should be jointly considered. - RAN4 acknowledges the UE requires sufficient transmit opportunities to optimize the output power accuracy (settling time). - FFS the feasibility of sufficient transmit opportunities and the impact of wait time during the test. - FFS, companies are encouraged to bring details of mechanisms that they believe cause error in Tx power level settling during initial access.

In last meeting, one main issue for tolerance is about whether this is test issue.

For RRC_CONNECTED state UE, there is no tolerance for spherical coverage requirements. Before spherical coverage testing, UE optimize it power accuracy with at least 200ms to achieve zero tolerance. This implies total 200ms is sufficient for UE to achieve zero tolerance. It is worth noting that UE’s transmission in this total 200ms is not contiguous due to TDD DL-UL configuration. In fact UE transmit for 0.15ms per 0.625ms periodicity (120kHz SCS) within this total 200ms, i.e. 2UL symbol+1UL slot per 0.625ms periodicity according to UL-DL configuration (tdd-UL-DL-ConfigurationCommon). Detailed configurations are listed in following fig 1 and fig 2.

Fig 1. BC test procedure in 38.521-2

Fig 2. TDD UL-DL test configuration in 38.521-2

Observation 1: In current CONNECTED state spec, UE is allowed to transmit 0.15ms per 0.625ms periodicity within total 200ms to make sure UE optimize output power accuracy with zero tolerance in BC testing.

For RRC_CONNECTED state, during the relative power control testing, SS first transmit 1dB TPC command from minimum output power until UE achieve target power which is less than min peak EIRP. Next, every 10ms gNB will schedule the UE’s PUCCH or PUSCH for 1ms transmission with 0 TPC command. This implies at target testing power, there is only 1ms for UE to optimize it power accuracy, i.e. to meet relative power control testing.

Fig 3. test procedure for relative power control in 38.521-2

Observation 2: In current CONNECTED state spec, only 1ms transmission is allowed for UE at target power level during the relative power control testing.

Based on above analysis, it seems if we could leave enough time for UE in IA state, for example, we make the UL Tx time pattern for msg1 similar as that of RRC_CONNECTED UE, UE at IA state could also optimize its output power accuracy with zero tolerance.

At IA state, UE will re-transmit PREAMBLE when ra-ResponseWindow expires on RACH occasion and the max number of re-transmission is defined by preambleTransMax with max 200 in 38.331. For ra-ResponseWindow, it is less than or equals to 10ms and is measured in number of slots.

Therefore, if we choose reasonable PRACH config, TDD UL-DL configuration and RAR window, the UL Tx time occupy pattern could be similar as in CONNECTED state.

Observation 3: if we choose reasonable RO configuration, TDD UL-DL configuration and RAR window, the UL Tx occupancy pattern could be similar as in CONNECTED state.

Following fig show the illustration of PRACH RO configuration.

Fig 4 detailed illustration of RO configuration

Therefore, to make UE transmit PREAMBLE with as long as possible time, one solution is to make PRACH duration(the last column in following table) as large as possible, i.e. use max 12 symbols. Detailed PRACH RO configurations are listed as below for information with PRACH duration as max 12 OFDM symbols. Accordingly, number of time-domain PRACH occasions within a PRACH slot (last but one column in following table) as less as possible and the number of slot number (5th column) as many as possible.

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum.

| PRACH Config. Index | Preamble format | nf mod x=y |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 112 | B4 | 16 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 113 | B4 | 16 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 114 | B4 | 8 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 115 | B4 | 8 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 116 | B4 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 117 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 118 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 119 | B4 | 4 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 120 | B4 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 1 | 12 |
| 121 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 122 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 123 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 124 | B4 | 1 | 0 | 19, 39 | 2 | 2 | 1 | 12 |
| 125 | B4 | 1 | 0 | 17, 19, 37, 39 | 0 | 1 | 1 | 12 |
| 126 | B4 | 1 | 0 | 24,29,34,39 | 2 | 1 | 1 | 12 |
| 127 | B4 | 1 | 0 | 9,19,29,39 | 2 | 2 | 1 | 12 |
| 128 | B4 | 1 | 0 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 129 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 1 | 12 |
| 130 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 1 | 12 |
| 131 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 132 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 1 | 12 |
| 133 | B4 | 1 | 0 | 9,11,13,15,17,19 | 0 | 1 | 1 | 12 |
| 134 | B4 | 1 | 0 | 3,5,7,9,11,13 | 2 | 1 | 1 | 12 |
| 135 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 136 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 1 | 12 |
| 137 | B4 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 2 | 2 | 1 | 12 |
| 138 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 139 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 | 12 |
| 140 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 2 | 1 | 1 | 12 |
| 141 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 0 | 2 | 1 | 12 |
| 142 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 | 12 |
| 143 | B4 | 1 | 0 | 0, 1, 2,…, 39 | 2 | 1 | 1 | 12 |

Based on above analysis, we have following candidate options of PRACH config which will have longer UL Tx opportunity or similar UL Tx opportunity compared with that of CONNECTED state BC testing (i.e. 0.15ms UL per 0.625ms periodicity within total 200ms, 0.24 UL ratio and 48ms sum of all RO duration within 200ms)

- Config 143: for this configuration index, UL RO occupy 12 symbols per 14 symbols(1 slots) with 60kHz SCS assumption, i.e. 0.21ms per 0.25ms, ratio is 0.84 and sum of all RO is 168ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 142: for this configuration index, UL RO occupy 12 symbols per 28 symbols(2 slots) with 60kHz SCS assumption, i.e. 0.21ms per 0.5ms, ratio is 0.42 and sum of all RO is 84ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 140: for this configuration index, UL RO occupy 12 symbol/slot *12 slots within total 40 slots with 60kHz SCS assumption, i.e. 2.57ms per 10ms, ratio is 0.257 and sum of all RO is 51.4ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 139: for this configuration index, UL RO occupy 12 symbols per 56 symbols(4 slots) with 60kHz SCS assumption, i.e. 0.21ms per 1ms, ratio is 0.21 and sum of all RO is 42ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 138: for this configuration index, UL RO occupy 12 symbols per 56 symbols(4 slots) with 60kHz SCS assumption, i.e. 0.21ms per 1ms, ratio is 0.21 and sum of all RO is 42ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

Proposal 1: The above list includes all the candidate options of PRACH configuration that either have a longer UL transmission occupancy or a similar UL transmission occupancy compared to the BC testing in the CONNECTED state.

In addition to PRACH config, we should also make sure RAR window will start after last preamble and end before next RO. If so, UE could send PREAMBLE in each RO according to the PRACH config in proposal 1. besides, define TDD UL-DL configuration to make sure all candidate RO is UL.

Proposal 2: TDD UL-DL configuration and RAR window should be configured accordingly based on chosen PRACH config to make sure UE send PREAMBLE on each RO.

- The RAR window starts after the previous RO and ends before the next RO.

- TDD UL-DL configuration to make sure all candidate RO is UL.

Based on above analysis, UE could optimize its output power accuracy with zero tolerance as what is tested for CONNECTED BC requirements. Therefore, tolerance requirement could be zero and there is no need for tolerance requirements at IA or INACTIVE state.

Proposal 3: there is no need for tolerance requirements of spherical coverage at IA/Inactive state.

2.2 UE capability

Last meeting agreement is listed as below:

Issue 1-3: Requirement applicability - Proposals - Option 1: Mandatory from Rel-18 (Nokia, ZTE, Samsung, Apple (conditionally), Sony) - Option 2: Optional (vivo (by declaration), Xiaomi, MediaTek, oppo) - WF - FFS

- It seems there is no further discussion in R19, optional applicability implies for some UE, there is no spherical coverage requirements at IA state or inactive state, which requirement is very important for cell coverage. Operators will have no ides of how to plan cell deployment because usually the UE not supporting these BC requirement would be the bottleneck for the coverage. Therefore, it’s suggested that requirement applicability is mandatory from R18.

- Proposal 4: it’s suggested that BC requirement is mandatory from Rel-18.

# 3. Conclusions

In this contribution, beam correspondence requirements for initial access are discussed with following observations and proposals:

Observation 1: In current CONNECTED state spec, UE is allowed to transmit 0.15ms per 0.625ms periodicity within total 200ms to make sure UE optimize output power accuracy with zero tolerance in BC testing.

Observation 2: In current CONNECTED state spec, only 1ms transmission is allowed for UE at target power level during the relative power control testing.

- Observation 3: if we choose reasonable RO configuration, TDD UL-DL configuration and RAR window, the UL Tx occupancy pattern could be similar as in CONNECTED state.

- Config 143: for this configuration index, UL RO occupy 12 symbols per 14 symbols(1 slots) with 60kHz SCS assumption, i.e. 0.21ms per 0.25ms, ratio is 0.84 and sum of all RO is 168ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 142: for this configuration index, UL RO occupy 12 symbols per 28 symbols(2 slots) with 60kHz SCS assumption, i.e. 0.21ms per 0.5ms, ratio is 0.42 and sum of all RO is 84ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 140: for this configuration index, UL RO occupy 12 symbol/slot *12 slots within total 40 slots with 60kHz SCS assumption, i.e. 2.57ms per 10ms, ratio is 0.257 and sum of all RO is 51.4ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 139: for this configuration index, UL RO occupy 12 symbols per 56 symbols(4 slots) with 60kHz SCS assumption, i.e. 0.21ms per 1ms, ratio is 0.21 and sum of all RO is 42ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

- Config 138: for this configuration index, UL RO occupy 12 symbols per 56 symbols(4 slots) with 60kHz SCS assumption, i.e. 0.21ms per 1ms, ratio is 0.21 and sum of all RO is 42ms within 200ms

  - Note: with preambleTransMax as 200, the total time for PRACH transmission would be larger than 200ms

Proposal 1: The above list includes all the candidate options of PRACH configuration that either have a longer UL transmission occupancy or a similar UL transmission occupancy compared to the BC testing in the CONNECTED state.

Proposal 2: TDD UL-DL configuration and RAR window should be configured accordingly based on chosen PRACH config to make sure UE send PREAMBLE on each RO.

- The RAR window starts after the previous RO and ends before the next RO.

- TDD UL-DL configuration to make sure all candidate RO is UL.

Proposal 3: there is no need for tolerance requirements of spherical coverage at IA/Inactive state.

- Proposal 4: it’s suggested that BC requirement is mandatory from Rel-18.

# 4. Reference

[1] R4-2314676, WF on beam correspondence in initial access and RRC_INACTIVE, Nokia