3GPP TSG-RAN WG4 Meeting #108bis	R4-2315230

Xiamen, China, October 09 – October 13, 2023

Source:	Huawei, Hisilicon

Title:	Remaining issues on Rel-18 Multi-carrier enhancements

Agenda item:			5.23.2.1

Document for:	Approval

# 1. Introduction

In RAN4#107, the big CR [1] for Rel-18 Tx switching was agreed with the following clarification in section 5.5C in TS38.101-1

## 5.5C Configurations for SUL … For the NR SUL band configurations with inter-band CA in sub-clause 5.5C, when the capability [BandCombination-UplinkTxSwitch-r18] is present, three or four bands can be configured in the uplink with simultaneous uplink transmission on up to two bands, and the corresponding requirements for SUL band configurations with inter-band CA and with uplink assigned to one or two bands shall apply. For each uplink band pair in the NR SUL band configurations with inter-band CA, according to the capability [uplinkTxSwitching-OptionSupport], – if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink assigned to one band on band X or band Y apply; – if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding requirements for SUL band configuration with inter-band CA and with uplink CA between the two uplink bands apply.

In RAN#101, the conclusions were made as follows.

Offline agreed WF for Issue 1 and Issue 2 - The scenario {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band} is supported in Rel-18 UL Tx switching framework with following restrictions. - For the band pair between {NUL and NUL}, “switched UL” or “dual UL” can be reported/configured. - For any other band pair including SUL (between {SUL and another SUL}, between {SUL and corresponding NUL}, and between {SUL and non-corresponding NUL}), only “switched UL” can be reported/configured in Rel-18. - Back-to-back transmission (i.e., switching without having switching gap) between {SUL and another SUL} and between {SUL and non-corresponding NUL} are not supported in Rel-18. - Note: this does not prevent a study and normative work to support simultaneous transmission between {SUL and another SUL} and/or between {SUL and non-corresponding NUL} as well as back-to-back transmissions between {SUL and another SUL} and/or between {SUL and non-corresponding NUL} in future release - The same restrictions are applied to the scenarios {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s). - For band pairs between {SUL and non-corresponding NUL} in this scenario, only “switched UL” can be reported/configured in Rel-18 - Back-to-back transmission (i.e., switching without having switching gap) between {SUL and non-corresponding NUL} in this scenario is not supported in Rel-18 - Note: this does not preclude a possible support based on other UE architecture than Rel-18 UL Tx switching - Clarify the restrictions above at least in TS38.101-1 e.g., section 5.5C “Configurations for SUL” - Note: TS38.214 may not need to capture the restrictions, and whether/how to capture restrictions in TS38.331/306 can be discussed in Q4 RAN2

In the contribution, we discuss the remaining issues on Rel-18 multi-carrier.

# 2. Discussion

2.1 Discussion on the supported scenarios in Rel-18 Tx switching

In both scenarios {SUL#1 + NUL#1} + { SUL#2 + NUL#2} and { SUL#1 + NUL#1} + NUL#2 supported in Rel-18 UL Tx switching framework, for the band pair between {NUL#1 and NUL#2}, “switched UL” or “dual UL” can be reported/configured. The switching pattern of “dualUL” can be supported for the band pair of NUL#1+NUL#2 in the SUL band combinations including such UL CA configurations listed in Table 1. And the Table 1 is the excerpt from Table 5.5C-4 and 5.5C-5 in TS38.101-1. The clarification in section 5.5C in the agreed CR R4-2310270 is sufficient for the 1st and 2nd sub-bullet in the RANP conclusion considering no UL CA configurations for the band pair of {SUL#1+SUL#2} or {NUL#1+SUL#2} in the existing specification and only “switchedUL” pattern can be supported.

The Tx switching happens within at least one band pair.

For the 3rd bullet, back-to-back transmission and switching without switching gap in a band pair is not Rel-18 Tx switching. Thus, further clarification in spec is not required.

Table 1 SUL band combination with inter-band CA including UL CA configuration

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A_n79C-n83A | SUL_79A-n83A CA_n79C-n83A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41C_n79A-n83A | SUL_n79A-n83A CA_n41C CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41C_n79C-n83A | CA_n41C CA_n79C SUL_n79A-n83A CA_n79C-n83A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n41A_n79C-n95A | SUL_79A-n95A CA_n79C-n95A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  | SUL_n79A-n95A CA_n41C CA_n41A-n79A | n41 | CA_n41C_BCS1 |  |
| CA_n41C_n79A-n95A |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n95 | 5, 10, 15 |  |
|  | CA_n41C CA_n79C SUL_n79A-n95A CA_n79C-n95A CA_n41A-n79A | n41 | CA_n41C_BCS1 |  |
| CA_n41C_n79C-n95A |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n98 | 5, 10, 15 |  |
|  | SUL_79A-n98A CA_n79C-n98A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A_n79C-n98A |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | SUL_n79A-n98A CA_n41C CA_n41A-n79A | n41 | CA_n41C_BCS1 |  |
| CA_n41C_n79A-n98A |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n41C CA_n79C SUL_n79A-n98A CA_n79C-n98A CA_n41A-n79A | n41 | CA_n41C_BCS1 |  |
| CA_n41C_n79C-n98A |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79A_n41C-n83A | SUL_n41A-n83A CA_n41C-n83A CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79C_n41A-n83A | SUL_n41A-n83A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79C_n41C-n83A | CA_n41C CA_n79C SUL_n41A-n83A CA_n41C-n83A CA_n41A-n79A | n41 | CA_n41C_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n83 | 5, 10, 15, 20, 30 |  |
| CA_n79C_n41A-n95A | SUL_n41A-n95A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79A_n41C-n95A | SUL_n41A-n95A CA_n41C-n95A CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79C_n41C-n95A | CA_n41C CA_n79C SUL_n41A-n95A CA_n41C-n95A CA_n41A-n79A | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n95 | 5, 10, 15 |  |
| CA_n79C_n41A-n98A | SUL_n41A-n98A CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79A_n41C-n98A | SUL_n41A-n98A CA_n41C-n98A CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n79C_n41C-n98A | CA_n41C CA_n79C SUL_n41A-n98A CA_n41C-n98A CA_n41A-n79A | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 | 0 |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

| SUL band combination with CA | Uplink CA configuration or SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n98A_n79A-n95A | SUL_n41A-n98A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n98A | SUL_n41A-n83A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n95A | SUL_n41A-n83A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n95 | 5, 10, 15 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

Observation 1: The clarification in section 5.5C in the agreed CR R4-2310270 is sufficient for the 1st and 2nd sub-bullet in the RANP conclusion.

Observation 2: Switching without switching gap for a band pair is not Rel-18 Tx switching.

Proposal 1: The clarification in section 5.5C in the agreed CR R4-2310270 is sufficient to the RANP conclusion, and no further revision for section 5.5C is required.

2.2 The ambiguous switching period for band pair

Switching period is related to PLL operation, such as, PLL retuning and initializing TX chains with parameters loading when frequency is changed, and PLL branches without frequency changing. PLL retuning and parameter loading may cost relatively longer period, e.g., 140us. PLL branching without frequency changing costs shorter period, e.g., 35us.

In Rel-18 Tx switching among 3/4 bands, the UE may apply different Tx chains/ PLLs for a specific band/ band pair in different band combinations, resulting in various switching period values in {35, 140, 210}us for the band pair.

Observation 3: Applying different PLLs to one specific band pair may result in various switching period for it.

In the CR [2] to TS 38.331 on UE capability reporting for Rel-18 Tx switching agreed in RAN2#122, the granularity of UE reporting capability for switching period is per band pair per band combination.

BandCombination-UplinkTxSwitch-v18xy ::= SEQUENCE {

supportedBandPairListNR-v18xy                    SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v18xy   OPTIONAL,

uplinkTxSwitchingMinimumSeparationTime-r18       ENUMERATED {n0us, n500us},

uplinkTxSwitchingAdditionalPeriodDualUL-List-r18 SEQUENCE (SIZE (1..maxULTxSwitchingBetweenBandPairs)) OF UplinkTxSwitchingAdditionalPeriodDualUL-r18   OPTIONAL

}

ULTxSwitchingBandPair-v18xy ::=     SEQUENCE {

uplinkTxSwitchingOptionForBandPair-r18       ENUMERATED {switchedUL, dualUL, both},

uplinkTxSwitchingMaintainedUL-Trans-r18      BIT STRING (SIZE(1..maxSimultaneousBands-2))         OPTIONAL,

uplinkTxSwitchingPeriodForBandPair-r18       SEQUENCE {

switchingPeriodFor2T-r18                ENUMERATED {n35us, n140us, n210us}               OPTIONAL,

switchingPeriodFor1T-r18                ENUMERATED {n35us, n140us, n210us}

}

}

According to rules in RAN2 specification, a UE would only report the support of the inter-band CA band combination with the largest number of bands among the ones with same capability. Meanwhile, the UE also supports the fallback combo(s) of the reported combo and the network can configures the fallbacks to the UE.

TS 38.331, 5.6.1.4 Setting band combinations, feature set combinations and feature sets supported by the UE The UE shall: - compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), …. 1> for each band combination included in the list of "candidate band combinations": 2> if the network (E-UTRA) included the eutra-nr-only field, or 2> if the requested rat-Type is eutra: 3> remove the NR-only band combination from the list of "candidate band combinations"; NOTE 4: The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below. 2> if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and 2> if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell or SUL according to TS 38.306 [26]: 3> remove the band combination from the list of "candidate band combinations";

Observation 4: In Rel-18, the UE capability of switching period is reported per band pair per band combination.

Observation 5: The UE does not report the support of one fallback band combination with the same capability of another band combination in the list of “candidate band combinations”.

If one band combination corresponds to two different parent band combinations reported by the UE, the network may configures the fallback one to the UE. However the switching period for the band pair reported in the two band combinations may differ. And then UE cannot tell the corresponding sw period value for the band pair in the configured fallback combo.

For instance as UE capability signaling illustrated in Figure 1, the UE reports to support band combination#1 A+B+C, and band combination#2 A+B+D. In A+B+C, the switching period for band pair A+B is 35us, and in A+B+D, the switching period for band pair A+B is 140us. If the network configures band combination A+B, as the fallback band combination of band combination#1 and band combination#2, to the UE. The UE may get confused which switching period should be applied to the band pair A+B, either 35us reported in band combination#1, or 140us reported in band combination#2.

In this case, one simple solution would be setting a default switching period value or a default rule for the band pair with ambiguous sw period, such as, taking the band combination# with the longer value by default, 140us for the band pair A+B.

Figure 1 The UE reporting band combination#1and band combination#2 with common band pair A+B corresponding to different sw period respectively

- Reference UE architectures for band combination#1: A+B+C

- Reference UE architectures for band combination#1: A+B+D

Figure 2 Reference UE architectures for band combination#1 and band combination#2 with different switching period for band pair A+B

Observation 6: A default switching period value or a default rule for the band pair with ambiguous switching period can be considered, such as, taking the band combination# with the longer value by default, 140us for the band pair A+B.

However, in another example with UE reporting 4-band combos, the situation would be more complicated. The UE reports to support band combination#3 A+B+C+D, and band combination#4 A+B+C+E. In band combination#3, the switching period for band pair A+B is 35us, and that for band pair A+C is 140us.

- 35μs sw period for band pair A+B                 (b) 140μs sw period for band pair A+C

Figure 3 Reference UE architectures for band combination#3 A+B+C+D

In band combination#4, the switching period for band pair A+B is 140us, and that for band pair A+C is 35us.

- 140μs sw period for band pair A+B                 (b) 35μs sw period for band pair A+C

Figure 4 Reference UE architectures for band combination#4 A+B+C+E

If the network configures band combination A+B+C, as the fallback of band combination#3 and band combination#4, to the UE. The UE may get confused which switching period should be applied to the band pair A+B and band pair A+C. In this case, the default rule, to take the longer sw period for one band pair, would not take effect. The longer sw period for band pair A+B leads to the shorter sw period for the band pair A+C. And which band pair to apply longer sw period further perplexed the UE.

Observation 7: The potential default rule in Observation 6 would not take effect when the capability of 4-band combo reported and 3-band combo configured.

In the discussion during RAN4#108, some companies raised that if only the UE also reported the capability for the 3-band combo, the ambiguity problem for the switching period in the band pair corresponding to two 4-band combos could be solved. However, according to Observation 5, taking band pair A+B for example, if the capability of switching period in combo A+B+C is same as that in combo A+B+C+D or A+B+C+E, the UE would not additionally report the capability in the 3-band combo. On the other hand, if the UE does report the switching period for A+B in combo A+B+C in addition to that in the two 4-band combos, and the network configures the combo A+B+C to the UE, there are three possible switching periods for the band pair A+B:

- Switching period#1 in the combo A+B+C

- Switching period#2, as the fallback of the combo A+B+C+D

- Switching period#3, as the fallback of the combo A+B+C+E

The UE still cannot tell which period to apply to the band pair A+B or A+C.

Observation 8: UE reporting the capability for the 3-band combo additionally cannot solve the ambiguity problem for the switching period in the band pair corresponding to two 4-band combos, instead, introducing one more capability to worsen the issue.

Thus a unified solution should be pursued to determine the sw period of one band pair, when the network configures a fallback band combination, of two parent band combinations corresponding to different sw period values respectively, for the band pair, to the UE. Possible solutions include

- Network indicates the index of the band combination in the UE capability list of "candidate band combinations" that the configured fallback combo corresponds to. And the UE would be crystal clear on the switching period valued used for the band pair A+B

- Network indicates the specific switching period value applied to the band pair with more than one candidate switching period.

Proposal 2: A unified solution should be pursued to determine the switching period of one band pair, when the UE support more than one switching period for the band pair.

- Network indicates the band combination index corresponding to the configured fallback combo.

- Network indicates the switching period value applied to the band pair.

Proposal 3: Send LS concerning the ambiguity problem according to the issue#1 in R4-2314933 to confirm with RAN2.

2.3 Issue on two Tx chains parallel switching across 4 bands

In RAN#101, the conclusion related to 4 bands was made as follows

Offline agreed WF for Issue 4 - Keep both 3 bands and 4 bands switching cases for Rel-18 UL Tx switching.

In RAN4#105, the baseline UE assumption was agreed for the ambiguity issue when two Tx chains are switched between two different band pairs.

For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that: - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

The reason to introduce the baseline UE assumption is that the network only schedules transmissions on switch-from bands and switched to bands, rather than indicating a Tx chain to switch in the exact band pair.

However, it was proposed to solve the ambiguity issue of the exact switching band pairs for the concurrent switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}. The switching period resulting from the baseline assumption max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} is related to four band pairs. In this case, the UE performs switching only in two band pairs and it is possible to have a shorter switching period than the baseline value.  The UE cannot utilize the spare resource to transmit uplink channel/signal as baseline.

Observation 9: The switching period resulting from the baseline assumption max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} is related to four band pairs. The UE performs switching only in two band pairs and it is possible to have a shorter switching period than the baseline value.

In RAN4#107 meeting, the WF [3] on Tx switching ambiguity issue was approved.

- RAN4 maintains the baseline assumption in Issue 1-2-3 agreed in R4-2220546 during RAN4#105. - Neither of the two Tx chains is expected to be used for transmission during the switching periods in Rel-18 - Encourage companies to study the benefit to resolve the switching pattern ambiguity issue when 2 Tx chains are switched between two different band pairs for one Tx switching instance - At least for single TAG - FFS whether it applies to dual TAG

There are two possible switching cases in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}:

Case #1: A-> C, B -> D   period#1

Case #2: A-> D, B -> C   period#2

If period#1, as the switching period of case#1, is longer than period#2, period #1 is the baseline max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. In Rel-18, the set of switching period value is {35μs, 140μs, 210μs}. The largest difference between period#2 and period #1 is up to 175μs, which is equivalent to 5 OFDM symbols for 30kHz subcarrier spacing as shown in Figure 5.

Therefore, there could be 35.7% more uplink resource than the baseline assumption in one slot that can be used if the UE can perform Tx switching with the two band pairs corresponding to shorter switching period.

- Case #1: A-> C, B -> D   period#1                 (b) Case #2: A-> D, B -> C   period#2

Figure 5 illustration of two possible switching cases in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}

If the UE is able to perform Tx switching with the band pairs A->D, B->C as Case#2 in Figure 5(b) that is corresponded to shorter switching period, the UE could utilize the symbol#8~ symbol#12 to transmit uplink signal/channels compared to Case#1.

Observation 10: If the UE is able to perform Tx switching with the band pairs that are corresponded to shorter switching period, the UE could utilize more resources to transmit uplink signal/channels compared to baseline.

During the discussion of Rel-18 Tx switching, we have introduced and have been trying to introduce several optional/advanced UE capability based on UE implementation to get better performance.

#### RAN4#105 Way forward: - For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

#### RAN4#106bis - Further discuss the following options in the next meeting - Option 1: For the switching scenario without ambiguity, i.e., the two Tx chains are on the same band before or after switching (switch between {1T, 1T, 0T} and {0T, 0T, 2T}), introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1). - Tswitch_1 and Tswitch_2 are the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2. - Option 1a: apply the same UE capability for this scenario and for the following scenario agreed in the previous meetings. (MTK, China Telecom, ZTE) - Option 2: Not introduce advanced optional UE ability

Observation 11: Rel-18 Tx switching introduces optional/advanced UE capability based on UE implementation to get better performance

If the UE is advanced enough and able to choose the pair of band pairs with shorter switching period, as an optional UE capability, the UE can report min{max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} for the two pairs of band pair as the switching period for {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}.

Proposal 4: For the parallel switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}, in addition to the baseline UE assumption agreed in RAN4 #105, introduce optional UE capability with the switching period min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

Proposal 5: Send LS to RAN2 concerning the issue based on the approved WF R4-2306660 and the LS R4-2314933.

# 3. Conclusions

This contribution presents the following observations and proposals:

Observation 1: The clarification in section 5.5C in the agreed CR R4-2310270 is sufficient for the 1st and 2nd sub-bullet in the RANP conclusion.

Observation 2: Switching without switching gap for a band pair is not Rel-18 Tx switching.

Observation 3: Applying different PLLs to one specific band pair may result in various switching period for it.

Observation 4: In Rel-18, the UE capability of switching period is reported per band pair per band combination.

Observation 5: The UE does not report the support of one fallback band combination with the same capability of another band combination in the list of “candidate band combinations”.

Observation 6: A default switching period value or a default rule for the band pair with ambiguous switching period can be considered, such as, taking the band combination# with the longer value by default, 140us for the band pair A+B.

Observation 7: The potential default rule in Observation 6 would not take effect when the capability of 4-band combo reported and 3-band combo configured.

Observation 8: UE reporting the capability for the 3-band combo additionally cannot solve the ambiguity problem for the switching period in the band pair corresponding to two 4-band combos, instead, introducing one more capability to worsen the issue.

Observation 9: The switching period resulting from the baseline assumption max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} is related to four band pairs. The UE performs switching only in two band pairs and it is possible to have a shorter switching period than the baseline value.

Observation 10: If the UE is able to perform Tx switching with the band pairs that are corresponded to shorter switching period, the UE could utilize more resources to transmit uplink signal/channels compared to baseline.

Observation 11: Rel-18 Tx switching introduces optional/advanced UE capability based on UE implementation to get better performance.

Proposal 1: The clarification in section 5.5C in the agreed CR R4-2310270 is sufficient to the RANP conclusion, and no further revision for section 5.5C is required.

Proposal 2: A unified solution should be pursued to determine the switching period of one band pair, when the UE support more than one switching period for the band pair.

Proposal 3: Send LS concerning the ambiguity problem according to the issue#1 in R4-2314933 to confirm with RAN2.

Proposal 4: For the parallel switching of two Tx chain in the case {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on bands {A, B, C, D}, in addition to the baseline UE assumption agreed in RAN4 #105, introduce optional UE capability with the switching period min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

Proposal 5: Send LS to RAN2 concerning the issue based on the approved WF R4-2306660 and the LS R4-2314933.

# 4. Reference

[1] R4-2310270, CR for 38.101-1: Time mask for switching across three or four uplink bands, RAN4#107

[2] R2-2306912, UE capability reporting for Rel-18 UL Tx switching enhancements, RAN2#122

[3] R4-2306660, WF on Tx switching ambiguity issue, MediaTek Inc, RAN4 #106bis-e.

[4] R4-2308929, Discussion on resolving ambiguity issue on Tx switching, RAN4 #107