# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315247

# Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.34.3.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements impacts for Network energy saving for NR

Document for:	Discussion

# Introduction

In RAN4#108 meeting, the Cell DTX/DRX impact on RRM requirements has been discussed in WF [1], as following. In this contribution, our views are provided for the RRM requirements impact beause of Cell DTX/DRX.

Issue 2-1-1: RRM impacts of Cell DTX/DRX – general

Proposals

  - Proposal 1: If RAN1 concludes that the RS for RRM measurement (e.g RRM/RLM/BFD/BM) is not impacted by cell DTX, there is no RRM impacts for objective#2 (Huawei)

  - Proposal 2: to indicate RAN1 that: (Apple)

    - Based on network configuration, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX:

    - PRS

    - CSI-RS configured by measObjectNR (for RRM)

    - CSI-RS associated with RLM, BFD, CBD

    - Periodic/Semi-persistent CSI-RS for beam measurement

    - FFS: PDCCH in USS to be decided by RAN1/2

    - FFS: PDCCH in Type-3 CSS to be decided by RAN1/2

To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

  - Proposal 3:  For cell DTX/DRX, the impact to RRM requirements is not necessary. By gNB implementation, adjustment of CSI-RS measurement period by RRC is always possible, and therefore no impact to spec is needed. UE assumes CSI-RS occasions are still available if the CSI-RS periodicity/offset configuration indicates the availability of CSI-RS during cell DTX in-active period (Vivo)

Issue 2-1-2: RRM impacts of Cell DTX/DRX – SSB based measurement

Agreement:

RAN4 to reuse the legacy requirement for SSB-based measurement in Cell DTX/DRX

Issue 2-1-3: RRM impacts of Cell DTX/DRX – CSI-RS based measurement

Proposals:

  - Proposal 1: There is no impact to CSI-RS based measurement for legacy UE. (Ericsson)

  - Proposal 2: RAN4 to discuss if the UE is expected to perform CSI-RS based measurements during non-active periods of cell DTX, including the CSI-RS based L3/L1/RLM/BFD measurements. (Nokia)

  - Proposal 3: RAN4 to wait RAN1 and RAN2’s progress on Cell DTX/DRX for NES UE’s CSI-RS measurement

Issue 2-1-4: RRM impacts of Cell DTX/DRX – Others

Proposals:

  - Proposal 1: RAN4 to study whether the measurement procedures will be impacted due to Cell DTX/DRX activation/deactivation. (Ericsson)

  - Proposal 2: RAN4 is to discuss the interruption requirements due to changes in Cell DTX/DRX configurations. (Intel)

  - Proposal 3: RAN4 to discuss how to minimize the impact to SCell activation delay due to cell DTX/DRX. In the best case, the SCell activation delay should not be interrupted or extended by the non-active periods of cell DTX/DRX. (Nokia)

# RRM impacts of Cell DTX/DRX

The corresponding R1 agreement on Cell DTX/DRX (RAN1 #112-bis-e, Apr. 2023) is excerpted as follows [2]:

Agreement

From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions.

- Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting)

- FFS:

  - PDCCH in USS

    - UE behaviour for retransmission

    - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation

  - PDCCH in Type-3 CSS

    - UE behaviour for retransmission

    - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation

  - PRS

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking)

  - Periodic/Semi-persistent CSI-RS (for BM)

    - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM

- FFS: Whether the same or different UE behaviour is applicable with or without C-DRX

- FFS: Whether the list of impacted signals/channels can be configurable

- FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX

- FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

It can be observed that in the current agreement, the R18 UE supporting cell DTX can skip CSI-RS for CSI reporting with reportQuantity including RI. Besides, the CSI-RS L3 measurements, CSI-RS L1 measurements (for RLM/BFD/L1-RSRS/CBD) are FFS and not impacted yet.

Observation 1: The R18 UE supporting cell DTX can skip CSI-RS for CSI reporting with reportQuantity including RI, while other CSI-RS measurements are not impacted yet.

Proposal 1: For Cell DTX/DRX, no RRM requirement impacts on CSI-RS based measurement. This can be revised if RAN1 concluded the impact on more CSI-RS because of cell DTX/DRX.

# Summary

The observations and proposals in this contribution are summarized:

Observation 1: The R18 UE supporting cell DTX can skip CSI-RS for CSI reporting with reportQuantity including RI, while other CSI-RS measurements are not impacted yet.

Proposal 1: For Cell DTX/DRX, no RRM requirement impacts on CSI-RS based measurement. This can be revised if RAN1 concluded the impact on more CSI-RS because of cell DTX/DRX.

# Reference

- R4-2314381, WF on RRM requirements for NR network energy saving, RAN4#108, Aug. 2023.

- “Draft Report of 3GPP TSG RAN WG1 #112-bis-e v0.1.0”, online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Report, April 2023.